3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2400615r05

Online, Jan 22 – 29, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Use case for NWDAF-assisted policy control and QoS

Document for:	Approval

Agenda Item:	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract: It is proposed a new use case for NWDAF-assisted policy control and QoS.

1. Introduction/Discussion

This paper proposes a new use case for WT#3.1:

- WT3.1 – Study whether and what additionally needs to be supported in order to enhance 5GC NF operations (i.e. policy control and QoS) assisted by NWDAF. The work will firstly identify the specific use cases to be considered, in order to identify the appropriate scope. The work will analyse the result impacts on NWDAF (e.g. the need to understand specific NF functionality), and the compatibility of new solutions wrt existing analytics, in order to determine the need and benefits of new solutions.

Currently, NWDAF can assist in policy control and QoS. For example, NWDAF can derive Service Experience analytics, which can be utilized by the PCF to determine QoS parameters to be applied for a specific service, as described in clause 6.4 of TS 23.288.

The policy control and QoS can be further enhanced with the assistance of NWDAF. For example, the SLA between the operator and the ASP only indicates expected service experience (e.g., MOS>=4), the PCF needs to determine the QoS parameters based on the SLA. However, based its own data and/or knowledge as well optional statistics and predictions from the NWDAF, the PCF may not be able to determine which QoS parameters can achieve the expected service experience. In this scenario, considering NWDAF can gather quite a lot data from 5GC and/or AF and/or OAM and can have wide variety of knowledge, the NWDAF can be considered to provide recommended QoS parameters that can better achieve the SLA requirements.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-84.

# * * * * First change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

# * * * * Next change (All new) * * * *

5.1.y	Use Case #y: NWDAF-assisted QoS recommendation

5.1.y.1	Description

Currently, NWDAF can provide Service Experience analytics to PCF, which can be utilized to determine whether the use of QoS parameters can achieve the service experience requirements, as described in clause 6.4 of TS 23.288 [x].

However, the PCF is not able to update the QoS parameters to achieve the expected service experience requirements, especially when the PCF has no knowledge of the network status and not able to do analytics, which happens to be the functionality of the NWDAF.

In this scenario, considering NWDAF can gather quite a lot data from 5GC and/or AF and/or OAM and can have wide variety of knowledge, it can be studied how the NWDAF can be enhanced to assist the PCF in determining QoS parameters that can achieve the expected service experience requirements.

# * * * * End of changes * * * *