3GPP TSG-SA2 Meeting #106-Ad Hoc-e	S2-2400648r03

22-29 Jan 2024	(revision of xx-yyxxxx)

# Source:	China Telecom

# Title:	New WID: TEI19_NetFailTol; Enhancing Network Failure Tolerance

# Document for:	Information

# Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Enhancing Network Failure Tolerance

Acronym: TEI19_NetFailTol

Unique identifier:

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | x |  |
| No | x | x | x |  | x |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| MINT_Ph2 | SA1 | 970041 | Minimization of Service Interruption During Core Network Failure Phase 2 |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 920062 | Stage 2 of MINT | R17 MINT enables a UE to perform Disaster Roaming to a 5G PLMN in the case of RAN failure. |

# 3	Justification

Scenario 1: UDM/UDR/AUSF is functional and the rest of core network fails. UEs perform Disaster Roaming.

R17 MINT enables a UE to obtain service from a PLMN offering Disaster Roaming service when a Disaster Condition applies to the UE's determined PLMN. However, Disaster Condition only applies to NG-RAN nodes.

Based on SA1 requirements introduced in R19 MINT_Ph2, the application of MINT function should include core network failure. But it should exclude any core network failure that renders the network subject to a disaster unable to authenticate its subscribers.

It is noted that UDM/UDR/AUSF failure is not considered in MINT function and its R19 enhancement.

Scenario 2: UDM/UDR/AUSF fails and the rest of core network is functional. UEs with UE context continue to obtain service from the subscribed PLMN (i.e. the PLMN with UDM/UDR/AUSF failure.) Roaming is not allowed.

UDM/UDR/AUSF failure occurs with the following events reported worldwide,

- Maintenance personnel mistakenly formatted the database hard drive, resulting in the deletion of user subscription data.

- Bugs in UDR software design made it unable to provide service.

- The core network was attacked, leading to the complete deletion of subscription data, formatted NF systems, and the compromised virtualization layer…

These events took hours and even days for the operator to recover user data and resume service. UE authentication and subscription data retrieval for registered UE may be skipped based on operator policy, to reduce the number of impacted UEs due to UDM/UDR/AUSF failure. Registered UEs include: a. UE in RM-REGISTERED state, and b. UE in RM-DEREGISTERED state with a valid 5G-GUTI and UE context. When AMF and SMF determine that there is no available UDM/UDR/AUSF based on the response from the NRF or SCP, the AMF shall be able to bypass UE authentication and subscription data retrieval and the SMF shall be able to bypass subscription data retrieval. By bypassing UE authentication and subscription data retrieval, operators can provide communication services in the case of UDM/UDR/AUSF failure according to local policies.

It is noted that under UDM/UDR/AUSF failure, roaming is not allowed.

# 4	Objective

This new WID aims to enable the following to enhance network failure tolerance. It contains two Work Tasks: WT1 is an enhancement of R17 MINT based on SA1 requirements, and WT2 is not related to R17 MINT.

WT1: Applying Disaster Roaming to core network failure with functional UDM/UDR/AUSF

NOTE:	In WT1, UDM/UDR/AUSF of the PLMN in Disaster Condition needs to be functional to support Disaster Roaming.

WT2: When Nudm_UECM_Registration service is not available, allow the AMF to complete registration procedures for non-roaming UEs previously authenticated and for which the AMF holds a valid UE context. When Nudm_SDM_Get service is not available, allow the SMF to complete PDU Session Establishment procedures for non-roaming UEs.

NOTE:	Feedback from SA3 will be sought before agreeing any CR on WT2.

The expected time for completing this work is 1 TUs. 0.5 time slots in one SA2 meeting for WT1 and 0.5 time slots in next SA2 meeting for WT2.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | - Indicate the support of MINT under core network failure. | SA#106 (Dec 2024) |  |
| TS 23.502 | - Handling of Registration while Nudm_UECM_Registration service is not available. - Handling of PDU Session Establishment procedures while Nudm_SDM_Get service is not available. | SA#106 (Dec 2024) |  |

# 6	Work item Rapporteur(s)

Yinglin Chen, China Telecom, chenyl37@chinatelecom.cn

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

# 9	Supporting Individual Members

Supporting IM name

China Telecom

ZTE

CATT

Inspur

vivo