3GPP TSG-SA WG2 Meeting #160AHE	S2-2400664

# 21 – 29 Jan., 2024, Elbonia

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5254 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on (e)RedCap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2024-01-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on TS38.300 clause 16.13, it has enabled support of RedCap and eRedCap NR devices. Currently, the NG-RAN can identify a RedCap UE or a (e)RedCap UE and send NR RedCap indication to the AMF. To identify traffic to/from UEs accessing over NR RedCap or NR eRedCap, e.g. for charging differentiation, the network needs to enable functionality of NR (e)RedCap UEs differentiation. This CR proposes to clarify the functionalities for (e)RedCap UE differentiation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In order to enable this, introduce the following: - Clarify that the NG-RAN can provide NR RedCap or NR eRedCap indication to the AMF for (e)RedCap UE differnetiation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network cannot differentiate eRedCap UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.41 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*******Start of changes*******

## 5.41	NR (e)RedCap UEs differentiation

This functionality is used by the network to identify traffic to/from UEs accessing over NR RedCap or NR eRedCap, e.g. for charging differentiation.

An NR (e)RedCap UE using NR shall provide an NR (e)RedCap indication to the NG-RAN during RRC Connection Establishment procedure as defined in TS 38.300 [27].

When the UE has provided an NR (e)RedCap indication to the NG-RAN during RRC Connection Establishment, the NG-RAN shall provide an NR (e)RedCap Indication to the AMF in the Initial UE Message (see clause 4.2.2.2.1 of TS 23.502 [3] and TS 38.413 [34]).

When the AMF receives an NR (e)RedCap Indication from NG-RAN in an Initial UE Message, the AMF shall store the NR (e)RedCap Indication in the UE context, consider that the RAT type is NR (e)RedCap and signal it accordingly to the SMSF during registration procedure for SMS over NAS, to the SMF during PDU Session Establishment or PDU Session Modification procedure. The PCF will also receive the NR (e)RedCap RAT type indication when applicable, from the SMF during SM Policy Association Establishment or SM Policy Association Modification procedure.

During handover from E-UTRA to NR, the target NG-RAN (i.e. gNB) provides the NR (e)RedCap indication to AMF in NGAP Path Switch Request message during Xn handover, or NGAP Handover Request Acknowledge message during N2 handover (including intra 5GS N2 handover and EPS to 5GS handover) based on the UE capability information provided by the source RAN to the target RAN as specified in TS 38.300 [27].

The NFs interacting with CHF shall include the NR (e)RedCap as RAT type.

Upon AMF change, the source AMF shall provide the "NR (e)RedCap Indication" to the target AMF.

Editor’s note: the above terminology on “(e)RedCap Indication” may need alignment with RAN3.

*******End of changes*******