3GPP TSG-SA WG2 Meeting #160AH-e	S2-2400693r01

Online, January 22–29, 2024

Source:	NEC

Title:	KI #2, New Sol: Support of store and forward in EPS.

Document for:	Approval

Agenda Item:	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: The contribution proposes a solution to Key Issue #2: KI#2: Support of store and forward in EPS.

1. Introduction

This contribution proposes a solution to Key Issue #2: KI#2: Support of store and forward in EPS.

2. Proposal

It is proposed to agree the following solution to 3GPP TR 23.700-29.

* * * Start of change * * * *

## 6.x	Solution #X: Support of store and forward in EPS

### 6.x.1	Key Issue mapping

Editor's Note:		This clause lists the key issue(s) addressed by this solution.

This solution addresses the scenarios mentioned in the KI#2.

### 6.x.2	Functional Description

Editor's Note:		This clause will describe the solution principles and assumptions for corresponding key issue(s). Sub-clause(s) may be added to capture details.

This solution follows following basic principles and assumptions:

- The eNB is in the payload of the satellite.

- The eNB broadcasts about the availability to feeder link to the UE indicating whether the feeder link is available or not.

- When the eNB detects that the feederlink is not available and the eNB has UE context for the UE supporting S&F then the eNB preserve the UE context.

- The UE and the EPS exchange capability whether the UE and the network supports store and forward feature. The HSS stores whether the UE is subscribed for the Store and Forward feature. The network sends in the attach procedure or TAU procedure whether the UE is accepted for the store and forward feature.

- The UE initiates attach procedure or TAU procedure or PDN connection establishment procedure when the UE determines that the feeder lin is connected.

- When the UE sends the MO data using the control plane IoT procedure the eNB stores the MO data if the feeder link is not available. The eNB forwards the stored the MO data to the MME when the feederlink is available.

- When the MME determines that the feeder link is lost then it indicates to the S-GW. The S-GW stores the MT data. When the feeder link connection is established then MME indicates to the S-GW and S-GW transmits the data to the MME and MME in turns sends this data to the UE using the existing mechanism as defined in the TS 23.401.

### 6.x.3	Procedures

Figure 6.X.3: PDN connection establishment procedure and MO and MT data transmission using control plane signaling for the UE supporting S&F operation.

The following steps define the procedure to establish PDN connection supporting S&F data transmission using control plan CIOT signgalling.:

- A UE supporting S&F operation are attached to the EPS network .

NOTE: The attach procedure can be performed when the UE was in Terrestrial network coverage.

EN: Attach procedure when service link and feeder link are not available simultaneously will be brought in next meeting.

- The UE has MO data to send. The UE establishes a RRC connection or sends the RRCEarlyDataRequest message as defined in TS 36.300 [5] and sends as part of it an integrity protected NAS PDU. The NAS PDU carries the EPS Bearer ID and encrypted Uplink Data.

NOTE: the UE can establish RRC connection and send MO data using control plane CIoT irrespective of the status of feeder link availability.

- If the eNB detects that the feeder link is not available the eNB stores NAS PDU.

4-5)The eNB detects that the feeder link is available then the eNB forwards the NAS PDU in the INITIAL UE CONTEXT S1-AP message to the MME.

6) the MME sends the DATA PDU to the P-GW via S-GW.

7) The eNB detects that the feeder link is lost. The eNB stores the UE context.

8) When the P-GW gets the DATA PDU for the UE then it forwards the DATA PDU to the MME via S-GW.

9) The MME detects that Feeder link is lost. The MME stores the DATA PDU.

10-11) The MME detects that the feeder link is available then the initiates paging procedure. Once the connection is established the MME encrypts the DATA PDU to NAS PDU and send it to the UE as described in sub clause 5.3.4B.3 of TS 23.401

.

### 6.x.4	Impacts on existing services, entities and interfaces

### UE, eNB, MME and S-GW

EN: The detail impact on the UE, eNB, MME and S-GW is FFS.

* * * End of change * * * *