3GPP TSG-SA WG2#160-Ad Hoc-e	S2-2400720r07

Electronic Meeting, 22-29 January 2024	(was S2-240xxxx)

Source:	Qualcomm Incorporated

Title:	23.700-66: KI#1: New solution to support NG-RAN node QoS flow PDU session and UE level energy related information report

Document for:	Discussion/Approval

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This contribution proposes a solution to support per QoS flow, PDU session and UE level energy related information exposure.

# 1.	Discussion

This contribution proposed to address the KI#1 to support the NG-RAN node QoS flow, PDU session and UE level energy related information.

# 2.	Text proposal

It is proposed to agree the following solution to solve KI#1 in TR 23.700-66.

# >>>>BEGINNING OF CHANGES (new solution) <<<<

## 6.x	Solution #X: NG-RAN node QoS flow PDU session and UE level energy related information report

### 6.x.1	Key Issue mapping

Table 6.X.1-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | … | X |
| Solution x: NG-RAN node QoS flow PDU session and UE level energy related information report | X |  |  |  |
| … |  |  |  |  |

### 6.x.2	Functional Description

NG-RAN may activate the PDU session / QoS flow level energy related information reporting procedure for specific UE, the procedure is triggered by a request from SMF.

PDU session / QoS flow level energy related information is provided by NG-RAN to SMF and will include the following parameters:

- The specific PDU session id

- PDU session level NG-RAN Energy Consumption: The PDU session level NG-RAN Energy Consumption is calculated in NG-RAN. The formula can use the total Energy Consumption (EC) of the NG-RAN node serving the UE for the specific slice (ECRANonlyns) divided by the number of PDU sessions of a specific slice that are served by the NG-RAN, or taking into account the Data Volume for specific PDU session, e.g., The EC of NG-RAN ECRANonlyns is defined in TS 28.554.

- (optional) QoS Flow id

- (optional) QoS flow level NG-RAN Energy Consumption: The EC of the NG-RAN node serving the UE for the specific slice (ECRANonlyns) divided by the number of QoS flows that are served by the NG-RAN node for this slice, or taking into account the the Data Volume for the specific QoS flow, e.g., The EC of NG-RAN ECRANonlyns is defined in TS 28.554.

Editor’s NOTE:	Whether NG-RAN can get the NG-RAN Energy Consumption is FFS

Editor’s NOTE:	Whether NG-RAN can provide the above parameters to SMF (via AMF) are FFS.

NG-RAN reports the corresponding energy consumption related information to SMF (via AMF). SMF generates CDRs to the CHF to report the NG-RAN energy consumption related information. Charging system may also generate the UE level NG-RAN energy related information by summing the PDU session level energy related energy consumption information for the specific UE.

Editor’s NOTE:	Whether SMF can get the 5GC layer Energy consumption information for a specific QoS flow or PDU session is FFS.

NOTE: The way the charging system exposes energy consumption related information to the 3rd party is out of SA2’s scope and will be studied by SA5.

### 6.x.3	Procedures

#### 6.x.3.1	NG-RAN Energy Info report procedure

Figure 6.x.3-1	NG-RAN Energy info report procedure

Step 0: The UE subscription per DNN and per S-NSSAI in UDM of the HPLMN indicates as defined in clause 5.6.1 of TS 23.501 the subscribed policy for report of Energy Consumption information and the specific granularity of reporting i.e. per PDU session Energy Consumption, (optional) per QoS Flow Energy Consumption. SMF stores the policy in attributes of the PDU session.

Step 1: At the establishment of PDU session, the SMF sends to NG-RAN the policy for reporting of Energy Consumption information that includes the granularity of reporting i.e per PDU session Energy Consumption, (optional) per QoS Flow Energy Consumption.

Step 2: AMF forwards the N2 SM request to NG-RAN as part of PDU session establishment procedure.

Step 3-4: NG-RAN sends the NG-RAN Energy info report in N2 SM messages sent to SMF. The NG-RAN Energy info report includes the following parameters:

-	PDU session id

-	PDU session level NG-RAN Energy Consumption

-	QoS Flow id (optional depending on the granularity of the policy)

- 	QoS flow level NG-RAN Energy Consumption (optional depending on the granularity of the policy)

-	start time stamp,

-	end time stamp.

The NG-RAN Energy info is contained in at least the following NGAP messages:

- PDU SESSION RESOURCE RELEASE COMMAND

- PDU SESSION RESOURCE NOTIFY

- PDU SESSION RESOURCE MODIFY INDICATION

- UE CONTEXT RELEASE COMMAND

- HO related N2 signaling for Xn and N2 based HO procedure

And possibly a new NGAP message needs to be defined by RAN3 in order to report the NG-RAN Energy info to assist "partial CDR" generation and if NG-RAN is instructed to also make periodic reports if no event has triggered a report before the period expires.

Editor’s Note:	A coordination with RAN WGs is required to determine whether the NG-RAN Energy information can be obtained and provided by the RAN.

step 5: SMF sends the NG-RAN Energy info report to Charging system.

### 6.x.4	Impacts on existing services, entities and interfaces

SMF

- Support new N2 signalling to send NG-RAN energy consumption info request, or

- Enhance the existing N2 SM message to include the NG-RAN energy consumption info request

NG-RAN

- Report the NG-RAN energy consumption info for specific QoS flow or PDU session or both,

- Support new N2 report signalling, or

- Enhance the existing N2 SM message to report the NG-RAN energy info.

CHF

- Receives NG-RAN Energy info report as CDR from SMF.

# >>>>END OF CHANGES<<<<