3GPP SA WG2 Meeting #160AHE	S2-2400759

22 – 29 January, 2024, Electronic

Source:	NEC, Intel

Title:	New KI: 5GS enhancements for traffic handlings between DNs

Document for:	Approval

Agenda Item:	19.9

Work Item / Release:	FS_eEDGE_5GC_ph3 / Rel-19

Abstract of the contribution: The contribution proposes a key issue about WT#2 in TR 23.700-49.

1. Introduction

This paper is to propose a new key issue according to WT2

WT#2) Whether and how to support traffic sent to/from PSA after being processed by Edge Hosting Environment, e.g. when there is no communication possibility between the local part of the DN and central part of the DN (e.g. due to usage of private IP address, lack of secure tunnel), whether and how to support  traffic being routed between two EASs located in two different Edge Environments when there is no pre-established communication path between the two local part of the DNs. Below scenario will be addressed:

-	Scenario: Support the User Plane routing and traffic steering of Application traffic between the local DN and a cloud server located in the central DN.

NOTE 2: For WT#2, all connectivity models in TS 23.548 need to be addressed and a single solution for all models is preferred.

2. Proposal

It is proposed to include the below changes into TR 23.700-49.

* * * Start of change * * * *

## 5.X	Key Issue #X: 5GS enhancements for traffic routing between two local DNs

### 5.X.1	Description

This key issue is to study 5GS enhancement for Edge Computing on traffic routing aspect between two EASs in two different local DNs serving two UEs in case there is no direct connectivity between two local DNs. The following aspects need to be studied:

Whether and how to support the traffic routing between two local DNs serving two different UEs in case there is no direct connectivity between the two local DNs.

NOTE: All connectivity models in TS 23.548 need to be addressed and a single solution for all models is preferred.

* * * End of change * * * *