SA WG2 Meeting #160-Ad-Hoc-e	S2-2400787r04

# January 22th – 29th, 2024; Elbonia               		       (revision of S2-220xxxx)

Source: 	OPPO

Title: 	23.700-54: New Key issue X: Policy enhancement for dual steer device

Document for: 	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes to add key issue on Policy enhancement for dual steer device to the FS_MASSS.

1. Proposal

The WT-D 1.3 of FS_MASSS SID includes the following:

WT#1.3: Session management enhancements and policies:

-	Study whether and how to enhance network policies provided by the HPLMN to the DualSteer Device and within the network to support DualSteer Devices.

-	Study whether and how to enhance session management procedures for initial steering and potential subsequent switching. Traffic policies are in full control of the HPLMN. For sessions subject to potential switching, assuming data anchoring in the HPLMN or, in case of PNI-NPN plus PLMN, assuming data anchoring in the PNI-NPN, study how to select the PSA UPF(s) in the HPLMN to allow routing the traffic across 3GPP accesses towards the same PSA UPF during the switching.

Whether and how to enhance policy to realize the session management above should be a standalone KI. Thus, it is proposed to agree on the following Key issue description in the TR of FS_MASSS.

Start of Changes (all new text below)

# 5	Key Issues

## 5.x	Key Issue #x: Policy enhancement for dual steer device

### 5.x.1	Description

It is assumed all the accesses from one Dual Steer device should be anchored in a HPLMN UPF. In order to realize it, it may need enhancement to UE policy and network policy. This Key Issue#x aims to study:

1.	whether and how to enhance policies applied to a dual-steer device and to network side to support traffic steering, switching. It might consider the following aspects:

-   what policies/rules are used for controlling steering and switching, e.g. whether they are based on ATSSS rules, MAR rules, PCC rules or not.

-   whether and what enhancements are needed in UE policy e.g. UE Route Selection Policies (URSP) to allow the UE of a DualSteer device to establish a PDU Session for DualSteer.

-   Whether and what enhancements are required in the policy subscription data to support generation of rules/policies for DualSteer.

-   What policy enhancement for Network selection

-   In the case of DualSteer device in two PLMNs, how the policies in two PLMNs cooperate to support the traffic steering and switching.

- what policies UE should be provided to select first access network and second access network for it to perform more than one registration and steering/switching ?

- which conditions trigger a DualSteer device to establish a connection to a second 3GPP access network.

-  whether and what new policy information is required for a DualSteer device to establish a connection to a second 3GPP access network that is to be used for steering and switching.

NOTE 1: The policies should be in full control of HPLMN

NOTE 2: Traffic splitting only applies to a dual steer device with two separate UEs

NOTE 3: If any PLMN selection enhancement is identified, SA2’s role could be limited to study system level impacts and/or trigger CT1 to handle the work.

2.	How to configure the enhanced polices to Dual Steer device and network respectively.

3.  How to apply the enhanced policies to relevant session management procedure to realize the two user plane paths via 3GPP accesses anchored in HPLMN UPF.

End of Changes