3GPP TSG-WG SA2 Meeting #160AH e-meeting 	S2-2400790r08

22 - 29 January, 2024, Electronic meeting	(revision of S2-24xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1043 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on ground truth retrieval of Location Accuracy Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vivo, Nokia, Nokia Shanghai-Bell, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To ensure accurate training of the model for LCS, it is essential to acquire ground truth location information. This information should align with the location data obtained from the LCS system, especially for a given UE in a given time or period. The existing solution, assumed to be provided by O&M, is unable to furnish this crucial location information. Consequently, it is imperative to clearly define the method by which the NWDAF procures the ground truth location information. There are several options: - Option#1, from DCAF. DCAF may obtain GNSS input data directly from the UE application. However, this poses risks to location information retrieval due to considerations around UE privacy and SA4's future efforts on DCAF. - Option#2, from AF, and AF may collect GNSS information which is out of 3GPP scope. - Option#3, from PRUs. Leveraging PRUs, deployed to assist the LCS system in calibrating positioning methods and improving accuracy, is another option. However, this approach involves fetching PRU IDs (i.e., UE IDs), which differs from the concept of obtaining UE IDs by utilizing AOI as a filtering criterion from the AMF. It is proposed to proceed with Option#2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NWDAF collects ground truth location information from AF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to obtain ground truth location information remains unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.17.2, 6.17.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change* * * *

### 6.17.2	Input Data

The input data for the analytics ID is listed in Table 6.17.2-1.

Table 6.17.2-1: Data Collected by NWDAF for Location Accuracy Analytics

| Information | Source | Description |
| --- | --- | --- |
| UE IDs | AMF | (List of) SUPI(s) |
| Finer granularity UE location | LCS NOTE 1 | UE location information. |
| >UE location |  | Geographical location or location in local coordinate (as defined in TS 23.032 [34]) |
| >positioning methods |  | The positioning methods used by the LMF when the location measurement was made |
| >LCS QoS |  | LCS QoS accuracy as defined in clause 4.1b in TS 23.273 [39] |
| >Indoor/outdoor indication |  | Indication of indoor/outdoor when LMF can distinguish indoor/outdoor environment |
| >NLOS/LOS indication |  | Indication whether the UE measurements were based on LOS/NLOS measurements |
| UE location (ground truth) | AF NOTE 2, NOTE 3 | Timestamped UE positions |
| >UE location |  | GNSS location information |
| >Timestamp |  | Time stamp for the UE location |
| >Accuracy |  | The accuracy of the GNSS location information |
| NOTE 1: The procedure to collect location data using LCS is described in clause 6.2.12. NOTE 2: How the AF obtains timestamped GNSS location information of a UE ID is out of 3GPP scope. Whether to trust the UE location from AF is up to the implementation. NOTE 3: When AF is deployed in the untrusted domain, NEF is employed to mediate the interactions between NWDAF and AF via the Naf_EventExposure_Subscribe service specified in clause 5.2.19.2 of TS 23.502 [3], as described in clause 6.2.8.2.3. |  |  |

# * * * * Second change* * * *

### 6.17.4	Procedures to request Location Accuracy Analytics

Figure 6.17.4-1: Location accuracy analytics retrieval

Pre-condition: NWDAF has a trained supervised ML model for predicting location accuracy. In the training phase, the NWDAF consumes input data as listed in clause 6.17.2. To pre-train the ML model, NWDAF may collect input data based on area of interest (AoI). The AoI is determined by NWDAF.

0-1.	NWDAF collects UE IDs from AMF as defined in Table 6.17.2 via the AMF event exposure service with AoI as filtering criteria.

0-2.	NWDAF collects the UE location information using LCS (location estimate, Position Methods Used, Indoor/Outdoor indication, NLOS/LOS indication) as described in clause 6.2.12 and Table 6.17.2 with UE ID as filtering criteria.

0-3.	NWDAF collects ground truth UE location information (i.e. timestamped GNSS location information) from AF.

0-4.	NWDAF prepares the analytics for statistics and/or pre-trains the ML model for prediction.

1.	The Analytics consumer, e.g. LMF, requests from NWDAF location accuracy statistics or predictions, giving as inputs the corresponding Analytics ID and UE location estimate (in the form of horizontal and/or vertical location) or corresponding area (cell ID/TAI). The analytics request to the NWDAF may be a one time or a subscription request. Other inputs such as UE positioning method, applicable period can be assistance data.

NOTE 1:	It is assumed that the analytics consumer determines a location. For instance, the LMF can be triggered by LCS client request to determine a location and then use location accuracy analytics to inquire the accuracy of the determined location.

NOTE 2:	Both Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo services can be used for step 1 above. The Nnwdaf_AnalyticsSubscription service can be used by a service consumer to receive notifications about location accuracy, e.g. when a change is detected.

2.	The NWDAF uses the trained ML model and provides the location accuracy as output to the analytics consumer.

NOTE 3:	The NWDAF can derive the accuracy based on the model without further input data.

# * * * * Third change* * * *