3GPP TSG-WG SA2 Meeting #160AH e-meeting 	S2-2400795r06

22 - 29 January, 2024, Electronic meeting	(revision of S2-24xxxxx)

Source:	vivo,NEC

Title:	New Solution on Enabling S&F operation with C-SGN onboard

Document for:	Approval

Agenda Item:	19.1

Work Item / Release:	FS_5GSAT_Ph3 / Rel-19

Abstract: A new solution to enable regenerative payload as satellite access.

# 1. Introduction

This solution addresses Key Issue #2 about the support of Store and Forward Satellite operation.

## 5.2 Key Issue #2: Support of Store and Forward Satellite operation ### 5.2.1 Description S&F Satellite operation is especially suited for the delivery of delay-tolerant/non-real-time satellite services (i.e. CIoT/MTC, SMS). To support S&F Satellite operation for such services, it is proposed to study the following items: - If applicable, what are the minimum necessary set of core network elements/network functions that should be placed on board the satellite(s) for the intended service(s); - Whether and how to trigger S&F Satellite operation, and how to execute S&F Satellite operation; - What enhancements on the related UE and network procedures are needed to support S&F Satellite operation, including: - Whether to inform the UE when the S&F Satellite operation is applied or not. NOTE 1: S&F for IoT NTN will be studied first and if there is remaining time available NR NTN can then be studied. NOTE 2: Coordination with SA3-LI on LI aspects is needed. NOTE 3: The candidate solutions need to indicate which service(s) they are addressing (CIoT CP Optimizations, CIoT UP Optimizations, SMS).

S&F operation is an operating mode when service link and feeder link cannot be available at the same time. There are 6 potential architectures can be discussed to support S&F operation.

Fig.1: Six potential architectures supporting S&F operation.

This solution considers the case of  RAN and C-SGN (MME+S-GW+P-GW) onboard with no available ISLs, i.e. UE can only be served by one satellite, and any single procedure involving interactions among UE, the onboarding RAN+C-SGN, and CN NFs on the ground needs one round of satellite orbit to be completed.

It is proposed to capture the following changes to TR 23.700-29.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |  |
| X |  | X |  |  |

## 6.X	Solution #X: Enabling S&F operation with C-SGN onboard

### 6.X.1	Description

This is a candidate solution for Key Issue 2 - "Support of Store and Forward Satellite operation".

To facilitate Store-and-Forward (S&F) operations for delay-tolerant services, the assumption is made to activate the entire C-SGN on board. The architecture is derived from Annex L, specifically the "Optimised EPS Architecture option for CIoT" outlined in TS23.401. In this architecture, the C-SGN incorporates the functionalities of MME, S-GW, and P-GW. Furthermore, it supports all the requisite interfaces to accommodate various types of delay-tolerant services.

Fig.1: the architecture and interfaces of C-SGN onboard

As shown in Fig.1, the C-SGN onboard architecture is assumed that:

- A UE is served by only one satellite;

- the onboarding C-SGN functionalities are only used for S&F operation, i.e. any UE that connects to the satellite without S&F operation cannot be served by the onboarding C-SGN;

- S-GW is responsible for storing/forwarding the data (or non-IP data) for CP CIoT EPS optimizations, UP CIoT EPS optimizations;

- MME is responsible for storing/forwarding the data for SMS services;

- ISL is not used to transfer the contexts among satellites;

Therefore, to finalize the registration of a UE that supports S&F operations within the network, multiple orbital cycles are required. The number of cycles is determined by the involvement of various procedures, encompassing the UE, onboarding functionalities (i.e. RAN, MME, S-GW, P-GW), and other CN NFs on the ground such as HSS, PCRF, AF, etc. This is due to the service link serves:

- AS layer management between UE and RAN

- NAS layer management between UE and MME

And feeder link serves,

- CN internal functionalities management between C-SGN onboard and CN NFs on the ground, i.e. MME/P-GW and HSS, P-GW and HSS/PCRF, P-GW and DN;

This architecture needs enhancements on

- Availability detection of service link/feeder link

- S&F operation triggered condition detection

- Signalling procedure development on pause/recover mechanisms regarding service link’s availability, NAS suspend/resume

- New procedure development on store/forward mechanism regarding UL/DL data transmission

### 6.X.2	Procedures

#### 6.x.2.1	Procedures of NAS Pause/Recovery of S&F Operation

#### This procedure can be applied to any NAS interactions between UE and MME as listed in TS 23.401. An example of how to utilize NAS Pause/Recovery to complete the initial attach procedure is given in clause 6.x.2.3.

- Pre-condition: UE has established RRC connection with RAN, and RAN has established S1 interface with MME

- MME decides to pause NAS.

The trigger condition is when MME receives S1 messages from RAN/NAS message from UE and the procedure is to detect the necessaries to interact with CN NFs on the ground. For example, when UE sends an Attach Request, MME will interact with HSS to initiate the authentication and authorization procedures, as HSS is on the ground, the procedure can proceed only when the feeder link is available.

- MME sends NAS Pause request to the UE with GUTI.

Editor’s Note: regarding how the MME handles NAS pause/recovery for initial UE is FFS.

- RAN may Pause AS if an indication is provided by MME in step 2 to save UE energy consumption. The AS pause can be established with the current RAN mechanism, e.g. RAN sends RRCRelease message to UE and moves UE to RRC_IDLE mode.

NOTE: 	whether the AS pause/recovery procedure can be proceeded via new RAN mechanism or not depends on RAN.

- UE starts to pause NAS during the procedure even though the service link is still available based on the request as received in step 2.

Pause NAS means any concurrent NAS messages should be stopped until receive a Recover Request from MME.

- UE sends a Pause NAS complete message to MME.

- When the RAN orbits back to serve the UE, the RAN and UE recover the AS connection, the resumption can be triggered by UE (more power saving), or by RAN. When the AS connection is back, the RAN notifies AMF that the UE is back. The AS recovery can be established with the current RAN mechanism, e.g. RAN sends RRCSetup message.

- MME decides to recover NAS, i.e. continue the paused NAS procedure as specified in Step 2 after receiving the notification from RAN.

For example, e.g. when MME receives the authentication request from HSS and the service link is available again, the MME will send a notification to the UE to indicate UE the concurrent NAS signaling can be continued.

- MME sends NAS Recovery request to UE with GUTI.

- UE sends back a Recovery NAS complete message to MME. To make full use of the service link available period, the MME/UE may include NAS messages within the recovery messages.

Afterward, normal NAS messages can be exchanged between UE and MME.

#### 6.x.2.2	Procedure of Store/Forward of Data transmission

This procedure applies to CP CIoT EPS Optimizations, UP CIoT EPS Optimizations, non-IP data transmissions.

Editor’s NOTE: SMS data transmission regarding S&F is FFS

##### 6.x.2.2.1	Procedure of UL Data transmission

Editor’s NOTE: UL data transmission when UE is in the ECM-IDLE state is FFS

- Pre-condition: UE is in ECM-CONNECTED state and service link is available.

- UE sends UL data via RAN and S-GW.

- S-GW stores the UL data

- S-GW detects the service link is unavailable and stops receiving the UL data. The unavailability detection of service link can be based on pre-configured information, or MME notifications.

- S-GW detects feeder link is available. The availability detection of feeder link can be based on pre-configured information, MME notifications, or gateway notifications (out of 3GPP scope). At the same time the connection between CN C-SGN onboard and CN NFs on the ground is recovered.

- S-GW forwards the stored data to P-GW, and P-GW forwards to the ground.

##### 6.x.2.2.2	Procedure of DL data transmission

Editor’s NOTE: DL data transmission when UE is in the ECM-IDLE state is FFS.

- Pre-condition: UE is in ECM-CONNECTED state and feeder link is available.

- P-GW and S-GW receives DL data.

- S-GW stores the DL data

- S-GW detects the feeder link is unavailable and stops receiving the DL data. The unavailability detection of service link can be based on pre-configured information, MME notifications, or gateway notifications (out of 3GPP scope).

- S-GW detects feeder link is available. The availability detection of feeder link can be based on pre-configured information, MME notifications. At the same time, connection between UE and MME is resumed.

- S-GW forwards the stored data to RAN, and RAN forwards to the UE.

#### 6.x.2.3	Example of initial attach procedure

This provides an example of how the UE can complete the attach procedure with C-SGN onboard. Generally, if the UE hasn’t established any connection with the network before, it needs at least 3 rounds of satellite orbiting periods, each round of orbiting UE and network can complete some parts of the procedure.

##### 6.x.2.3.1	1st round orbiting

- When service link is available, UE initiates the RRC connection with RAN and sends Attach Request to MME.

- MME pause the NAS because MME needs to wait for the response from HSS

- When feeder link is available, MME initiates the interactions with HSS for the authentication and authorization.

- MME waits for the Auth response from UE.

##### 6.x.2.3.2	2nd round orbiting

- When service link is available, MME recovers the interactions with UE to complete the authentication and authorization procedure.

- MME pause the NAS because MME needs to wait for the response from HSS, S-GW-P-GW and etc.

- When feeder link is available, MME recovers the interactions with CN NFs on the ground to complete e.g. UE context setup, PDN session establishment, optionally dynamic PCC rules retrieval, IP address registration in HSS.

- MME waits for the attach response from UE.

##### 6.x.2.3.3	3rd round orbiting

- When the service link is available, MME resumes the interactions with UE to complete the attach procedure. Afterwards, UE can send UL data.

### 6.X.3	Impacts to Services, Entities and Interfaces

MME:

- Detection of service link/feeder link availability

- Initiate NAS pause/recover procedure when CN interactions with CN NFs on the ground is needed

- Pause/recover NAS based on service link’s availability

RAN:

- Optionally, initiate AS pause procedure if NAS is paused, and initiate AS recovery procedure

UE:

- Pause/recover NAS

- Optionally, Pause/recover AS

S-GW:

- Detection of service link/feeder link availability

- Store/Forward UL/DL data

# * * * End of changes * * * *