3GPP TSG-WG SA2 Meeting #160-AH-E	S2-2400807r02

# Electronic Meeting, Jan 22th – 26th		(revision of S2-2312965)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4644 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 Corrections on Offload Identifier and misalignments in services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - For a HR-SBO PDU Session, the H-SMF may send Offload Identifier(s) and corresponding VPLMN Specific Offloading Information to V-SMF, and source V-SMF may send the above identifier(s) and information in SM context to target V-SMF during the intra PLMN handover procedure. This contribution proposes to add Offload Identifier(s) in Nsmf_PDUSession_Create/Update and Nsmf_PDUSession_ContextRequest Service to align with the descriptions in TS 23.548. - During inter PLMN handover, informations related to HR-SBO (e.g. VPLMN Specific Offloading Information) should not be transferred to target V-SMF since HR-SBO should be re-authorized for the target PLMN. - As described in clause 6.7.3 in TS 23.548, the EAS IP replacement information and the related target DNAI may be included in SM context in V-SMF insertion/change/removal case. And for intra VPLMN HO, the source AMF can provide the target AMF with the HR-SBO allowed indication. - For EDI and EACI subscribe services, the descriptions are misaligned with TS 23.548. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add Offload Identifier(s) in Nsmf_PDUSession_Create/Update and Nsmf_PDUSession_ContextRequest service operations. - Clarify that the HPLMN address information and DNS Server address provided by HPLMN are only transferred in intra-PLMN V-SMF change and V-SMF insertion case. - Add EAS IP replacement information and related target DNAI in Nsmf_PDUSession_ContextRequest service operation. - Clarify the inputs for Nnef_EASDeployment_Subscribe and Nnef_ECSAddress_Subscribe service operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignments with TS 23.548. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.8.2.2, 5.2.8.2.3, 5.2.8.2.10, 5.2.6.26.6, 5.2.6.37.5, 5.2.8.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.503 CR 1226 TS 23.548 CR 0202 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.2.8.2.2	Nsmf_PDUSession_Create service operation

Service operation name: Nsmf_PDUSession_Create.

Description: Create a new PDU Session in the H-SMF or SMF or create an association with an existing PDN connection in the home SMF+PGW-C.

Input, Required: SUPI, V-SMF ID or I-SMF ID, V-SMF SM Context ID or I-SMF SM Context ID, DNN, V-CN Tunnel Info or I-UPF Tunnel Info, addressing information allowing the H-SMF to request the V-SMF to issue further operations about the PDU Session or addressing information allowing the SMF to request the I-SMF to issue further operations about the PDU Session, Serving Network (PLMN ID, or PLMN ID and NID, see clause 5.18 of TS 23.501 [2]).

Input, Optional: S-NSSAI, Alternative S-NSSAI, PCO, Requested PDU Session Type, 5GSM Core Network Capability, Requested SSC mode, PDU Session ID, Number Of Packet Filters, UE location information, subscription get notified of PDU Session status change, PEI, GPSI, AN type, PCF ID, PCF Group ID, DNN Selection Mode, UE's Routing Indicator optionally with Home Network Public Key identifier or UDM Group ID for the UE, Always-on PDU Session Requested, Control Plane CIoT 5GS Optimisation Indication, information provided by V-SMF related to charging in home routed scenario (see TS 32.255 [45]), AMF ID, EPS Bearer Status, extended NAS-SM timer indication, DNAI list supported by I-SMF (from I-SMF to SMF), HO Preparation Indication. MA PDU request indication, MA PDU Network-Upgrade Allowed indication, Indication on whether the UE is registered in both accesses; QoS constraints from the VPLMN (as defined in clause 5.7.1.11 of TS 23.501 [2]), Satellite backhaul category, Notification of the SM Policy Association Establishment and Termination, PCF binding information, Disaster Roaming service indication, HR-SBO request indication, VPLMN EASDF address, [ECS Address Configuration Information associated with PLMN ID of visited network], Indication of UE supports non-3GPP access path switching.

Output, Required: Result Indication and if success a SM Context ID and in addition: QFI(s), QoS Profile(s), Session-AMBR, QoS Rule(s), QoS Flow level QoS parameters if any for the QoS Flow(s) associated with the QoS rule(s), H-CN Tunnel Info or PSA UPF Tunnel Info, Enable pause of charging indication, Selected PDU Session Type and SSC mode.

Output, Optional: PDU Session ID, S-NSSAI, Cause, PCO, UE IP address, IPv6 Prefix allocated to the PDU Session, information needed by V-SMF in the case of EPS interworking such as the PDN Connection Type, EPS bearer context(s), linked EBI, Reflective QoS Timer, Always-on PDU Session Granted, information provided by H-SMF related to charging in home routed scenario (see TS 32.255 [45]), DNAI(s) of interest for this PDU Session (from SMF to I-SMF), indication of multi-homing support (from SMF to I-SMF). MA PDU session Accepted indication, Indication on whether Small Data Rate Control applies or not, HR-SBO authorization result, VPLMN Specific Offloading Information for HR-SBO, Offload Identifier(s), HPLMN DNS Server address, HPLMN address information (e.g. H-UPF IP address on N6), Internal Group Identifier(s).

The V-SMF SM Context ID in the Input provides addressing information allocated by the V-SMF (to be used for service operations towards the V-SMF for this PDU Session).

The I-SMF SM Context ID in the Input provides addressing information allocated by the I-SMF (to be used for service operations towards the I-SMF for this PDU Session).

See clause 4.3.2.2.2 clause 4.11.1.2.2 clause 4.11.1.3.3 and clause 4.24 for details on the usage of this service operation.

See clauses 4.22.2.2 and 4.22.3 for detailed usage of this service operation for ATSSS.

See clause 6.7 of TS 23.548 [74] for HR-SBO request indication, HR-SBO authorization result, VPLMN EASDF address, VPLMN Specific Offloading Information for HR-SBO, Offload Identifier(s), HPLMN DNS Server address, HPLMN address information.

5.2.8.2.3	Nsmf_PDUSession_Update service operation

Service operation name: Nsmf_PDUSession_Update.

Description: Update the established PDU Session.

This service operation is invoked by the V-SMF towards the H-SMF in the case of UE or serving network requested PDU Session Modification in order for the V-SMF to transfer the PDU Session Modification request. It can also be invoked by the V-SMF to indicate to the H-SMF that the access type of the PDU session can be changed. This service operation is also invoked by the V-SMF to insert or remove UL CL or BP controlled by the V-SMF.

This service operation is invoked by the I-SMF towards the SMF in the case of UE or serving network requested PDU Session Modification in order for the I-SMF to transfer the PDU Session Modification request. It can also be invoked by the I-SMF to indicate to the SMF that the access type of the PDU session can be changed. This service operation is also invoked by the I-SMF towards the SMF to insert or remove ULCL or BP controlled by the I-SMF or to report usage offloaded via UL CL or BP controlled by I-SMF.

This service operation is invoked by the H-SMF towards the V-SMF for both UE initiated and HPLMN initiated PDU Session Modification and PDU Session Release cases in order to have the SM PDU Session Modification request or SM PDU Session Release request sent to the UE. It can also be invoked by the H-SMF towards the V-SMF to release the 5GC and 5G-AN resources in e.g. handover from 5GC-N3IWF to EPS and from 5GS to EPC/ePDG, wherein the UE is not notified.

This service operation is invoked by the SMF towards the I-SMF for both UE initiated and SMF/PCF initiated PDU Session Modification and PDU Session Release cases in order to have the SM PDU Session Modification request or SM PDU Session Release request sent to the UE. It can also be invoked by the SMF towards the I-SMF to release the 5GC and 5G-AN resources in e.g. handover from 5GC-N3IWF to EPS and from 5GS to EPC/ePDG, wherein the UE is not notified. This service operation is also invoked by the SMF towards the I-SMF to provide updated N4 information or updated DNAI list of interest for this PDU Session when SMF receives updated PCC rules.

This service operation is invoked by the V-SMF or I-SMF and the H-SMF or SMF in the case of PDU Session Establishment authentication/authorization by a DN-AAA Server defined in clause 4.3.2.3: it is used to carry DN Request Container information between the DN-AAA Server and the UE.

Input, Required: SM Context ID.

Input, Optional: UE location information (ULI), UE Time Zone, AN type, indication of PDU Session Release, H-SMF SM Context ID (from H-SMF to V-SMF) or SMF SM Context ID (from SMF to I-SMF), QoS Rule and QoS Flow level QoS parameters if any for the QoS Flow associated with the QoS rule (from H-SMF to V-SMF or from SMF to I-SMF), EPS bearer context(s) and Linked EBI (from H-SMF to V-SMF or from SMF to I-SMF), N9 Tunnel Info (from V-SMF to H-SMF or from I-SMF to SMF), Information requested by UE for e.g. QoS (from V-SMF to H-SMF or from I-SMF to SMF), 5GSM Core Network Capability, Information necessary for V-SMF or I-SMF to build SM Message towards the UE (from H-SMF to V-SMF or from SMF to I-SMF), Trigger PDU release indication (V-SMF to H-SMF or from I-SMF to SMF), Start Pause of Charging indication, Stop Pause of Charging indication, DN Request Container information, indication that the UE shall not be notified, EBI Allocation Parameters (ARP list), Secondary RAT usage data, indication that the access type of the PDU session can be changed (V-SMF to H-SMF or from I-SMF to SMF) or from SMF to I-SMF), extended NAS-SM timer indication, DNAI list supported by I-SMF (from I-SMF to SMF), indication of multi-homing support (from SMF to I-SMF), indication of ULCL or BP insertion (from I-SMF to SMF), indication of ULCL or BP removal (from I-SMF to SMF), IPv6 prefix @local PSA (from I-SMF to SMF), DNAI(s) supported by local PSA (from I-SMF to SMF), Tunnel info of ULCL or BP (from I-SMF to SMF), N4 information (from I-SMF to SMF or from SMF to I-SMF), Handover Complete Indication, Relocation Cancel Indication. MA PDU request indication, MA PDU Network-Upgrade Allowed indication, Indication on whether the UE is registered in both accesses, MA PDU session Accepted indication, access for MA PDU Session Release, access type for GBR QoS flow, Indication of access unavailability (with access type), QoS Monitoring Indication (from SMF to I-SMF), QoS Monitoring reporting frequency(from SMF to I-SMF), QoS monitoring policy (from SMF to I-SMF), QoS Monitoring Result from (I-SMF to SMF), Notification of the SM Policy Association Establishment and Termination, PCF binding information, Satellite backhaul category, N9 forwarding tunnel to support the EAS session continuity required (from SMF to I-SMF), traffic filter for N9 forwarding (from SMF to I-SMF), value of the timer to detect the end of activity on the N9 forwarding tunnel to support the EAS session continuity (from SMF to I-SMF), EAS rediscovery indication, EAS information to be refreshed for EAS re-discovery, IP address of V-EASDF, ECS Address Configuration Information, Alternative HPLMN S-NSSAI, HR-SBO authorization result, VPLMN Specific Offloading Information for HR-SBO, Offload Identifier(s), HPLMN address information, DNS Server address provided by HPLMN, Indication of UE supports non-3GPP access path switching.

Output, Required: Result indication, <ARP, Cause> pair.

Output, Optional: UE location information, AN Type, SM information from UE (from V-SMF to H-SMF or from I-SMF to SMF), list of Rejected QoS Flows (from V-SMF to H-SMF or from I-SMF to SMF), a list of <ARP, EBI> pair, Secondary RAT Usage Data, DNAI(s) of interest for this PDU Session (from SMF to I-SMF), N4 Information (from SMF to I-SMF), QFI(s), QoS Profile(s), Session-AMBR, QoS Rule(s), QoS Flow level QoS parameters if any for the QoS Flow(s) associated with the QoS rule(s), EPS bearer context(s), linked EBI, DNAI(s) of interest for this PDU Session, HR-SBO authorization result, VPLMN Specific Offloading Information for HR-SBO, Offload Identifier(s), HPLMN address information, DNS Server address provided by HPLMN, Internal Group Identifier(s).

The H-SMF SM Context ID in the Input provides addressing information allocated by the H-SMF (to be used for service operations towards the H-SMF for this PDU Session).

The SMF SM Context ID in the Input provides addressing information allocated by the SMF (to be used for service operations towards the SMF for this PDU Session).

See clause 4.3.3.3 for an example usage of this service operation.

See clauses 4.22.6.3, 4.22.7, 4.22.8.3 and 4.22.10.3 for detailed usage of this service operation for ATSSS.

See clause 6.7.3 of TS 23.548 [74] for detailed usage of this service for EAS re-discovery.

# * * * * Second change * * * *

5.2.8.2.10	Nsmf_PDUSession_ContextRequest service operation

Service operation name: Nsmf_PDUSession_ContextRequest.

Description: This service operation is used by the NF Consumer to request for SM Context (e.g. during EPS IWK, HO, SM Context transfer indication), or during mobility procedure with I-SMF (or V-SMF) changes or may be triggered by OAM.

Input, Required: SM Context ID, SM context type.

Input, Optional: Target MME Capability, EBI list not to be transferred, PDU Session ID (include PDU Session ID when available), SMF transfer indication, indication of no NG-RAN change.

Output, Required: One of the following:

-	SM Context Container.

-	Endpoint where SM Context can be retrieved.

Output, Optional: Small Data Rate Control Status.

The SM context type indicates the type of SM context to be requested, e.g. PDN Connection Context, 5G SM Context or both. The SM context type may also indicate that only a specific part of 5G SM Context is requested, e.g. only the AF Coordination Information part. If the SM context type is PDN Connection Context, the SM Context included in the SM Context container is the PDN Connection Context. If the SM context type is all, the SM Context included in the SM Context container includes both the PDN Connection Context and the 5G SM Context.

Table 5.2.8.2.10-1 illustrates the SM Context that may be transferred between I-SMF(s) or between V-SMF(s) in home-routed roaming case.

Table 5.2.8.2.10-1: SM Context of a PDU Session transferred between I-SMF(s) or between V-SMF(s) or between I/V-SMF and (H-)SMF

| Field | Description |
| --- | --- |
| SUPI | SUPI (Subscription Permanent Identifier) is the subscriber's permanent identity in 5GS. |
| Trace Requirements | Trace reference: Identifies a record or a collection of records for a particular trace. |
|  | Trace type: Indicates the type of trace |
|  | OMC identity: Identifies the OMC that shall receive the trace record(s). |
| S-NSSAI | The S-NSSAI of the PDU Session for the serving PLMN. |
| HPLMN S-NSSAI | The S-NSSAI of the PDU Session for the HPLMN (Home-Routed PDU Session) |
| Network Slice Instance id | The network Slice Instance information for the PDU Session |
| DNN | The associated DNN for the PDU Session. |
| AMF Information | The associated AMF instance identifier and GUAMI. |
| Access Type | The current access type for this PDU Session. |
| RAT Type | RAT Type for this PDU Session. |
| PDU Session ID | The identifier of the PDU Session. |
| H-SMF Information or SMF Information | The associated H-SMF identifier and H-SMF address for the HR PDU Session ( applies only for a V-SMF), or the SMF identifier and SMF address for PDU Session (applies for I-SMF). |
| Context ID of the PDU Session in H-SMF or Context ID of the PDU Session in SMF | The context ID of the PDU Session in H-SMF or in SMF. |
| Forwarding Indication | An indication on whether forwarding tunnel needs be established in order to forward buffered DL data. |
| Uplink Tunnel Info of UPF controlled by the SMF / H-SMF | The Tunnel Information to be used to send UL traffic towards the UPF controlled by the SMF / H-SMF that interfaces the UPF controlled by the I-SMF. |
| Tunnel Info of NG-RAN | The N3 Tunnel Information in the NG-RAN for the PDU Session. This information is transferred if the target I/V-SMF indicates no NG-RAN change. |
| Disaster Roaming | An indication that the UE is registered for Disaster Roaming service. |
| EAS information to be refreshed for EAS re-discovery | Identifies EAS(s) which needs to be refreshed corresponding to the old target DNAI if available. See details in clause 6.7.3 of TS 23.548 [74]. This applies only for a V-SMF (for V-SMF change case) or H-SMF (for V-SMF insertion case). |
| EAS IP replacement information | Identifies the information enables EAS IP replacement (i.e. source EAS IP address and port number, target EAS IP address and port number). See details in clause 6.7.3 of TS 23.548 [74]. |
| Target DNAI for EAS IP replacement | Target DNAI corresponding to the EAS IP replacement information. See details in clause 6.7.3 of TS 23.548 [74]. |
| Authorization Result for HR-SBO | Indicates whether HR-SBO is authorized. See details in clause 6.7.3 of TS 23.548 [74]. This applies only for a V-SMF (for V-SMF change within same PLMN case). |
| VPLMN Specific Offloading Information | Includes traffic description information authorized for HR-SBO in VPLMN and the corresponding policy for the traffic. See details in clause 6.7.3 of TS 23.548 [74] (for V-SMF change within same PLMN case). |
| Offload Identifier | Identifies a certain VPLMN Specific Offloading Information. A HPLMN ID is expected to be included in this Identifier, see details in clause 6.7.2 of TS 23.548 [74] (for V-SMF change within same PLMN case). |
| HPLMN address information | Identifies the address information in HPLMN (e.g. H-UPF IP address on N6). See details in clause 6.7.3 of TS 23.548 [74]. This applies for a V-SMF (for V-SMF change within same PLMN case), H-SMF (for V-SMF insertion). |
| DNS Server address provided by HPLMN | Identifies the DNS Server provided by HPLMN to VPLMN for HR-SBO. See details in clause 6.7.3 of TS 23.548 [74]. This applies for a V-SMF (for V-SMF change within same PLMN case), H-SMF (for V-SMF insertion). |
| AF Coordination Information: |  |
| Source DNAI | The DNAI from where the UE is moving. |
| UE IP address in Source DNAI | The UE IP address in the Source DNAI. |
| List of Notification Correlation IDs | Notification Correlation IDs for UP path change event as received in the PCC Rules |
| For each notification correlation ID: Uplink buffering indication | Uplink buffering indication as received from the AF for this notification correlation id during Early Notification. |
| For each QoS Flow in the PDU Session: |  |
| 5G QoS Identifier (5QI) | Identifier for the authorized QoS parameters for the service data flow. |
| ARP | The Allocation and Retention Priority for the service data flow consisting of the priority level, the pre-emption capability and the pre-emption vulnerability. |
| GFBR | Guaranteed Flow Bit Rate (GFBR) - UL and DL. |
| MFBR | Maximum Flow Bit Rate (MFBR) - UL and DL. |
| Priority Level | Indicates a priority in scheduling resources among QoS Flows. |
| Averaging Window | Represents the duration over which the guaranteed and maximum bitrate shall be calculated. |
| Maximum Data Burst Volume | Denotes the largest amount of data that is required to be transferred within a period of 5G-AN PDB. |
| Reflective QoS Control | Indicates to apply reflective QoS for the SDF in the TFT. |
| QoS Notification Control (QNC) | Indicates whether notifications are requested from 3GPP RAN when the GFBR can no longer (or can again) be guaranteed for a QoS Flow during the lifetime of the QoS Flow. |
| Maximum Packet Loss Rate | Maximum Packet Loss Rate - UL and DL. |
| Mapped EPS Bearer Context for Each QFI to support interworking with EPS: |  |
| EPS Bearer Id | An EPS bearer identity uniquely identifies an EPS bearer for one UE accessing via E-UTRAN. |
| TI | The GERAN/UTRAN Transaction ID (if any) that is associated with the EPS Bearer ID which is part of the Bearer Context received from the MME. |
| BSS Container | The GERAN BSS Container (if any) that is associated with the EPS Bearer ID which is part of the Bearer Context received from the MME. |
| Mapped EPS Bearer QoS | ARP, GBR, MBR, QCI. |
| PGW-U tunnel Information | PGW-U S5/S8 GTP-U tunnel IP address and TEID information. |
| TFT | Traffic Flow Template. |

# * * * * Fourth change * * * *

5.2.6.26.6	Nnef_EASDeployment_Subscribe service operation

Service operation name: Nnef_EASDeployment_Subscribe

Description: provided by the NEF for NF consumers to explicitly subscribe the notification of changes of EAS Deployment Information.

Inputs, Required: Event ID, Notification Target Address(+Notification Correlation ID).

Inputs, Optional: list of (DNN, S-NSSAI), Application Identifier, Internal Group Identifier.

Outputs, Required: Result Indication. When the subscription is accepted: Subscription Correlation ID (reference of the subscription).

Outputs, Optional: None.

# * * * * Fifth change * * * *

5.2.6.37.5	Nnef_ECSAddress_Subscribe service operation

Service operation name: Nnef_ECSAddress_Subscribe

Description: Provided by the NEF for NF consumers to explicitly subscribe the notification of changes of ECS Address Configuration Information.

Inputs, Required: Event ID, Notification Target Address (+Notification Correlation ID).

Inputs, Optional: list of (DNN, S-NSSAI), Internal Group Identifier.

Outputs, Required: Result Indication. When the subscription is accepted: Subscription Correlation ID (reference of the subscription).

Outputs, Optional: None.

# * * * * Sixth change * * * *

5.2.8.2.3	Nsmf_PDUSession_Update service operation

Service operation name: Nsmf_PDUSession_Update.

Description: Update the established PDU Session.

This service operation is invoked by the V-SMF towards the H-SMF in the case of UE or serving network requested PDU Session Modification in order for the V-SMF to transfer the PDU Session Modification request. It can also be invoked by the V-SMF to indicate to the H-SMF that the access type of the PDU session can be changed. This service operation is also invoked by the V-SMF to insert or remove UL CL or BP controlled by the V-SMF.

This service operation is invoked by the I-SMF towards the SMF in the case of UE or serving network requested PDU Session Modification in order for the I-SMF to transfer the PDU Session Modification request. It can also be invoked by the I-SMF to indicate to the SMF that the access type of the PDU session can be changed. This service operation is also invoked by the I-SMF towards the SMF to insert or remove ULCL or BP controlled by the I-SMF or to report usage offloaded via UL CL or BP controlled by I-SMF.

This service operation is invoked by the H-SMF towards the V-SMF for both UE initiated and HPLMN initiated PDU Session Modification and PDU Session Release cases in order to have the SM PDU Session Modification request or SM PDU Session Release request sent to the UE. It can also be invoked by the H-SMF towards the V-SMF to release the 5GC and 5G-AN resources in e.g. handover from 5GC-N3IWF to EPS and from 5GS to EPC/ePDG, wherein the UE is not notified. It can also be invoked by the H-SMF towards the V-SMF to provide EAS IP replacement information and target DNAI.

This service operation is invoked by the SMF towards the I-SMF for both UE initiated and SMF/PCF initiated PDU Session Modification and PDU Session Release cases in order to have the SM PDU Session Modification request or SM PDU Session Release request sent to the UE. It can also be invoked by the SMF towards the I-SMF to release the 5GC and 5G-AN resources in e.g. handover from 5GC-N3IWF to EPS and from 5GS to EPC/ePDG, wherein the UE is not notified. This service operation is also invoked by the SMF towards the I-SMF to provide updated N4 information or updated DNAI list of interest for this PDU Session when SMF receives updated PCC rules.

This service operation is invoked by the V-SMF or I-SMF and the H-SMF or SMF in the case of PDU Session Establishment authentication/authorization by a DN-AAA Server defined in clause 4.3.2.3: it is used to carry DN Request Container information between the DN-AAA Server and the UE.

Input, Required: SM Context ID.

Input, Optional: UE location information (ULI), UE Time Zone, AN type, indication of PDU Session Release, H-SMF SM Context ID (from H-SMF to V-SMF) or SMF SM Context ID (from SMF to I-SMF), QoS Rule and QoS Flow level QoS parameters if any for the QoS Flow associated with the QoS rule (from H-SMF to V-SMF or from SMF to I-SMF), EPS bearer context(s) and Linked EBI (from H-SMF to V-SMF or from SMF to I-SMF), N9 Tunnel Info (from V-SMF to H-SMF or from I-SMF to SMF), Information requested by UE for e.g. QoS (from V-SMF to H-SMF or from I-SMF to SMF), 5GSM Core Network Capability, Information necessary for V-SMF or I-SMF to build SM Message towards the UE (from H-SMF to V-SMF or from SMF to I-SMF), Trigger PDU release indication (V-SMF to H-SMF or from I-SMF to SMF), Start Pause of Charging indication, Stop Pause of Charging indication, DN Request Container information, indication that the UE shall not be notified, EBI Allocation Parameters (ARP list), Secondary RAT usage data, indication that the access type of the PDU session can be changed (V-SMF to H-SMF or from I-SMF to SMF) or from SMF to I-SMF), extended NAS-SM timer indication, DNAI list supported by I-SMF (from I-SMF to SMF), indication of multi-homing support (from SMF to I-SMF), indication of ULCL or BP insertion (from I-SMF to SMF), indication of ULCL or BP removal (from I-SMF to SMF), IPv6 prefix @local PSA (from I-SMF to SMF), DNAI(s) supported by local PSA (from I-SMF to SMF), Tunnel info of ULCL or BP (from I-SMF to SMF), N4 information (from I-SMF to SMF or from SMF to I-SMF), Handover Complete Indication, Relocation Cancel Indication. MA PDU request indication, MA PDU Network-Upgrade Allowed indication, Indication on whether the UE is registered in both accesses, MA PDU session Accepted indication, access for MA PDU Session Release, access type for GBR QoS flow, Indication of access unavailability (with access type), QoS Monitoring Indication (from SMF to I-SMF), QoS Monitoring reporting frequency(from SMF to I-SMF), QoS monitoring policy (from SMF to I-SMF), QoS Monitoring Result from (I-SMF to SMF), Notification of the SM Policy Association Establishment and Termination, PCF binding information, Satellite backhaul category, N9 forwarding tunnel to support the EAS session continuity required (from SMF to I-SMF), traffic filter for N9 forwarding (from SMF to I-SMF), value of the timer to detect the end of activity on the N9 forwarding tunnel to support the EAS session continuity (from SMF to I-SMF), EAS rediscovery indication, EAS information to be refreshed for EAS re-discovery, IP address of V-EASDF, ECS Address Configuration Information, Alternative HPLMN S-NSSAI, HR-SBO authorization result, VPLMN Specific Offloading Information for HR-SBO, HPLMN address information, DNS Server address provided by HPLMN, Indication of UE supports non-3GPP access path switching, EAS IP replacement information (from H-SMF to V-SMF), target DNAI (from H-SMF to V-SMF).

Output, Required: Result indication, <ARP, Cause> pair.

Output, Optional: UE location information, AN Type, SM information from UE (from V-SMF to H-SMF or from I-SMF to SMF), list of Rejected QoS Flows (from V-SMF to H-SMF or from I-SMF to SMF), a list of <ARP, EBI> pair, Secondary RAT Usage Data, DNAI(s) of interest for this PDU Session (from SMF to I-SMF), N4 Information (from SMF to I-SMF), QFI(s), QoS Profile(s), Session-AMBR, QoS Rule(s), QoS Flow level QoS parameters if any for the QoS Flow(s) associated with the QoS rule(s), EPS bearer context(s), linked EBI, DNAI(s) of interest for this PDU Session, HR-SBO authorization result, VPLMN Specific Offloading Information for HR-SBO, HPLMN address information, DNS Server address provided by HPLMN, Internal Group Identifier(s).

The H-SMF SM Context ID in the Input provides addressing information allocated by the H-SMF (to be used for service operations towards the H-SMF for this PDU Session).

The SMF SM Context ID in the Input provides addressing information allocated by the SMF (to be used for service operations towards the SMF for this PDU Session).

See clause 4.3.3.3 for an example usage of this service operation.

See clauses 4.22.6.3, 4.22.7, 4.22.8.3 and 4.22.10.3 for detailed usage of this service operation for ATSSS.

See clause 6.7.3 of TS 23.548 [74] for detailed usage of this service for EAS re-discovery.

# * * * * End of changes * * * *