SA WG2 Meeting #S2-160AHE	S2-2400841r01

22 - 29 January, 2024, Electronic	(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0352 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the Multicast MBS procedures for UEs using power saving functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At MBS Session Activation, when the AMF perform group paging, the AMF also includes the CM-IDLE UEs using power saving function(s). This shall be applied to UE (using power saving function) in reachable state and deep sleep state. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Moving the “AMF includes the CM-IDLE UEs using power saving function(s)” to general bulletin part. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | During activation, for the UE in the deep sleep, the AMF does not include the CM-IDLE UEs using power saving function(s). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 7.2.10	Multicast MBS procedures for UEs using power saving functions

Editor's note:	Alignment may be needed with RAN WG(s).

For a UE using power saving function to receive multicast MBS Session data, the following applies:

-	For an MBS multicast session, a UE needs to join the multicast session as defined in clause 7.2.1.3 prior to MBS data reception. To join the multicast session, the UE needs to be in RRC_CONNECTED state. The UE may select any suitable time when it is in RRC_CONNECTED state or transition to RRC_CONNECTED state before joining the multicast session.

NOTE 1:	If the UE has not previously joined the MBS multicast session, at the possible scheduled activation time it performs MBS join procedure. Whether the UE performs MBS join procedure in advance and stays "joined" or every time at activation time is left up to UE implementation.

-	At MBS Session Activation, when the AMF perform group paging, the AMF also includes the CM-IDLE UEs using power saving function(s).

-	If a UE is in RRC_CONNECTED state or in CM-IDLE state or CM-CONNECTED with RRC_INACTIVE state, and reachable (e.g. in active time in MICO or PTW for eDRX), due to other reasons at the start time and the scheduled activation times, the UE follows the related MBS procedures (e.g. clause 7.2.1 for UE join and clause 7.2.5.2 for MBS Session Activation).

-	If a UE is in CM-IDLE state or CM-CONNECTED with RRC_INACTIVE and in deep sleep (i.e. unreachable for paging to the network) at the possible start time and the possible scheduled activation times, the UE leaves the deep sleep state at the session start time and the possible scheduled activation times to allow MBS related procedures (e.g. clause 7.2.1 for UE join and clause 7.2.5.2 for MBS Session Activation):

-	At the possible start time, an RRC_IDLE or CM-CONNECTED with RRC_INACTIVE UE needs to send a request to transition to RRC_CONNECTED state and join the MBS multicast session (if not done before).

NOTE 2:	The UE become reachable in the network for the unicast service as well.

-	At the possible scheduled activation times, an RRC_IDLE UE that already joined the multicast MBS session needs to listen for paging requests and if paged by the network follow the existing procedures in clause 7.2.5.2. How long the device listens to paging is left up to device implementation.

NOTE 3:	How long the UE need to listen to paging is left up to UE implementation.

-	When the UE is in the middle of an MBS data transfer, and the UE is scheduled to move to deep sleep due to power saving, e.g. end of PTW for extended idle mode DRX, expiration of active time for MICO or the UE transitioning from CM-CONNECTED to CM-IDLE in the case of MICO with no active time, then the UE does not go to deep sleep during the remainder of the current MBS data transfer.

NOTE 4:	If at the end of the current MBS data transfer the UE knows there is another MBS data transfer scheduled soon, depending on the time between MBS data transfers, the UE can decide to go to sleep between MBS data transfers.

END OF CHANGES