3GPP TSG-SA WG2#160-Ad Hoc-e	S2-2400845r02

Electronic Meeting, 22-29 January 2024	(Revision of S2-240xxxx)

Source:	Sony, Qualcomm Incorporated, Nokia, Nokia shanghai Bell, IIT Bombay, Samsung, LG Electronics, DISH Network, Ericsson

Title:	Architecture assumptions and requirements for TR 23.700-06 on VMR_Ph2

Document for:	Approval

Agenda Item:	19.6

Work Item / Release:	FS_VMR_Ph2 / Rel-19

Abstract of the contribution: This contribution proposed the Architecture assumptions and requirements for TR 23.700-06 on VMR_Ph2.

# 1.	Discussion

This contribution proposed the Architecture assumptions and requirements for VMR_Ph2.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-06:

# >>>>BEGINNING OF CHANGES<<<<

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[w]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[x]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description".

[y]	3GPP TS 38.401: "NG-RAN Architecture description".

[z]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

# >>>>Start of next CHANGES<<<<

# 4	Architecture assumptions and requirements

## 4.1	Architecture assumptions

The study should be based on the following architecture assumptions:

-	the MWAB consists of a gNB component (MWAB-gNB) and a UE component (MWAB-UE);

-	the MWAB-gNB is based on the gNB functionality specified in TS 38.300 [x] and TS 38.401 [y];

NOTE 1:	Architecture impact on MWAB-gNB may depend on the RAN study output and needs to be coordinated with RAN WGs.

NOTE 2:	In this release CU/DU split of the MWAB-gNB is not supported.

-	the MWAB-gNB's N2/N3 and OAM access are over the IP connectivity provided by the PDU sessions(s) of the MWAB-UE;

-	the interface between MWAB-UE and MWAB-gNB is not in scope of SA2 if it needs to be standardized;

-	the MWAB-UE has a single NR Uu hop to the NG-RAN (i.e. MWAB-UE access a gNB via NR Uu interface which may use either TN or NTN technology);

-	the MWAB may serve UEs located inside or outside the vehicle mounted with the relay;

-	NR Uu is used for the radio link between a MWAB-gNB and served UEs. The NR Uu radio link between the MWAB-gNB and served UE does not use NTN technology;

-	LCS framework as defined in TS 23.273 [z] is used for providing the location service to the served UEs;

-	the MWAB may connect to an NG-RAN of a PLMN or an SNPN;

-	the MWAB-gNB may broadcast and connect to a PLMN that is different from the PLMN where the IP connectivity for backhauling is provided by the MWAB-UE;

-	the UE is served by the PLMN advertised by the MWAB-gNB it registers with. This may be different from the PLMN serving the MWAB-UE.

-	The MWAB-UE supports emergency services.

## 4.2	Architecture requirements

Solutions of the study should provide architecture and system level enhancements to the 5G system to support the operation of MWAB to satisfy the normative requirements of a mobile base station relay specified in TS 22.261 [w]. Specifically::

-	a MWAB shall be capable to serve legacy UE(s) to connect via MWAB

-	support end-to-end service continuity for the UEs served by a MWAB upon MWAB mobility should be specified

-	support the mobile network operator to configure, provision and control the operation of a MWAB;

NOTE 1:	Configuration of the MWAB needs to be coordinated with RAN WGs.

NOTE 2:	Charging support will be coordinated with SA5 if there is such need.

-	support of regulatory requirements (e.g. for support of emergency services, priority services) when UEs access 5GS via a MWAB;

-	support roaming of the MWAB-UE from its HPLMN into a VPLMN;

# >>>>END OF CHANGES<<<<