3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e 	S2-2400854

Online, Jan 22 - 29, 2024

Source:	Huawei, HiSilicon

Title:	KI#3: New Solution for UE-satellite-UE communication with UPF onboard

Document for:	Approval

Agenda Item:	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract: A solution for UE-satellite-UE communication of IMS voice and video based on local switching of the media plane in a UPF onboard the satellite

# 1. Introduction/Discussion

In 23700-29 v0.2.2, it is proposed to study the support of UE-satellite-UE communication as below:

-	Architectural enhancement to support UE-satellite-UE communication, including minimum necessary set of 5GC network functions and IMS components (if any) onboard the satellite(s), including:

- 	How to ensure routing of user plane traffic (for MMTEL and MC applications) remains in the satellite.

-  Potential impacts on IMS procedures and functions to manage UE-satellite-UE communication

- 	Conditions (such as eligibility, restrictions, authorization …) for realizing UE-satellite-UE communication for UEs.

In this paper, we present a possible architecture where a local UPF is deployed on the satellite to enable the traffic to be local switched at the local UPF onboard. The architecture assume all the IMS components are on the ground. To make the solution work, the following aspects need to be addressed:

- How both the originating and terminating IMS determine the voice/video call data that can be local switched without crossing the IMS media plane (in particular without crossing the IMS-AGW) and authorize 5GC to configure call local switch in 5GC

- How 5GC discovers two UEs are call parties of the same communication session when the call is being setup so that it can apply local switch of the media plane. Some interactions between IMS and 5GC is required to inform 5GC about which two UEs are call parties

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-29.

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[x]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

# * * * * Second change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 Support of Regenerative-based satellite access | 2 Support of Store and Forward Satellite operation | 3 Support of UE-satellite-UE communication |  |
| Solution X |  |  | X |  |

# * * * * Third change (All new text) * * * *

## 6.X	Solution X: UE-satellite-UE communication for IMS voice and video (UPF onboard)

### 6.X.1	Description

This solution resolves Key Issue #3: Support of UE-satellite-UE communication and focus on IMS voice and video service.

This solution based on the following assumptions:

-	the PSA UPF anchored by IMS PDU session is on the ground and the UPF acting as UL CL/BP/local PSA to be deployed onboard satellite

-	All the IMS components are on the ground

-	The call parties (UE-A and UE-B) are served by the same PCF and the same SMF

-	The call parties are under the same satellite (or the different satellites belonging to the same constellation if ISL is deployed and has suitable capabilities)

-	The call parties can either be roaming (with LBO) or non-roaming UEs

UE-A

UE-B

UPF

IMS

UPF

IMS data

IMS signaling

Figure 6.X.1-1: Basic architecture for IMS voice and video service UE-satellite-UE communication with UPF local switching (example of the call parties are under the same satellite)

### 6.X.2	Procedures

UE

RAN

5GS

IMS

3. UL CL/BP/local PSA configuration

5. Completion of Session establishment

1. Determines local switching in the UPF onboard is possible

2. UE policy request

4. Response

Figure 6.X.2.-1: Procedure of UE-satellite-UE communication for IMS voice and video (UPF onboard local switching)

1.	During a call session setup process, the IMS component P-CSCF determines that local switching in the UPF onboard is possible. To determine this, the P-CSCF takes into account whether the UE is accessing network by satellite, operator policies and other conditions such as whether media transcoding is required, whether home routing is requested for the session in roaming cases.

2. The P-CSCF includes the identifications of call parties in the policy request towards the 5GC.

3.	If the 5GC determines the peer UE is also served by the same PCF and the same SMF, the 5GC configures UPF onboard local switching. Otherwise, this step is omitted and the 5GC informs P-CSCF the UPF onboard local switching is impossible in step4.

Editor’s note: It is FFS how the network determines that the same PCF is selected for the two UE’s PDU Sessions.

Editor’s note: It is FFS how the network determines that the same SMF is selected for the two UE’s PDU Sessions.

4. The 5GC sends the result of UPF onboard local switching configuration to the IMS.

5.	Other steps to complete the call session setup process as defined in TS 23.228 [x] clause 5.6.

The media is local switched on the satellite via the local PSA UPF onboard.

Editor's note: how the serving PLMN performs lawful interception when the traffic is local switched on satellite is FFS

Editor's Note: whether the use of local switching needs to be included in the charging information is FFS.

Editor’s note: It is FFS how a change of satellite is handled.

### 6.X.3	Impacts on services, entities and interfaces

IMS:

-	The P-CSCF determines whether UPF local switching is possible when the UEs involved in the communication service with each other are accessing network via the satellite

-	The P-CSCF informs 5GC which two UEs are call parties for UE-satellite-UE communication with UPF local switching

-	The P-CSCF receive the result of UPF onboard local switching configuration from 5GC

5GC:

-	The 5GC receives identifications of call parties and indication that UPF onboard local switching is possible from IMS

-	The 5GC sends the result of UPF onboard local switching configuration to the P-CSCF

Editor’s note: The impacts to 5GC need to be broken down to different NFs

# * * End of changes * * * *