SA WG2 Meeting #160AHE	S2-2400919r01

January 22 – 29, 2024, Electronic Meeting	(revision of S2-240xxxx)

Source:	Charter Communications, CabelLabs, Tencent, Tencent Cloud, Nokia, Nokia Shanghai Bell

Title:	KI#6: New Solution: L4S support in non-3GPP accesses

Document for:	Approval

Agenda Item:	19.3

Work Item / Release: 	FS_XRM_Ph2 / Rel-19

Abstract: This paper proposes solutions for L4S support in non-3GPP accesses.

# 1. Introduction

As the use-cases and applications for XRM are not limited to 3GPP access, XRM devices and applications may use non-3GPP access as a means of communication. In such a scenario where the XRM device(s) use non-3GPP access path with 5GC backhaul, the intermediate nodes such as N3IWF, TNGF, W-AGF may experience UP traffic congestion (for example, an N3IWF/TNGF/W-AGF deployment supports multiple access connections and as a result may experience congestion).

The Rel-19 approved SID titled Study on Architecture enhancement for XRM Ph2 (SP-231198, updated in SP-231805) includes the below non-3GPP aspects to be studied as part of Rel-19:

WT#3 Further enhancement to support XR based on non-3GPP access.

WT#3.1 Study how to support L4S for non-3GPP access networks and intermediate 5GS nodes (N3IWF, TNGF and W-AGF) to perform ECN marking for L4S.

-	Support L4S in untrusted/trusted access (e.g. N3IWF, TNGF).

-  Support L4S in wireline access (e.g. W-AGF).

At SA2#160, S2-2312848 pCR was agreed, which defines the following Key Issue related to the above WT:

## 5.6	Key Issue #6: L4S for non-3GPP access networks and intermediate 5GS nodes

5.6.1	Description

As the use-cases and applications for XRM are not limited to 3GPP access, XRM devices and applications may use non-3GPP access as a means of communication.

The objective of this Key Issue is to extend the L4S mechanism to non-3GPP access networks and the potential impacts of such extension on the non-3GPP access-specific intermediate nodes.

The following aspects should be studied:

-	How to support L4S for non-3GPP access networks and intermediate 5GS nodes (N3IWF, TNGF and W-AGF) to perform ECN marking for L4S.

-	Support L4S in untrusted/trusted access (e.g., N3IWF, TNGF).

-	Support L4S in wireline access (e.g. W-AGF).

NOTE: 	It is limited to re-using existing control plane and user plane between 5GC and non-3GPP access networks. Assumptions on W-AGF functionality are to be verified with BBF and CableLabs.

This pCR proposes alternative solutions for KI#6 on L4S support in non-3GPP accesses considering wireline access and untrusted/trusted access.

# 2. Proposal

This contribution proposes to include Solutions #X, #X+1, #X+2and #X+3 in TR 23.700-70 clauses 6.X, 6.X+1, 6.X+2, 6.X+3 and update clause 2 References.

Start of 1st changes

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policies and Charging control framework for the 5G System; Stage 2".

[5]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)", March 2004.

[6]	IETF RFC 6904: "Encryption of Header Extensions in the Secure Real-time Transport Protocol (SRTP)".

[7]	IETF RFC 9335: "Completely Encrypting RTP Header Extensions and Contributing Sources".

[8]	IETF draft-ietf-avtcore-rtp-over-quic: "RTP over QUIC (RoQ)".

[9]	IETF draft-ietf-moq-transport: "Media over QUIC Transport".

[10]	IETF experimental draft-ietf-avtext-framemarking: "Frame Marking RTP Header Extension".

[11]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport".

[Z]	IETF RFC 9330: "Low Latency, Low Loss, and Scalable Throughput (L4S) Internet Service: Architecture

[Y]	IETF RFC 9331: "The Explicit Congestion Notification (ECN) Protocol for Low Latency, Low Loss, and Scalable Throughput (L4S)".

[X]	IETF draft-ietf-tsvwg-ecn-encap-guidelines-22: "Guidelines for Adding Congestion Notification to Protocols that Encapsulate IP".

[W]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[V]	CableLabs DOCSIS MULPI: "Data-Over-Cable Service Interface Specifications DOCSIS 3.1, MAC and Upper Layer Protocols Interface Specification".

[U]	IEEE 802.11-23/2065r0: “L4S and Implications for Wi-Fi”.

[T] 	IETF RFC 9332: " Dual-Queue Coupled Active Queue Management (AQM) for Low Latency, Low Loss, and Scalable Throughput (L4S)".

Editor's note:  References [8], [9], [10] and [X] cannot be formally referenced until published as RFC.

Start of 2nd changes

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  | Key Issues |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 |  | 2 | 3 | 4 | 5 | 6 | 7 | X | Y |
| #X: Support for L4S in wireline access |  |  |  |  |  |  |  | X |  |  |  |
| #X+1: L4S support in non-3GPP untrusted/trusted accesses |  |  |  |  |  |  |  | X |  |  |  |
| #X+2: Support L4S mechanism for non-3GPP access networks and intermediate 5GS nodes |  |  |  |  |  |  |  | X |  |  |  |
| #X+3: Supporting L4S in intermediate non-3GPP access nodes |  |  |  |  |  |  |  | X |  |  |  |

Start of 3rd changes (all new text)

## 6.X	Solution #X: L4S in wireline access

6.X.1	Key Issue mapping

This solution proposes a solution for the following issue in KI #6:

- Support L4S in wireline access (e.g. W-AGF).

6.X.2	Description

6.X.2.1	General

In 5G System, ECN marking for L4S may be supported. ECN marking for L4S is enabled on a per QoS Flow basis in the uplink and/or downlink direction and may be used for GBR and non-GBR QoS Flows. ECN marking for L4S in the IP header is supported in either the NG-RAN (see TS 23.501 [2] clause 5.37.3.2), or in the PSA UPF (see TS 23.501 [2] clause 5.37.3.3) via NG-RAN. In extending ECN marking for L4S via wireline access, the following shows components in the route of an IP packet via wireline access, where ECN marking could be supported:

- Wireline access: Wireline access node(s) perform ECN marking for downlink/uplink

- 5G-RG: 5G-RG performs ECN marking for downlink/uplink

For an IP packet via wireline access, ECN marking could occur in one of the above components. Based on L4S behaviour as described in IETF RFC 9331 [Y], congestion indication/marking by a component is performed only when congestion indication has not been set by previous component in a route. Depending on capabilities of each component, ECN marking on a route may be supported by a subset of the above, none of the above, or all the above.

For wireline cable access network e.g., DOCSIS, provides support for low-latency services through the dual-queue approach, by separating queue-building and non-queue-building traffic [V]. DOCSIS, such as Low Latency DOCSIS (LLD), also supports ECN marking for L4S, where L4S traffic would be managed separately from other class traffic (e.g., separate low latency service flow from other classic service flow(s)).

Besides, the RG can be a bottleneck link experiencing local congestion. Last-mile congestion on WLAN is not negligible and support of L4S in last-mile route would provide considerable performance benefits [U]. Thus, the solution also addresses this by allowing the RG to enable ECN marking for L4S.

The proposed solution has the following principles:

- ECN marking for L4S in W-5GAN is enabled on a per QoS Flow basis in the UL and/or DL direction.

- Dedicated W-UP resources are used for carrying L4S-enabled IP traffic.

- For DL, W-AGF maps the L4S-enabled QoS Flows to W-UP resources.

- For UL, 5G-RG maps the L4S-enabled QoS Flows to W-UP resources.

- 5G-RG may enable ECN marking for L4S in the IP header of the user IP packets based on its local congestion information.

Fig 6.X.2.1-1 L4S support in wireline access

6.X.2.2	Support of ECN marking for L4S in wireline node(s)

In case of DL direction, when the W-AGF receives N2 requests related with PDU Session resources, the W-AGF maps the QoS profile(s) received from the 5GC to W-UP level QoS. The SMF may be instructed, based on either dynamic or predefined PCC rule, to provide an indication for L4S for a corresponding QoS Flow(s) to W-AGF. W-AGF maps the L4S-enabled QoS Flow(s) to L4S-enabled wireline QoS resource(s).

When the W-AGF receives a DL PDU via N3, it identifies the QFI from the GTP-U header, and if the QFI corresponds to a QoS Flow with L4S enabled, it determines the corresponding L4S-enabled wireline QoS resource to use for sending the DL PDU to the RG.

Details on ECN marking for L4S by wireline nodes (such as a CMTS) are outside of 3GPP scope.

In case of UL, when the 5G-RG receives NAS message related with PDU Session QoS, the 5G-RG maps the QoS rule(s) received in NAS to W-UP level QoS. The SMF may be instructed, based on either dynamic or predefined PCC rule, to provide an indication for ECN marking for L4S for a corresponding QoS Flow(s) to the 5G-RG. 5G-RG maps the L4S-enabled QoS Flow(s) to L4S-enabled wireline QoS resources(s).

When the 5G-RG transmits an UL PDU, if it determines (by using the QoS rules of the PDU Session) that the QFI corresponds to a QoS Flow with L4S enabled, it determines the corresponding L4S-enabled wireline QoS resource to use for sending the UL PDU to the W-AGF.

6.X.2.3	Support of ECN marking for L4S in 5G-RG

5G-RG may be requested by SMF to perform ECN marking for L4S in the IP header of the user IP packets, based on its local congestion conditions, for those QoS flows for which it received an indication for L4S in the QoS rule(s) via the NAS message. The criteria based on which 5G-RG decides to mark ECN bits for L4S is 5G-RG implementation specific.

6.X.3	Procedures

Fig. 6.X.3-1 High-level procedure of the solution

- Steps 1-2a specified in TS 23.316 [d] clause 7.3.1.1, with the difference that the 5G-RG 5GSM Core Network Capability included in the PDU Session Establishment Request within the N1 SM container indicates whether the 5G-RG supports ECN marking for L4S.

- The N2 SM information carries information that the AMF shall forward to the W-AGF which includes: For each QoS Flow, an ECN marking for L4S indicator to W-5GAN in the case of ECN marking for L4S in W-5GAN.

The N1 SM container that the AMF shall provide to the 5G-RG contains the: For each QoS Flow, an ECN marking for L4S indicator to 5G-RG in the case of ECN marking for L4S in the 5G-RG, if the 5G-RG indicated support for ECN marking for L4S.

- The AMF shall under request of the SMF send a N2 PDU Session Resource Setup Request message, which includes the ECN marking for L4S indicator, to W-AGF to establish the access resources for this PDU Session.

- Based on the QoS flows and QoS parameters received in the previous step, W-AGF determines the corresponding L4S-enabled wireline QoS resource needed for the PDU session.

- The W-AGF sets up the W-UP resources for the PDU session.

- After all W-UP resources are established, the W-AGF shall forward to 5G-RG via the W-CP signalling connection the PDU Session Establishment Accept message (including the ECN marking for L4S indicator) received in step 2b.

- The W-AGF shall send to AMF an N2 PDU Session Resource Setup Response including: established QoS Flows status (active/not active) for ECN marking for L4S in wireline access.

- All steps specified in TS 23.502 [3] clause 4.3.2.2.1 after step 14 are executed according to the PDU Session Establishment procedure over 3GPP access.

6.X.4	Impacts on services, entities and interfaces

SMF:

- Indication of ECN marking for L4S to 5G-RG over N1

- Configuration of QoS profiles to W-AGF including L4S handling

W-AGF:

- Mapping of L4S-enabled QoS profile to L4S-enabled W-UP resource

5G-RG:

- Mapping of L4S-enabled QoS rule to L4S-enabled W-UP resource

- ECN marking for L4S in the IP header of the user IP packets

- Indication of support for ECN marking for L4S during PDU Session Establishment Request

## 6.X+1	Solution #X+1: L4S in non-3GPP untrusted/trusted accesses

6.X+1.1	Key Issue mapping

Proposes solutions for KI#6:

- The mechanisms needed to support L4S on untrusted/trusted non-3GPP accesses in UL and DL directions, including how to support ECN marking for L4S in various network elements such as UE, UPF, N3IWF, TNGF, WLAN AP and TNAP.

6.X+1.2	Description

6.X+1.2.1	General

Untrusted and trusted non-3GPP accesses are able to connect 3GPP UEs to the 5GC via a N3IWF (in the case of un-trusted access) or a TNGF (in the case of trusted access) which interfaces directly to the 5GC’s CP and UP functions via N2 and N3 reference points, respectively. The N3IWF and TNGF support similar functionality and reference points as an NG-RAN towards the 5GC, specifically communicating to 5GC CP and UP functions over N2 and N3 interfaces.

Figure 6.X+1.2.1-1 L4S support in non-3GPP (i.e., w/ WiFi AP+N3IWF or w/ TNAP+TNGF) accesses

Figure 6.X+1.2.1-1 above is a high-level illustration of 3GPP devices (i.e., UEs) connecting to a 3GPP 5GS via non-3GPP accesses and where support for L4S is investigated. 3GPP R18 specifies dedicated user plan resources for carrying L4S-enabled IP traffic, which is realized in 3GPP access via QoS flows.

Figure 6.X+1.2.1-2 below illustrates how 3GPP QoS flow is carried over non-3GPP accesses.

Figure 6.X+1.2.1-2 non-3GPP (i.e., w/ WLAN+N3IWF or TNAP+TNGF) QoS Architecture (i.e., Rules, Profiles, SDF Template, & Flows, etc.)

If congestion in a non-3GPP access (i.e., un-trusted and trusted) is experienced it will most probably occur at the wireless access point (i.e., AP), which is not in scope of 3GPP specifications, although the N3IWF and TNGF could also experience congestion independently within its internal managed traffic queues as well as the UE.

How to provide congestion information (i.e., ECN marking and level of congestion) on behalf of the AP is an area to investigate, as well as:

- If it is possible, for the AP to provide percentage of congestion information similar to how NG-RAN provides.

- If the wireless access point does not support L4S, it should not impact the IP header’s ECN field as specified in RFC 9330 [Z], is there any impact in N3IWF and TNGF for support of L4S.

- If the N3IWF and TNGF can determine that the AP was congested based on internal means.

The solutions provided propose that 3GPP-defined non-3GPP access nodes (i.e., N3IWF and TNGF) provide mechanisms to extend support for L4S in non-3GPP untrusted and trusted accesses.

The proposed solutions have the following principles:

- Dedicated UP resources are used for carrying L4S-enabled IP traffic.

- N3IWF and/or TNGF maps the L4S-enabled QoS Flows to UP resources.

- N3IWF and/or TNGF relays ECN marking up the stack to the Inner most IP header, so the end-to-end applications are aware of the ECN marking.

- Option for the UE to perform ECN marking when UL congestion is experienced at the UE.

6.X+1.2.2	Supporting L4S in N3IWF

It is most probable that if congestion is experienced in the uplink, it is related to the wireless link (i.e., WiFi AP).  3GPP does not have responsibility for the AP specification but can relay any congestion notification that the WiFi AP provides to the inner most IP layer at the N3IWF. This is accomplished by leveraging IETF draft RFC, draft-ietf-tsvwg-ecn-encap-guidelines-22 [X], which provides Guidelines for Adding Congestion Notification to Protocols that Encapsulate IP.

Figure 6.X+1.2.2-1 Congestion in uplink detected at WLAN AP of non-3GPP untrusted access.

Figure 6.X+1.2.2-1 illustrates how support for L4S is accomplished when congestion is detected at the WLAN AP in the uplink.

- WLAN AP indicates via IP Header ECN field that congestion was experienced, assuming that the ECN field was set to ECT(1).

- N3IWF detects outer IP header ECN field is set indicating congestion.

- N3IWF relays information to inner-most IP header as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [X].

- N3IWF relays information to outer IP header to UPF as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [X] and if the PSA UPF is responsible for performing the ECN marking in the UL, the N3IWF can provide congestion information related to congestion experienced by the N3IWF but may not be able to provide congestion information experienced by the WiFi AP. Based on implementation it may be possible for the N3IWF to estimate congestion information relative to the access point (i.e., level of congestion of WLAN AP) and provide via GTP-U header.

- N3IWF provides GTP-U header with congestion information to UPF and UPF marks ECN field indicating congestion was experienced.

- N3IWF relays information to UPF inner IP header as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [X] and UPF marks ECN field as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [X] indicating congestion was experienced.

If congestion is experienced in the uplink by the N3IWF, not illustrated, it is capable of setting the IP header ECN field of the inner IP as well as providing congestion information via GTP-U to PSA UPF in the case the PSA UPF is instructed to perform the ECN marking.

If congestion is experienced in the uplink by the N3IWF and the WiFi AP, the ECN marking provided by the WiFi AP (if supported) would take precedent and relayed to the inner most IP layer for use by the receiving application. If the congestion in the uplink at the N3IWF persists, it will eventually perform the ECN marking once the congestion from the WiFi AP is removed.

Like uplink, it is most probably if congestion is experienced in the downlink it is related to the wireless link (i.e., WiFi AP) and 3GPP does not have responsibility for the AP specification but can relay any congestion notification that the WiFi AP provides to the inner most IP layer at the UE. This is accomplished by leveraging IETF draft RFC, draft-ietf-tsvwg-ecn-encap-guidelines-22 [X], which provides Guidelines for Adding Congestion Notification to Protocols that Encapsulate IP.

Figure 6.X+1.2.2-2 Congestion in downlink detected at WLAN AP of non-3GPP untrusted access.

Figure 6.X+1.2.2-2 illustrates how support for L4S is accomplished when congestion is detected at the WLAN AP in the downlink.

- WLAN AP indicates via IP Header ECN field that congestion was experienced, assuming that the ECN field was set to ECT(1).

- UE detects outer IP header ECN field is set indicating congestion. UE relays information to inner IP header as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [X].

- N3IWF detects outer IP header ECN field is set indicating congestion.

- N3IWF relays information to inner-most IP header as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [X].

- N3IWF relays information to outer IP header to UPF as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [X] and if the PSA UPF is responsible for performing the ECN marking in the UL, the N3IWF can provide congestion information related to congestion experienced by the N3IWF but it may not be able to provide congestion information experienced at the WiFi AP. Based on implementation it may be possible for the N3IWF to estimate congestion information relative to the access point (i.e., level of congestion of WLAN AP) and provide via GTP-U header.

- N3IWF provides GTP-U header with congestion information to UPF and UPF marks ECN field in the inner-most header indicating congestion was experienced.

- N3IWF relays information to UPF outer IP header as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [X] and UPF marks ECN field of the inner-most IP header as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [X] indicating congestion was experienced.

In summary, Figures 6.X+1.2.2-1 and 6.X+1.2.2-2 illustrates the following options for ECN marking when congestion is detected in the WLAN AP in an untrusted deployment:

- UL

  - option 1 (Fig 6.x+1.2.2-1) N3IWF performs marking: steps 1, 2, 3.

  - option 2 (Fig 6.x+1.2.2-1) UPF performs marking: steps 1, 2, 4, 5 and/or 1, 2, 4, 6.

    - Note that if UPF is doing marking, it also provides config info.

- DL

  - option 1 (Fig 6.x+1.2.2-2) UE performs marking: steps 1, 2.

  - option 2 (Fig 6.x+1.2.2-2) N3IWF performs marking: steps 1, 3, 4.

  - option 3 (Fig 6.x+1.2.2-2) UPF performs marking: steps 1, 3, 5, 6 and/or 1, 3, 5, 7.

    - Note that if UPF is doing marking, it also provides config info.

In the downlink the N3IWF supports the same reference point N3, as the NG-RAN and should be able to support similar L4S functionality as the NG-RAN. Such as:

- Method 1: Performing ECN marking according to IETF RFC 9330 [Z]and RFC 9331 [Y] for downlink in IP layer of the received packets. Also, dedicated QoS flow(s) can be used for carrying L4S enabled IP traffic.

- Method 2: If the PSA UPF performs the ECN marking in the downlink, the N3IWF can provide the congestion information via the GTP-U header. The N3IWF shall also be able to receive an indication from the SMF to report congestion information (i.e., a percentage of packets that UPF uses for ECN marking for L4S) of the QoS flow on DL direction via GTP-U header extension to PSA UPF. If there is no UL packet when report for DL needs to be provided the N3IWF may generate an UL Dummy GTP-U packet for such reporting. That is, if congestion is experienced in the downlink by the N3IWF, it can set the IP header ECN field of the inner IP as well as providing congestion information via GTP-U to PSA UPF in the case the PSA UPF is instructed to perform the ECN marking.

Figure 6.X+1.2.2-3 Congestion in downlink detected at N3IWF of non-3GPP untrusted access.

Figure 6.X+1.2.2-3 illustrates how support for L4S is accomplished when congestion is detected at the N3IWF in the downlink.

- N3IWF indicates via IP Header ECN field that congestion was experienced, assuming that the ECN field was set to ECT(1).

- If the PSA UPF is responsible for performing the ECN marking in the DL, the N3IWF can provide congestion information related to congestion experienced by the N3IWF via GTP-U header.

- UPF receives congestion information from the GTP-U header and set the ECN field accordingly.

In summary, Figure 6.X+1.2.2-3 illustrates the following options for ECN marking when congestion is detected in the N3IWF in an untrusted deployment:

- DL

  - option 1 (Fig 6.x+1.2.2-3) N3IWF performs marking: steps 1.

  - option 2 (Fig 6.x+1.2.2-3) UPF performs marking: steps 1, 2, 3.

    - Note that if UPF is doing marking, it also provides config info.

6.X.2.3	Supporting L4S in TNAN

Trusted (i.e. TNAN) access support for L4S is similar to un-trusted, although 3GPP has responsibility for specifying NWt with can leverage the draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [X] as described in clause 6.X+1.2.2 Supporting L4S in N3IWF.

Figure 6.X+1.2.3-1 Congestion in uplink detected at TNAP of non-3GPP trusted access.

Figure 6.X+1.2.3-1 illustrates how support for L4S is accomplished when congestion is detected in the uplink at the TNAP.

- TNAP indicates via IP Header ECN field that congestion was experienced, assuming that the ECN field was set to ECT(1).

- TNGF detects outer IP header ECN field is set indicating congestion.

- TNGF relays information to inner IP header as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [X].

- TNGF relays information to outer IP header to UPF as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [X] and if the PSA UPF is responsible for performing the ECN marking in the UL, the TNGF can provide congestion information related to congestion experienced by the TNGF but may not be able to provide congestion information experienced by the TNAP. Based on implementation it may be possible for the TNGF to estimate congestion information relative to the access point (i.e., level of congestion of TNAP) and provide via GTP-U header.

- TNGF provides GTP-U header with congestion information to UPF and UPF marks ECN field indicating congestion was experienced.

- TNGF relays information to UPF inner IP header as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [X] and UPF marks ECN field as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [X] indicating congestion was experienced.

If congestion is experienced in the uplink by the TNGF, not illustrated, it is capable of setting the IP header ECN field of the inner IP as well as providing congestion information via GTP-U to PSA UPF in the case the PSA UPF is instructed to perform the ECN marking.

If congestion is experienced in the uplink by the TNGF and the WiFi AP, the ECN marking provided by the WiFi AP (if supported) would take precedent and relayed to the inner most IP layer for use by the receiving application. If the congestion in the uplink at the TNGF persists, it will eventually perform the ECN marking once the congestion from the TNAP is removed.

Like uplink, it is most probably if congestion is experienced in the downlink it is related to the wireless link (i.e., TNAP) and 3GPP does not have responsibility for the AP specification but can relay any congestion notification that the TNAP provides to the inner most IP layer at the UE. This is accomplished by leveraging IETF draft RFC, draft-ietf-tsvwg-ecn-encap-guidelines-22 [X], which provides Guidelines for Adding Congestion Notification to Protocols that Encapsulate IP.

Figure 6.X+1.2.3-2 Congestion in downlink detected at TNAP of non-3GPP trusted access.

Figure 6.X+1.2.3-2 illustrates how support for L4S is accomplished when congestion is detected at the TNAP in the downlink.

- TNAP indicates via IP Header ECN field that congestion was experienced, assuming that the ECN field was set to ECT(1).

- UE detects outer IP header ECN field is set indicating congestion. UE relays information to inner IP header as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [X].

- TNGF detects outer IP header ECN field is set indicating congestion.

- TNGF relays information to inner IP header as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [X].

- TNGF relays information to outer IP header towards UPF as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [X] and if the PSA UPF is responsible for performing the ECN marking in the DL, the TNGF can provide congestion information related to congestion experienced by the TNGF but may not be able to provide congestion information experienced by the TNAP. Based on implementation it may be possible for the TNGF to estimate congestion information relative to the access point (i.e., level of congestion of TNAP) and provide via GTP-U header.

- TNGF provides GTP-U header with congestion information to UPF and UPF marks ECN field indicating congestion was experienced.

- TNGF relays information to UPF outer IP header as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [X] and UPF marks ECN field of inner-most IP header as specified in draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [X] indicating congestion was experienced.

In summary, Figures 6.X+1.2.3-1 and 6.X+1.2.3-2 illustrates the following options for ECN marking when congestion is detected in the TNAP in a trusted deployment:

- UL

  - option 1 (Fig 6.x+1.2.3-1) TNGF performs marking: steps 1, 2, 3.

  - option 2 (Fig 6.x+1.2.3-1) UPF performs marking: steps 1, 2, 4, 5 and/or 1, 2, 4, 6.

    - Note that if UPF is doing marking, it also provides config info.

- DL

  - option 1 (Fig 6.x+1.2.3-2) UE performs marking: steps 1, 2.

  - option 2 (Fig 6.x+1.2.3-2) TNGF performs marking: steps 1, 3, 4.

  - option 3 (Fig 6.x+1.2.3-2) UPF performs marking: steps 1, 3, 5, 6 and/or 1, 3, 5, 7.

    - Note that if UPF is doing marking, it also provides config info.

In the downlink the TNGF supports the same reference point N3, as the NG-RAN and should be able to support similar L4S functionality as the NG-RAN. Such as:

- Method 1: Performing ECN marking according to IETF RFC 9330 [Z]and RFC 9331 [Y] for downlink in IP layer of the received packets. Also, dedicated QoS flow(s) can be used for carrying L4S enabled IP traffic.

- Method 2: If the PSA UPF performs the ECN marking in the downlink, the N3IWF can provide the congestion information via the GTP-U header. The TNGF shall also be able to receive an indication from the SMF to report congestion information (i.e., a percentage of packets that UPF uses for ECN marking for L4S) of the QoS flow on DL direction via GTP-U header extension to PSA UPF. If there is no UL packet when report for DL needs to be provided the TNGF may generate an UL Dummy GTP-U packet for such reporting. That is, if congestion is experienced in the downlink by the TNGF, it can set the IP header ECN field of the inner IP as well as providing congestion information via GTP-U to PSA UPF in the case the PSA UPF is instructed to perform the ECN marking.

Figure 6.X+1.2.3-3 Congestion in downlink detected at TNGF of non-3GPP trusted access.

Figure 6.X+1.2.3-3 illustrates how support for L4S is accomplished when congestion is detected at the TNGF in the downlink.

- TNGF indicates via IP Header ECN field that congestion was experienced, assuming that the ECN field was set to ECT(1).

- If the PSA UPF is responsible for performing the ECN marking in the DL, the TNGF can provide congestion information related to congestion experienced by the TNWF via GTP-U header.

- UPF receives congestion information from the GTP-U header and set the ECN field accordingly.

In summary, Figure 6.X+1.2.3-3 illustrates the following option for ECN marking when congestion is detected in the TNGF in a trusted deployment:

- DL

  - option 1 (Fig 6.x+1.2.3-3) TNGF performs marking: steps 1.

  - option 2 (Fig 6.x+1.2.3-3) UPF performs marking: steps 1, 2, 3.

    - Note that if UPF is doing marking, it also provides config info.

6.X+1.2.4	Supporting L4S in UE

A 3GPP UE may also provide uplink support for L4S when connecting to a 5GS via a non-3GPP access.

Figure 6.X+1.2.4-1 Congestion in uplink detected at UE via non-3GPP un-trusted access.

Figure 6.X+1.2.4-1 illustrates how support for L4S is accomplished when congestion is detected in the uplink at the UE when accessing a 5GS via a non-3GPP un-trusted access (i.e., WLAN AP + N3IWF).

- UE indicates via inner-most IP Header ECN field that congestion was experienced, assuming that the ECN field was set to ECT(1).

- If the PSA UPF is responsible for performing the ECN marking in the UL, the UE can provide congestion information related to congestion experienced by the UE via GRE to the N3IWF.

- N3IWF detects GRE message and provides the UE’s congestion information on the GTP-U header to UPF.

- N3IWF sends GTP-U message containing congestion information to UPF.

- UPF receives congestion information from the GTP-U header and set the ECN field in the inner-most IP header accordingly.

In summary, Figure 6.X+1.2.4-1 illustrates the following option for ECN marking when congestion is detected in the UE in an untrusted deployment:

- UL

  - option 1 (Fig 6.x+1.2.4-1) UE performs marking: step 1.

  - option 2 (Fig 6.x+1.2.4-1) UPF performs marking: steps 2, 3, 4, 5.

    - Note that if UPF is doing marking, it also provides config info.

6.X+1.3	Procedures

TBD

6.X+1.4	Impacts on services, entities and interfaces

- PCF enables the non-3GPP access (i.e. N3IWF and TNGF) to relay ECN marking from the WLAN AP IP header.

- UE, N3IWF, TNGF, & UPF support for draft RFC draft-ietf-tsvwg-ecn-encap-guidelines-22 [X]

## 6.X+2	Solution #X+2: Support L4S mechanism for non-3GPP access networks and intermediate 5GS nodes

### 6.X+2.1	Key Issue mapping

This solution applies to Key Issue #6 i.e. L4S for non-3GPP access networks and intermediate 5GS nodes.

### 6.X+2.2	Description

This solution takes the architecture models for non-3GPP access networks as basis which is specified in Clause 4.2.8 of TS 23.501 and includes the following aspects.

- Non-roaming architecture for 5G Core Network with untrusted non-3GPP access (Figure 4.2.8.2.1-1 of TS 23.501)

- Non-roaming architecture for 5G Core Network with trusted non-3GPP access (Figure 4.2.8.2.1-2 of TS 23.501)

- Non- roaming architecture for 5G Core Network for 5G-RG with Wireline 5G Access network and NG RAN (Figure 4.2.8.4-1 of TS 23.501)

### 6.X+2.3 Procedures and Impacts to support L4S

### 6.X+2.3.1 Support of L4S in Non-roaming architecture for 5G Core Network with untrusted non-3GPP access

Referring to Figure 4.2.8.2.1-1 of TS 23.501, impacts to NFs are listed as follows:

N3IWF:

-    N3IWF supports L4S handling like NG-RAN/gNB as N2/N3 termination point including ECN marking for L4S

SMF:

- Configure QoS profiles to N3IWF as to NG-RAN for L4S handling

UPF, AF:

-  Same functions as L4S mechanisms from NG-RAN case

### 6.X+2.3.2 Support of L4S in Non-roaming architecture for 5G Core Network with trusted non-3GPP access

Referring to Figure 4.2.8.2.1-2 of TS 23.501, impacts to NFs are listed as follows:

TNGF:

-    TNGF supports L4S handling like NG-RAN/gNB as N2/N3 termination point and support ECN marking for L4S

SMF:

- Configure QoS profiles to TNGF as to NG-RAN for L4S handling

UPF, UE, AF:

-  Same functions as L4S mechanisms from NG-RAN case

### 6.X+2.3.2 Support of L4S in Non-roaming architecture for 5G Core Network for 5G-RG with Wireline 5G Access network and NG RAN

Referring to Figure 4.2.8.4.-1 of TS 23.501, the impacts are listed as follows:

5G-RG:

- 5G-RG supports ECN marking for L4S functions in 5G UE as specified in TS 23.501

W-AGF:

- W-AGF supports the ECN marking for L4S as NG-RAN node as specified in TS 23.501

SMF:

- Configuration of QoS profiles to W-AGF including L4S handling

## 6.X+3	Solution #X+3: Supporting L4S in intermediate non-3GPP access nodes

### 6.X+3.1	Key Issue Mapping

This solution addresses Key Issue #6.

### 6.X+3.2	Description

L4S (Low Latency, Low Loss and Scalable Throughput) is described in IETF RFC 9330 [Z], IETF RFC 9331 [Y] and IETF RFC 9332 [T]. It exposes congestion information by marking ECN bits in the IP header of the user IP packets between the UE and the application server to trigger application layer rate adaptation.

In 5G System, ECN marking for L4S may be supported in intermediate non-3GPP access nodes such as N3IWF (for untrusted non-3GPP access), TNGF (for trusted non-3GPP access) or W-AGF (for Wireline non-3GPP access). The following topics should be handled:

- On which granularity and for what traffic type ECN marking for L4S is supported at intermediate non-3GPP access nodes?

- What are the triggers for enabling ECN marking for L4S?

The current solution proposes that ECN marking for L4S is enabled on a per QoS Flow basis. An AF might request enabling ECN marking for L4S via the Nnef_AFsessionWithQoS service operation. Alternatively, ECN marking for L4S may be enabled by the SMF based on dynamic or static policies for the DNN/S-NSSAI of the PDU session. The SMF provides an indication for ECN marking for L4S to the intermediate non-3GPP access node for a corresponding QoS Flow. ECN marking for L4S might apply to GBR flows or non-GBR flows. It might also apply to uplink and/or downlink direction. QoS rules in the UE and PDRs in the PSA UPF determine the PDUs bound to the L4S enabled QoS Flow.

The criteria based on which an intermediate non-3GPP access node decides to mark ECN bits when congestion occurs are implementation specific.

### 6.X+3.3	Procedure

The network requested PDU Session Modification procedure of 3GPP TS 23.502 clause 4.3.3.2 applies, where RAN is replaced by an intermediate non-3GPP access node (N3IWF, TNGF, W-AGF):

- SMF triggered by static or dynamic PCC rules decides to enable ECN marking for L4S for a dedicated QoS Flow. An AF request via Nnef_AFsessionWithQoS might be the trigger for a dynamic PCC rule.

- The intermediate non-3GPP access node is notified accordingly (via the AMF). If requested a new QoS flow is setup.

- The intermediate non-3GPP access node replies to the SMF via the AMF.

- The SMF updates if needed the UPF and the UE, e.g. if a new QoS flow is established.

- The intermediate non-3GPP access node performs ECN marking for L4S, when applicable, for the traffic of the QoS Flow dedicated to ECN marking for L4S as requested by the SMF.

- UPF handles the congestion information when received from the intermediate non-3GPP access node like for the NG-RAN case.

6.X+3.4	Impacts on services, entities and interfaces

-	SMF – Indicates the intermediate non-3GPP access nodes that for a dedicated QoS Flow ECN marking for L4S is enabled.

-	N3IWF/TNGF/W-AGF – When requested by the SMF for a dedicated QoS Flow performs, when applicable, ECN marking for L4S for the traffic of the dedicated QoS Flow.

End of changes