3GPP SA WG2 Meeting #160-Ad Hoc-e	 S2-2400967

22 - 29 January, 2024, Electronic

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4326 | Rev | 3 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Congestion handling with Alternative S-NSSAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The details of the congestion mitigation information has not been specified. If the S-NSSAI is congested then a complete replacement would likely result in the Alternative S-NSSAI becoming congested leading to a new trigger of replacement i.e. the replacement due to congestion needs to be done in a controlled way. If an S-NSSAI can be replaced by more than one S-NSSAI then the load distribution between the alternative S-NSSAIs should be controlled by the NSSF determining the Alternative S-NSSAI i.e. the congestion mitigation information would need to be able to provide the necessary information how to distribute the load. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified the congestion mitigation information from the NSSF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The replacement feature leads to new congestion in the Alternative S-NSSAI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.16.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 5.2.16.3.3	Nnssf_NSSAIAvailability_Notify service operation

Service operation name: Nnssf_NSSAIAvailability_Notify

Description: This service operation enables the NSSF to update a NF Service Consumer (e.g. AMF) with any S-NSSAIs restricted per validity time, or per TA and if needed subsequently lift any restriction per TA. This service operation also enables the NSSF to notify to the NF Service Consumer (e.g. AMF or NSSF in the VPLMN) that an S-NSSAI has to be replaced by an Alternative S-NSSAI or a Network Slice instance should be replaced.

Inputs, Required: Subscription Correlation ID.

Inputs, Conditional Required:

If this service operation is invoked by subscription for changes in the status of the NSSAI availability information, then the following inputs are required:

-	a list of TAIs and the S-NSSAIs for which the status is changed (restricted/unrestricted) per each TAI.

If this service operation is invoked by subscription for event notification for Network Slice Replacement, then the following inputs are required:

-	For replacement of an S-NSSAI of the serving PLMN: an Alternative S-NSSAI and the corresponding mapping to the S-NSSAI to be replaced.

-	For replacement of an HPLMN S-NSSAI: an Alternative S-NSSAI and the corresponding mapping to the S-NSSAIs of the HPLMN, PLMN ID.

-	For termination of the Network Slice replacement:

1)	indication to stop the Network Slice replacement for new UEs; or

2)	indication to terminate the Network Slice replacement and move back the UEs and PDU Sessions from the Alternative S-NSSAI to the S-NSSAI.

If this service operation is invoked by subscription for event notification for Network Slice Instance Replacement, then the following inputs are required:

-	The S-NSSAI(s) and NSI ID(s) for which the status is changed (e.g. congested or no longer available).

If this service operation is invoked by subscription for event notification for update of slice validity time, then the following inputs are required:

-	The S-NSSAI(s) and associated validity time for each S-NSSAI.

Inputs, Optional:

-	NSI IDs in the S-NSSAI for which the status is changed (restricted/unrestricted) per each TAI.

If this service operation is invoked by subscription for event notification for Network Slice Replacement or Network Slice Instance Replacement, then the following inputs are optionally provided with the Alternative S-NSSAI:

-	For replacement due to congestion: congestion mitigation information which includes information that the Network Slice replacement applies 1) for a percentage and/or 2) for new UEs registering to the Network Slice.

NOTE:	If the AMF does not receive any congestion mitigation information the AMF executes the replacement until the NSSF notifies the AMF to either stop the replacement or to terminate the replacement and move back the UEs and PDU Sessions from the Alternative S-NSSAI to the S-NSSAI as the S-NSSAI is available again for all the UEs.

Outputs, Required: None.

Outputs, Optional: None.

END CHANGE