3GPP TSG-WG SA2 Meeting #160ad	S2-2400981r01

January 22 – 26, 2024	(revision of S2-2312337)

Source: 	Samsung

Title: 	23.700-77: KI#3: Solution for Data channel interworking with MTSI UE

Document for: 	Approval

Agenda Item: 	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract of the contribution: This paper proposes solution for KI#3.

1 Introduction

This paper proposes solution for KI#3 in TR 23.700-77 for the SID FS_NG_RTC_Ph2, how the IMS DC can be interworking with MTSI UE.

2 Proposal

It is proposed to approve following content in TR 23.700-77.

# * * * * First change * * * *

Mapping of Solutions to Key Issues

Table6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | X |  |  |  |  |  |  |  |
| X |  |  | X |  |  |  |  |  |

# * * * * Second change (all new text)* * * *

6.X	Solution #X: DC interworking with DC service data provision to MTSI UE

6.X.1	Description

In some DC application service, it can serve the MTSI UE which only support conventional media (such as video, audio, text) with the same or similar user experience comparing to DC MTSI UE. For example, when a conference call with screen sharing, some users who join via data channel can control the screen or make some interactions (e.g. draw or write on the screen, make sticker/emoticon effects on the screen, etc.), while the other users who join via audio call cannot use the interactions with the screen. In this case, when the users who join via audio call can interacts with the local web browser, while the others can still use over data channel session.

This solution proposes how to provide the method to access to the DC interworking service (the http link) to the MTSI UE (originating side) and to establish a bootstrap/application data channel session to the DC MTSI UE (terminating side).

6.X.2	Procedures

The procedure for provision of DC service data for MTSI UE to access to the DC AS over IMS network is described in Figure 6.X.2.1-1. The procedure for provision of DC service data for MTSI UE to access to the DC AS over SMS is described in Figure 6.X.2.1-2.

Figure 6.X.2.1-1: Procedure for DC interworking with MTSI UE with DC service data provision over IMS network

- DC AS subscribes IMS AS via NEF and DCSF to notify when DC interworking for MTSI service profile needs to be applied for the user. The DC AS subscribes with the list of service id(s) which DC interworking is applicable, and/or the list of user id(s) which can use DC interworking service.

Editor's note: How to support event subscription and exposure of DC interworking for MTSI service is triggered by IMS AS is FFS (in this solution, or in the other solution in KI#1/KI#2).

- During IMS registration procedure for UE#1, IMS AS checks the DC interworking for MTSI service profile provided by the DC AS in step 1. If the user id of UE#1 is included in the list received from the DC AS, The IMS AS provides the DC interworking for MTSI service profile to the S-CSCF.

Editor's note: How to trigger IMS AS to check the DC interworking for MTSI service profile is FFS.

Editor's note: Whether the IMS AS should provide the DC interworking for MTSI service profile to S-CSCF is FFS.

- UE#1 initiates an INVITE request for audio RTP and RTCP streams.

- IMS AS checks the DC interworking for MTSI service profile provided by the DC AS in step 1, and determines whether the UE#1 subscribes to the DC interworking for MTSI service. I

Editor's note: How to trigger IMS AS to check the subscription data to determine whether the UE#1 subscribes to the DC interworking for MTSI service. DC interworking for MTSI service profile is FFS.

- IMS AS notifies to the DC AS via DCSF that DC interworking for MTSI service profile needs to be applied to the user of the UE#1. The IMS AS notifies with the service id, and/or the user id.

Editor's note: How to send Notifications is FFS and it is based on how to support event subscription and exposure of DC interworking for MTSI service is triggered by IMS AS is FFS (in this solution, or in the other solution in KI#1/KI#2).

- DC AS performs authentication and authorization procedures to verify whether this user is allowed to get the DC service data.

- After successful authentication and authorization, the DC AS provides the DC service data to IMS AS via DC AS. The DC service data includes http link for UE#1 to access to the DC AS.

- MF/MRF resources (MDC1, MDC2) for terminating side can be allocated.

- IMS AS adds SDP offer for DC bootstrap with an indication of DC interworking for MTSI to establish a DC bootstrap session between the DCSF of UE#1 side and the terminating side and UE#2. Any DC session between the DCSF of UE#1 side and the UE#1 needs not be established.

- The updated INVITE is transferred to UE#2.

11-16. UE#2 answers to the offer, and the UE#2 side resource information is transferred.

- 200 OK is sent to UE#1. IMS AS provides the DC service data which includes the http link received from the DC AS in step 7.

Editor's note: Whether and how the UE#1 (MTSI UE) can process the above DC service data is FFS.

- Voice channel is activated between UE#1 and UE#2 according to the audio answer in 200 OK.

- UE#1 accesses to the DC AS with the received DC service data (the http link) via local application (e.g. web browser).

- DC bootstrap for UE#2 is established.

- UE#2 downloads DC application, and the application DC for UE#2 is established.

- UE#1 interacts with DC AS via local application after step 19. The data generated in DC AS based on interaction with UE#1 is transferred to UE#2 via data channel.

Figure 6.X.2.1-2: Procedure for DC interworking with MTSI UE with DC service data provision over SMS

1-7. As same as step 1 to 7 of Figure 6.X.2.1-1.

- IMS AS adds SDP offer for DC bootstrap with an indication of DC interworking for MTSI to establish a DC bootstrap session between the DCSF of UE#1 side and the terminating side and UE#2. Any DC session between the DCSF of UE#1 side and the UE#1 needs not be established.

Editor's note: whether and how IMS AS can add SDP offer for DC bootstrap session for terminating side is FFS.

- The updated INVITE is transferred to UE#2.

10-15. UE#2 answers to the offer, and the UE#2 side resource information is transferred.

- 200 OK is sent to UE#1.

- Voice channel is activated between UE#1 and UE#2 according to the audio answer in 200 OK.

- IMS AS notifies to the DC AS that the IMS session for UE#1 is set up, and it is ready for DC interworking.

- DC AS provides the DC service data to UE#1 via DC AS. The DC service data includes http link for UE#1 to access to the DC AS.

Editor's note: Whether the SMS is used is FFS.

- UE#1 accesses to the DC AS with the received DC service data (the http link) via local application (e.g. web browser).

- DC bootstrap for UE#2 is established.

- UE#2 downloads DC application, and the application DC for UE#2 is established.

- UE#1 interacts with DC AS via local application after step 19. The data generated in DC AS based on interaction with UE#1 is transferred to UE#2 via data channel.

6.X.3	Impacts on Existing Nodes and Functionality

This solution may have the following impacts to exiting entities and interfaces:

DC AS:

- DC AS subscribes to IMS network via NEF for notification of the need of DC interworking for MTSI UE for certain service(s), and/or for certain user(s).

- DC AS verifies whether the user of MTSI UE is allowed to get the DC interworking service data with the DCSF.

- DC AS provides the DC interworking service data over IMS network or SMS that assists MTSI UE to access to the DC AS via local application.

NEF:

- NEF supports the event subscription and exposure of DC interworking for MTSI service between DC AS and IMS AS.

DCSF:

- DCSF supports the event subscription and exposure of DC interworking for MTSI service between DC AS and IMS AS.

- DCSF supports establishing a bootstrap DC only for terminating side based on the INVITE received from the IMS AS.

IMS AS:

- IMS AS provides the service profile for DC interworking to the S-CSCF during the IMS registration procedure.

- IMS AS request/get the DC interworking service data from the DC AS to provide to the MTSI UE.

- IMS AS updates the INVITE by adding the SDP offer for bootstrap DC establishment only for terminating side.

UE:

- DC MTSI UE notices the IMS media stream (video/audio/text) of the MTSI UE is associated with the application DC stream of the DC AS.

# * * * * End of changes * * * *