SA WG2 Meeting #160-Ad Hoc-e	S2-2400986

# E-meeting, January 22 – 29, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0107 | rev | 0 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding SL Positioning Server UE discovery as part of the initial discovery procedure. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CEWiT, Reliance Jio |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN2 has concluded that multiple UE roles can be broadcasted in the UE discovery message. This could enable combining the SL positioning server discovery together with the existing SL Reference UEs/Located UEs discoveries at the initial stage itself which could potentially optimize the procedures of ranging/sidelink positioning control and eliminate the need for SL positioning discovery at a later stage. The wording “… type of result received ...” is ambiguous. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding to Step 2 SL positioning server discovery, if needed. Consequently rephrasing Step 5. The wording of step 12 is modified for better clarity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multiple UE roles included in the discovery metafield would be redeemed as unnecessary in Release 18. SL positioning server cannot be discovered in step 2 even though the discovery message has the provision to identify server candidates. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Clause 7.12 in 38.305 would have impact. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.8	Procedures of Ranging/Sidelink Positioning control

Either UE-only Operation or Network-based Operation is applied in the Ranging/Sidelink Positioning control procedures.

UE-only Operation as specified in this clause is applied for the following cases:

-	Neither Target UE nor SL Reference UE is served by NG-RAN.

-	Network-based Operation is not supported by the 5GC network as described in clause 5.2.3.

-	Response to SL-MO-LR request allows the UE only operation for a period of time by the network due to congestion.

For any other cases, Network-based Operation as specified in clauses 6.20 of TS 23.273 [8] is applied.

Figure 6.8.1-1: Procedures for Ranging/Sidelink Positioning control (UE-only operation)

1.	UE1 (i.e. Target UE or a SL reference UE) may receive a Ranging/SL Positioning Service request from:

1a.	SL Positioning Client UE over PC5 during procedures for Ranging/SL Positioning service exposure through PC5 as defined in clause 6.7.1.1.

For absolute location, the service request includes the SL Positioning Client UE's user info and Target UE's user info, and required positioning QoS and may also include the user info for a list of candidate Located UE(s).

For relative location or ranging information, the service request includes the SL Positioning Client UE's user info, Target UE's user info, SL Reference UE's user info(UE2/.../UEn), and Ranging/SL Positioning QoS information.

1b.	RSPP application layer.

The service request includes type of the result (i.e. absolute location, relative location or ranging information) and the required QoS.

2.	UE1 discovers UE2/.../UEn (i.e. SL Reference UEs/Located UEs) as defined in clause 6.4, if needed. If UE1 does not support SL Positioning Server functionalities, a SL Positioning Server UE (either co-located with a SL Reference UE/Located UE or operated by a separate UE) may be discovered.

NOTE 1:	Details of security related procedures during UE discovery are developed by SA WG3.

3.	If none of UE1/.../UEn are served by NG-RAN or the serving network does not support Ranging/SL Positioning, UE-only Operation is applied.

4.	UE1 and UE2/.../UEn perform capability exchange. Step 4 may be performed during step 7 with coordination of SL Positioning Server UE.

5.	If UE1 does not support SL Positioning Server functionalities or UE1 opts to select a SL Positioning Server UE different from UE1, a SL Positioning Server UE (either co-located with a SL Reference UE/Located UE, or operated by a separate UE) is discovered (if not yet discovered in step2) and is selected. If a SL Positioning Server UE is co-located with a SL Reference UE/Located UE or operated by a separate UE, UE1 discovers (if not yet discovered in step2) and selects the SL Positioning Server UE as described in clauses 5.2.3 and 6.4. UE1 establishes the secure PC5 link with the selected SL Positioning Server UE.

If the Located UE is served by NG-RAN, it may use 5GC-MO-LR procedure to retrieve its absolute location.

NOTE 2:	Details of security and privacy related procedures during SL Positioning Server UE discovery and operation are developed by SA WG3.

6.	If a SL Positioning server UE is selected, UE 1 sends a Ranging/SL positioning request using supplementary RSPP signalling message to the selected SL Positioning Server UE. This request indicates the other UEs 2 to n using the Application layer ID and indicates the Ranging/SL positioning result types needed (e.g. absolute locations, relative locations or distances and directions between pairs of UEs). The required QoS for Ranging/SL positioning is also indicated.

7.	The SL Positioning Server UE sends requests to UE1 for capability of UE1 using the SLPP message and for the capabilities of UE2/.../UEn using the supplementary RSPP signalling (e.g. including SLPP containers that may contain Sidelink Positioning capability request for UE2/…/UEn) message with the corresponding Application Layer ID of UE2/.../UEn. UE1 responds to the SL Positioning Server UE with its own capability using SLPP message and the capabilities of UE2 to n using the supplementary RSPP signalling message (e.g. including SLPP containers that may contain Sidelink Positioning capability of UE2/…/UEn) with the corresponding Application Layer ID of UE2/.../UEn. If step 4 did not occur, UE1 retrieves capabilities from UE2/…/UEn using SLPP messages during this step.

The SL Positioning Server UE may downselect the UEs (e.g. UEx/…/UEy) from UE2/.../UEn for the subsequent SL positioning/ranging operation (e.g., based on UE capability).

8.	The SL Positioning Server UE provides the Sidelink Positioning assistance data to UE1.

-	For the Sidelink Positioning assistance data used by UE1, it is transmitted by SLPP message.

-	For the Sidelink Positioning assistance data used by UEx/…/UEy, it is transmitted using the supplementary RSPP signalling messages (e.g. including SLPP containers that may contain Sidelink Positioning assistance data for UEx/…/UEy) with the corresponding Application Layer ID of UEx/…/UEy and then UE1 sends them to each UEs (UEx/…/UEy) by SLPP messages.

9.	The SL Positioning Server UE sends requests to UE1 for SL measurement information of UE 1 and UEx/…/UEy, if the SL Positioning Server UE performs the result calculation. For the SL measurement information of UE1, the request uses the SLPP message. For the SL measurement information of UEx/…/UEy, the request uses the supplementary RSPP signalling message (e.g. including SLPP container that may contain Sidelink Positioning location measurements request for UEx/…/UEy) with the corresponding Application Layer ID of UEx/…/UEy. In addition, the SL Positioning Server UE may also request for the absolute location of UEx/…/UEy from UE1 using the supplementary RSPP signalling message with the corresponding Application Layer ID of UEx/…/UEy.

10.	SL-PRS measurement is performed between UE1 and UEx/…/UEy and possibly also amongst UEx/…/UEy. The UE1 requests for the SL measurement information from UEx/…/UEy by the SLPP messages and/or the absolute locations of UEx/…/UEy from UEx/…/UEy by supplementary RSPP signalling messages if requested in step9 by SL Positioning Server UE. The SL-PRS measurement data is transferred to UE1 if it supports SL Positioning Server functionalities and UE1 has not selected a SL Positioning Server UE (different from UE1) in step 5.

11.	SL-PRS measurement data of UE1 is transferred by SLPP message to the SL Positioning Server UE and SL-PRS measurement data of UEx/…/UEy is transferred by UE1 using the supplementary RSPP signalling message (e.g. including SLPP container that may contain SL-PRS measurement data of UEx/…/UEy) with the corresponding Application Layer ID of UEx/…/UEy to the SL Positioning Server UE if requested in step 9 in order to perform result calculation. Absolute location of UEx/…/UEy is transferred by UE1 to the SL Positioning Server UE by the supplementary RSPP signalling message with the corresponding Application Layer ID of UEx/…/UEy if requested in step9.

12.	Based on the result types requested in step 6, absolute location, relative location or ranging information is calculated at the SL Positioning Server UE.

NOTE 3:	Details of step 4-12 are developed by RAN WGs.

NOTE 4:	The supplementary RSPP signalling message mentioned above is conveyed by PC5-U and handled in the Ranging/SL Positioning layer, which detail design is left to stage 3.

NOTE 5:	The privacy aspects of transferring the location of Located UE via UE1 to the Server UE are developed by SA WG3.

13.	The SL Positioning Server UE sends a Ranging/SL positioning response using supplementary RSPP signalling message to UE1 including the result that is required in step 6.

14.	Ranging/SL Positioning result is transferred to:

14a.	SL Positioning Client UE over PC5 during procedures for Ranging/SL Positioning service exposure through PC5 as defined in clause 6.7.1.1;

14b.	Application layer.

# * * * * End of changes * * * *