3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2400987

# Online, Jan 22 – 29, 2024		(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1048 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Qos related correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The information in the procedure for "QoS Sustainability" analytics in a fine granularity area does not match the general description in 6.9.1, such as the description of fine granularity and the content of the additional information. The description of the procedure needs to be updated for alignment. The UL/DL packet delay GTP can be measured per UE per 5QI in both OAM as described in the clause 5.4.8 and clause 5.4.9 of TS 28.552 and UPF as described in clause 5.33.3 of TS 23.501. |  |  |  |  |  |  |  |  |
| ui |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the description of the procedure for "QoS Sustainability" analytics in a fine granularity area. - Add UPF as one of the data sources for obtaining UL/DL packet delay GTP in Table 6.13.2-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The description of UL/DL packet delay GTP is incomplete. - The description does not mismatch between 6.9.1 and 6.9.4.2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9.4.2, 6.13.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 6.9.4.2	Procedure for QoS Sustainability in a fine granularity area

Figure 6.9.4.2-1: Procedure for "QoS Sustainability" analytics in a fine granularity area

1.	The NF consumer requests or subscribes to analytics information on "QoS Sustainability" provided by the NWDAF. The parameters included in the request are defined in clause 6.9.1 of TS 23.288 [5]. The NF can request statistics or predictions or both.

The consumer provides the  "Location information" as defined in clause 6.9.1 when requesting QoS Sustainability analytics. If the AF doesn't provide TAIs or Cell IDs, the NWDAF as an 5GC internal NF needs to determine which cells are related to the fine granularity area.

The consumer may optionally provide UE Context or Subscription information such as one or more of the following: device speed or speed range, MEI or IMEISV or TAC range, equipment type.

2a.	If the request is authorized, and in order to provide the requested analytics, the NWDAF decides the AMF(s) based on the TAIs/Cell IDs, and obtains the UE list in the TAs/Cells from AMF by invoking Namf_EventExposure_Subscribe service operation using event ID "Number of UEs present in a geographical area" as described in TS 23.502 [3].

2b.	The NWDAF invokes Namf_EventExposure_Subscribe service operation to get the update of the UE list using event ID "UE moving in or out of Area of Interest" as described in TS 23.502 [3].

3.	The NWDAF obtains location information using LCS by initiating the LCS Service Request to the GMLC to get the location and optionally the speed of UEs from UE list provided by the AMF in step 2.

4.	The GMLC initiates the UE location service procedure and gets the location of the UEs as described in TS 23.273 [39].

5.	The GMLC provides location information for each UE in the UE list to the NWDAF.

6.	From the list of UE locations returned by the GMLC, the NWDAF identifies the UEs located in fine granularity area by comparing the UEs' locations to the fine granularity area, provided in step 1.

7.	The NWDAF invokes Namf_EventExposure_Subscribe service operation to get the serving SMF for the UE.

8.	Based on the serving SMF in step 7, the NWDAF invokes Nsmf_EventExposure_Subscribe service operation to get the UPF information and 5QI for the UE.

9.	The NWDAF may collect QoS information either from the UPF directly, or subscribe to the UPF via the SMF. The QoS information may include the bandwidth, packet delay for the UE and the information on the serving UPF node id.

NOTE 1:	How the data from UPF is retrieved (subscribed to on UPF and notified then by UPF) is defined in clause 5.8.2.17 of 23.501 [2] and in clause 4.15.4 of TS 23.502 [3].

10.	Optionally, the NWDAF may collect additional information for the UE Context or UE Subscription from the UDM such as PEI (if available). PEI may be used to retrieve, from GSMA database, additional information such as IMEI or IMEISV or TAC range, equipment type. Such additional information may be used by NWDAF to add more information to the collected measurements and filter those measurements that are applicable to the UE Device and Context Information for which analytics are requested by the service consumer.

11.	The NWDAF verifies whether the triggering conditions are met and derives the requested analytics. The NWDAF can detect the need for notification based on comparing the requested analytics of the target 5QI against the Reporting Threshold(s) provided by the consumer in any cell over the requested Analytics target period.

12.	The NWDAF provides the response or notification on "QoS Sustainability" to the NF consumer.

NOTE 2:	NWDAF may decide to ignore some of the filters if collected measurements are not sufficient to derive meaningful analytics.

NOTE 3:	In order to reduce the amount of information collected per measurement point, the additional information from UDM may only be collected for the events of GBR unfulfillment. In this way the additional analytics filter information may only be supported for 5QI of resource type GBR and for events of GBR unfulfillment.

# * * * * Second change * * * *

### 6.13.2	Input Data

The NWDAF supporting data analytics on Redundant Transmission Experience shall be able to collect UE mobility information from OAM, MDAS/MDAF, 5GC and AFs and service data from AF, as described in clause 6.7.2.2. In addition, NWDAF shall be able to collect the information for PDU session which is established with redundant transmission from the SMF. UE mobility information is specified in Table 6.7.2.2-1 and service data from AF related to UE mobility in Table 6.7.2.2-2. In addition, the NWDAF shall be able to collect performance measurements on user data congestion as specified in Table 6.8.2-1 for user data congestion analytics.

Additionally, the NWDAF collects the following input (see Table 6.13.2-1 and Table 6.13.2-2) according to existing measurements defined in clause 5.33.3 QoS Monitoring to Assist URLLC Service of TS 23.501 [2]. The NWDAF also collects the input from MDAS/MDAF of the end to end latency analysis in Table 6.13.2-3, as defined in clause 8.4.2.4.3 of TS 28.104 [45].

Table 6.13.2-1: Packet drop and/or packet delay measurement per QFI or GTP level

| Information | Source | Description |
| --- | --- | --- |
| UL/DL packet drop rate GTP-U | OAM (see NOTE 2) | UL/DL packet drop rate measurement on GTP path on N3. |
| UL/DL packet delay GTP | OAM (see NOTE 2 | UL/DL packet delay measurement round trip on GTP path on N3 |
| E2E UL/DL packet delay | UPF (see NOTE 3) | End-to-End (between UE and UPF) packet delay measurement between the UE and the UPF. |
| UL/DL packet drop/loss rate of RAN part | OAM (see NOTE 2) | UL/DL packet drop/loss rate measurement of RAN part. |
| NOTE 1: The information in this table is provided both as the base to compare with the redundant transmission performance as well as when redundant transmission is enabled. NOTE 2: Refer to clause 5.1 of TS 28.552 [8] for the performance measurement in NG-RAN and clause 5.4 of TS 28.552 [8] for the performance measurement in UPF. In addition, Annex A of TS 28.552 [8] describes various performance measurements. NOTE3: Refer to clause 5.33.3 of TS 23.501[2] for the packet delay measurement in the UPF. |  |  |

Table 6.13.2-2: The information related to PDU Session established

| Information | Source | Description |
| --- | --- | --- |
| DNN | SMF | Data Network Name associated for URLLC service. |
| UP with redundant transmission | SMF | Redundant transmission is setup or terminated. |

Table 6.13.2-3: Data collection from MDAS/MDAF of end-to-end latency analysis

| Information | Source | Description |
| --- | --- | --- |
| E2ELatencyIssueType | MDAF | Indication the type of the E2E latency issue. The allowed value is one of the enumerated values: RAN latency issue, CN latency issue. |
| AffectedObjects | MDAF | The managed object instances of subnetwork, managed elements or network slices where the latency issue happens. |

# * * * * End of changes * * * *