3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2401013r01

Online, Jan 22 – 29, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	New Key Issues for 5G Femto

Document for:	Approval

Agenda Item:	19.12

Work Item / Release:	FS_5G_Femto / Rel-19

Abstract: This contribution would like to propose new key Issues for 5G Femto.

# 1. Introduction/Discussion

There are two WTs captured in the SID objective of FS_5G_Femto:

WT#2: How to enable interworking between CAG and CSG cells.

WT#3: Study whether and how to support enabling the provisioning of subscribers allowed to access CAG cell and to manage access control by the CAG owner or an authorized administrator.

This contribution proposes new Key Issues corresponding to these two work tasks.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-45.

# * * * * First change * * * *

## 5.y	Key Issue #y: Enabling provisioning of subscribers allowed to access CAG cell

### 5.y.1	Description

The 5G NR Femto re-uses existing CAG mechanism defined for PNI-NPN for access control. In order to add flexibility to the 5G NR Femto, the owner of 5G NR Femto needs to be able to control which UE(s) can access to the HgNB.

NOTE X: Ownership of the 5G NR Femto (or CAG or both) concept and a mechanism will be defined in coordination with SA3.

This key issue aims to address the following aspects:

-	Whether and how can the CAG owner or an authorized administrator to enable provisioning CAG information to the network for 5G Femto only.

-	Whether and how to provide the CAG information to the UE to enable the UE to access to a CAG cell of a 5G Femto.

-	Whether and how can the CAG owner or an authorized administrator update access control information of the CAG cells of a 5G Femto.

-	Investigating the gaps/needs for provisioning related to CAG information in subscription data.

-	Identify solutions and entities for enhancements related to CAG information provisioning in subscription data if further enhancement h is need.

NOTE: The application of CAG information in 5GS after provisioning re-uses the exiting mechanism for PNI-NPN.

-	How to manage the NR Femto cell subscription by an operator;

-	How the NR Femto cell subscription is mapped to the CAG subscription;

-	How the UE is provisioned with the NR Femto cell subscription.

-	Whether to standardize and how the CAG owner or authorized administrator could provision subscribers to a CAG cell

-	Whether to standardize and how the CAG owner or authorized administrator could manage access control for a CAG cell.

-	How to support enabling the provisioning of subscribers allowed to access CAG cell and to manage access control by the CAG owner or an authorized administrator.

-	Whether and how to support provisioning subscribers allowed to access a CAG cell in the home and visited network

-	How to manage access control for 5G Femto cells:

  - How a 5G Femto owner can manage access control.

  - How an authorized administrator/3rd party can manage access control.

# * * * * End of changes * * * *