3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2401013r09

Online, Jan 22 – 29, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	New Key Issues for 5G Femto

Document for:	Approval

Agenda Item:	19.12

Work Item / Release:	FS_5G_Femto / Rel-19

Abstract: This contribution would like to propose new key Issues for 5G Femto.

# 1. Introduction/Discussion

There are two WTs captured in the SID objective of FS_5G_Femto:

WT#2: How to enable interworking between CAG and CSG cells.

WT#3: Study whether and how to support enabling the provisioning of subscribers allowed to access CAG cell and to manage access control by the CAG owner or an authorized administrator.

This contribution proposes new Key Issues corresponding to these two work tasks.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-45.

# * * * * First change * * * *

## 5.y	Key Issue #y: Enabling provisioning of subscribers allowed to access CAG cell and managing access control by the CAG owner or an authorized administrator

### 5.y.1	Description

The 5G NR Femto aims to re-use the existing CAG mechanism defined for PNI-NPN for access control. In order to add flexibility to the 5G NR Femto, the owner of 5G NR Femto needs to be able to control which UE(s) can access to the 5G NR Femto .

NOTE X: Ownership of the 5G NR Femto (or CAG or both) concept and a mechanism will be defined in coordination with SA3.

This key issue aims to address the following aspects:

-	Whether and how to enable the CAG owner or an authorized administrator to provision CAG information to the network for 5G Femto access control. The provisioning of CAG info to the network that 5G Femto serves and the network that the UE has subscription will be considered.

-	Whether and how to provide the CAG information to the UE to enable the UE to access to a CAG cell of a 5G Femto where the CAG cell can be in the home or visited network.

This key issue will consider the scenario of allowing the access to a CAG cell in the home and visited network

.

NOTE 1 It is assumed that the existing CAG concept defined for PNI-NPN is re-used for Femto access control without impacts to PNI-NPN.

NOTE 3: Ownership of the 5G NR Femto (or CAG or both) concept and a mechanism will be defined in coordination with SA3.

.

-

# * * * * End of changes * * * *