3GPP TSG-SA WG2 Meeting #160AH-e	S2-2401050

Online, January 22-29 2024

Title:	[draft] LS on Clarification of Allowed NSSAI for network slice replacement

Response to:	-

Release:	Rel-18

Work Item:	eNS_ph3

Source:	SA2

To:	CT1

Cc:	-

Contact Person:

Name:	alessio casati

E-mail Address:	alessio.casati@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none

1. Overall Description:

SA2 has discussed the inclusion of the replaced NSSAI and alternative s-NSSAI in the Allowed NSSAI when:

- The maximum number of S-NSSAIs in the allowed NSSAI would be overflown.

- The replaced S-NSSAI is not supported in the cell from which the UE is performing registration, but the Alternative s-NSSAI is supported in the same cell.

The current text in the SA2 specification implies the Alternative S-NSSAI is sent "additionally" i.e. both are included together. In addition the replaced S-NSSAI is included in PDU session establishment together with the alternative s-NSSAI.

Based on existing URSP logic, if the replaced S-NSSAI was not present in the Allowed NSSAI, related RSDs are not valid so no PDU session could be established for the replaced S-NSSAI. In addition, it is not possible to establish a PDU session indicating the replaced S-NSSAI as the S-NSSAI of the PDU session, together with the Alternative S-NSSAI, unless the replaced S-NSSAI is in the allowed NSSAI, per existing logic.

SA2 assumes the need to follow existing logic for the inclusion of the replaced S-NSSAI in the Allowed NSSAI and handling of the URSPs and PDU sessions SM signalling (with the additional inclusion of the Alternative S-NSSAI when the network slice replacement happens) . For this reason, assuming existing logic is adopted, and considering the Alternative S-NSSAI is provided additionally to the replaced S-NSSAI, both the S-NSSAIs are kept or removed together based on the logic of inclusion of the replaced S-NSSAI in the Allowed NSSAI.

This implies possible limitations for the cases 1) and 2) and therefore SA2 asks CT1 whether it is doable to allow the replaced S-NSSAI to be removed from the Allowed NSSAI (due to mentioned scenarios) while maintaining the Alternative S-NSSAI in the Allowed NSSAI and maintaining the PDU Sessions associated with the Alternative S-NSSAI and replaced S-NSSAI.

2. Actions:

To CT1 group.

ACTION: 	SA2 kindly asks CT1 to take into account the above information and provide an answer to SA2 question.

3. Date of Next SA2 Meetings:

Next SA2 Meetings dates