3GPP TSG-SA2 Meeting #160-Ad Hoc-e	S2-2401116

# Online, , 22nd Jan 2024 - 29th Jan 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0111 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates SL positioning control using supplementary RSPP signalling message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The followings needs to be shared in supplementary RSPP signalling message. - In subclause 6.7.1.1 procedure for ranging/SL positioning service exposure through PC5, there is a step for information sharing step and SL positioning service request between client UE and other UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified when supplementary RSPP signalling message is used and which information will be carried by supplementary RSPP signalling message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ranging/SL positioning operation between UEs will not work properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.1.1, 6.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 6.7.1.1	Procedures for Ranging/SL Positioning service exposure through PC5

Figure 6.7.1-1: Service exposure to SL Positioning Client UE via PC5

1.	When a Ranging/SL Positioning service is triggered, SL Positioning Client UE may perform discovery procedure for the Ranging/SL Positioning service defined in clause 6.4 with following updates:

-	When SL positioning service is triggered, the client UE tries to discover the target UE and/or one or more SL Reference UE(s)/Located UE(s). When Model B discovery used, in the Solicitation message, target UE's user info is included and role of target UE and role of SL Reference UE/Located UE are indicated in the RSPP metadata.

-	When Ranging service is triggered, the client UE tries to discover UE1 and/or UE2. When Model B discovery used, in the Solicitation message, UE1's user info and UE2's user info are included and role of target UE is indicated in the RSPP metadata.

During discovery for SL positioning service, the information of SL reference UE(s)/Located UE(s) is acquired.

2.	If discovery procedure succeeds, the SL Positioning Client UE establishes a PC5 connection with the discovered UE.

3.	The SL Positioning Client UE is informed of UE's status information indicating that UE has NAS connection or no NAS connection from the discovered UE(s).

Editor's note:	Need to determine if such indication is provided during discovery or step 3, and whether or not it requires RAN WG2 involvement.

The SL Positioning Client UE may receive a supplementary RSPP signalling message including list of SL Reference UE(s)/Located UE(s) from target UE over the PC5 connection.

4.	Based on UE's status information, the SL Positioning Client UE may downselect UE(s) to perform SL Positioning operation and select a UE (here UE1) to receive SL Positioning Service Request. The SL Positioning Client UE sends a SL Positioning Service Request using supplementary RSPP signalling message which includes info of SL Positioning Client UE, info of Target UE and info of SL Reference UE(s)/Located UE(s).

For absolute location, the SL Positioning Client UE sends a SL Positioning Service Request which includes info of SL Positioning Client UE, info of Target UE, and required positioning QoS. The Service Request may include info of list of candidate Located UE(s).

For relative location, the SL Positioning Client UE sends a SL Positioning Service Request which includes info of SL Positioning Client UE, info of Target UE, info of SL Reference UE(s), and required positioning QoS.

For ranging, the SL Positioning Client UE sends a SL Positioning Service Request which includes info of SL Positioning Client UE, info of UE1, info of UE2, and required positioning QoS.

For ranging, if UE1 and UE2 are not in proximity, the SL Positioning Service Request is responded with reject by the receiving UE (either UE1 or UE2) and remaining steps are not performed.

5.	Authorization check of the SL Positioning Client UE for the service request may be performed via SL Positioning Server UE or 5GC or internally by UE1 or target UE.

6.	The UE which received SL Positioning Service Request (here UE1) selects LMF or SL Positioning Server UE based on its status whether UE has NAS connection. Ranging/SL Positioning operation is performed as described in clause 6.8 among target UE, SL Reference UE(s)/Located UE(s) and SL Positioning Server UE or LMF.

7.	The Ranging/SL Positioning result is provided to the SL Positioning client UE by UE1.

# * * * * End of change * * * *