3GPP TSG-SA2 Meeting #160-Ad Hoc-e	S2-2401127

# Online, , 22nd Jan 2024 - 29th Jan 2024											   rev of S2-2312049

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0066 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on Application layer ID resolution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For Ranging/SL Positioning, Application layer ID to GPSI and GPSI to Application layer ID translation service is used to identify the UE in SL positioning operation by the network. And it is assumed that mapping of application layer ID to GPSI is provided by AF. At SA2 #158 meeting, it has been proposed to add translation service between application layer ID and SUPI but it is not possible to have such an translation service because SUPI as subscription data and application layer ID as application data shall be stored at separate storage. Therefore, the translation service between application layer ID and SUPI should be based on translation service between application layer ID and SUPI and translation service between SUPI and GPSI. According to current procedure, the procedure may be triggered by LCS client without AF. In this case, it is not clear how UE involved by SL positioning can be identified by 5G NW because LCS client may request location service based on SUPI and AF may not be involved for the location service. Therefore, it should be possible that mapping between application layer ID and GPSI and parameter provisioning of User Info ID from the preprovisioned data by operator. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Mapping between GPSI and Application ID may be configured by operator for regulatory location service. And User Info ID is added in parameter provisioned to the UE by NW. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In Ranging/SL Positioning without AF involvement, discovered UE may not be identified by NW. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.5, 4.3.9, 5.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.3.5	UDR

In addition to the functions defined in TS 23.287 [6] and TS 23.304 [7], the UDR performs the following functions:

-	Stores Ranging/SL Positioning service parameters.

-	Stores the mapping between Application Layer ID and GPSI provisioned by AF in application data.

NOTE: mapping between Application Layer ID and GPSI can be preprovisioned by operator in application data.

# * * * * Second change * * * *

### 4.3.9	GMLC

In addition to the functions defined in TS 23.273 [8], the GMLC supports the following:

-	enable trusted AFs and NFs to perform MT-LR by accessing GMLC directly with additional service parameters for Ranging/SL Positioning.

-	enable AFs and NFs to perform MT-LR by accessing GMLC through NEF with additional service parameters for Ranging/SL Positioning.

-	determine the serving AMF instances for the UEs involved in the Ranging/SL Positioning request and forward the request to respective serving AMF.

-	receive response from AMF and event reports from and return Ranging/SL Positioning result to NFs and AFs.

-	perform Application Layer ID to GPSI or GPSI to Application Layer ID resolution by querying NEF.

NOTE: When receiving location request by using SUPI of a UE from LCS client after translating SUPI to GPSI by using NUDM_sdm get service, GMLC performs Application Layer ID to GPSI or GPSI to Application Layer ID resolution by querying NEF.

# * * * * Third change * * * *

#### 5.1.1.2	Policy/Parameter provisioned to UE

The following sets of information for Ranging/SL positioning service is provisioned to the UE:

-	When the UE is served by NG-RAN, the PLMNs in which the UE is authorized to perform Ranging/SL positioning service over PC5 reference point for acquiring relative distance, relative direction, high accuracy relative distance and high accuracy relative direction.

-	When the UE is not served by NG-RAN, indicates whether the UE is authorized to perform Ranging/SL positioning service over PC5 reference point for acquiring relative distance, relative direction, high accuracy relative distance and high accuracy relative direction.

-	The mapping between Ranging/SL positioning services (e.g. ProSe identifiers, V2X service types) and Ranging/SL positioning QoS parameters as defined in clause 5.7.2.

-	The mapping between Ranging/SL positioning services (e.g. ProSe identifiers, V2X service types) and PQI values for RSPP transport QoS as defined in clause 5.7.3.

-	The authorized Ranging/SL Positioning role per PLMN, e.g. Target UE, SL Reference UE, Assistant UE, Located UE, SL Positioning Client UE, and SL Positioning Server UE.

-	When the UE is not served by NG-RAN, indicates the authorized Ranging/SL positioning role, e.g. Target UE, SL Reference UE, Assistant UE, Located UE and SL Positioning server UE.

-	When the UE is served by NG-RAN and when Network-based Operation is not supported by the 5GC network, whether it is allowed to use UE-only operation to perform Ranging/ SL Positioning.

-	User Info ID for Ranging/SL Positioning UE discovery.

Editor's note:	FFS if authorized roles for Assistant UE is needed.

NOTE: User Info ID is expected to be assigned uniquely for Ranging/SL positioning.

NOTE: User Info ID can be assigned via the Ranging/SL Positioning application server that allocates it.

# * * * * End of change * * * *