3GPP TSG-SA2 Meeting #160-Ad Hoc-e	S2-2401141

# Online, , 22nd Jan 2024 - 29th Jan 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0112 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update in the UE discovery and selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For alignment with RAN, UE’s coverage status, location information, etc should be considered for use in anchor UE selection and be specified by SA2. - Agreement from RAN2 in RAN2#123: Tell SA2 that like anchor UEs, a normative requirement on which server UE to select (e.g., ranking) will not be specified, and RAN2 do not intend to specify when the selection takes place; we leave it to SA2 to determine whether to specify anything. UE’s information such as UE’s coverage status and location information are included in selection criteria of located UE and SL positioning server UE. For alignment with RAN, those shall also be included in SL reference UE selection criteria. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE discovery procedure is update to include SL Reference UE discovery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ranging/SL positioning UE Discovery and Selection procedure will not work properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.2.X	SL Reference UE Discovery & Selection

When Sidelink Positioning is applied for Target UE in a SL-MO-LR, SL-MT-LR, 5GC-MO-LR or 5GC-MT-LR procedure, SL Reference UE may be discovered and selected:

-	When LMF determines to apply Sidelink Positioning for Target UE, LMF triggers the Target UE to perform discovery of the SL Reference UE.

-	When the Target UE determines to apply Sidelink Positioning, the Target UE triggers the discovery of the SL Reference UE.

The discovery of SL Reference UEs follows the same principles as specified in clause 5.2.1. The UE can indicate its role “SL Reference UE” in its list of supported roles during discovery, if it is authorized for Ranging/SL positioning service as specified in clause 5.1.

A Target UE may discover and select one or more SL Reference UEs to be used in the Ranging/SL positioning procedures as specified in clauses 5.3 to 5.5.

When multiple candidate SL Reference UEs are discovered, the SL Reference UE(s) is selected based on:

-	Capabilities of the candidate SL Reference UE(s), e.g. the supported Sidelink Positioning methods.

-	The required positioning QoS.

-	UE's information including whether UE is in coverage or not, etc.

# * * * * End of change * * * *