3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2401219r09

Online, Jan 22 – 29, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon, OPPO, China Mobile, Vivo?, LG Electronics?, TOYOTA MOTOR CORPORATION, KDDI?, NEC, ZTE?, Lenovo?, Nokia?, Nokia Shanghai Bell?, Samsung?

Title:	KI#3: New solution: supporting energy aware NF selection

Document for:	Approval

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract: This document proposes enhancement to support energy aware NF selection addressing KI#3 5GS enhancements for network energy saving and efficiency.

# 1. Introduction

## 1.1	Background

KI#3 proposes to study:

“‐	Whether and how to enhance the existing operations and procedures (e.g. to degrade the QoS profile within an SLA agreement, or to transmit the data flows in lower bandwidth), including

-	Whether and how to enhance the NF selection/re-selection related functionalities considering energy saving and energy efficiency based on e.g., NF energy states (e.g., coordination with the OAM in control plane procedures), analytics, UE subscription, capability of NFs, energy related information;”

Current NF selection defined in clause 6.3 of TS 23.501 mainly considers criteria such as Location information, Data Network Name, slicing information, capability, local policy, load condition etc. There is no considerations on energy related criteria, such as maximize the energy efficiency of the network, trade the performance with energy consumption following the SLA agreement, etc., and may lead to a sub-optimal result from the energy perspective.

## 1.2	Enhancements regarding energy aware NF selection

It is proposed that NRF maintains also energy related information (e.g., configured by OAM in NRF or registered by each individual NF) as part of NF profile for registered NF instance. Such energy related information includes:

-	Renewable Energy Ratio/Carbon emission factor: Amount of carbon emissions relative to an amount of resource consumption (e.g., carbon emissions / GB transmitted, carbon emissions / FLOPS, carbon emissions / GB stored).

-	Normalized Total energy: Amount of energy consumption relative to an amount of resource consumption (e.g., energy consumption / GB transmitted, energy consumption / FLOPS, energy consumption / GB stored).

NRF may returns energy related information as part of the NF profile in response to a NF discovery request. For the above additional information, NRF discovers a NF service considering also energy aware NF discovery policy. The energy aware NF discovery policy includes a score Metric for different information elements in the NF profile for the NF selection; or a network mode (e.g., low energy mode, high performance mode, green energy mode, etc.) which is mapped to a certain energy aware NF discovery policy. NRF evaluates the discovered NF instances based on energy aware NF discovery policy (i.e., sum up the scores of each evaluated information element of a candidate NF instance, prioritizes the NF instance(s) with higher final score, and feedback to the related service consumer).

In addition, NF profile can also be enhanced to guarantee a flexible NF selection regarding energy. The PDU Session Anchor is not expected to be changed during the lifetime of the PDU Session for SSC mode 1. However, energy related executions may require the NF to be switched to "inactive"/"energy saving" or other state to save energy or improve energy efficiency, which could result in the unavailability of the UPF. Therefore, it may negatively affect the flexibility of switching off the UPFs for energy saving, if more UPFs are managing PDU session with SSC mode1. Those UPFs should be deprioritized to be chosen as the PDU Session Anchor of the PDU session with SSC mode 1. Therefore, the energy related information also includes:

-	Indication of potential Energy execution: Indication that the whether the NF is subject to energy related execution present or future (e.g., deactivated/turned off because of the energy consumption in the off-peak hours), and it may also include the possibility (or the priority) to be subject to energy related execution, and the interval of the energy related execution.

This solution proposes to enhance the NF discovery and selection related functionalities for energy aware NF selection.

This paper also merges the proposal from S2-2400080, S2-2400085, S2-2400234, S2-2400541, S2-2400728, S2-2400757, S2-2400839, S2-2400931, S2-2401174, and S2-2401280.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-66.

# * * * * First change * * * *

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | X |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| X |  |  | X |  |
| … |  |  |  |  |

# * * * * Second change * * * * (all new)

## 6.x	Solution #X: Energy aware NF selection

### 6.x.1	Key Issue mapping

This solution addresses Key Issue #3.

### 6.x.2	Functional Description

This solution proposes to enhance the NF selection related functionalities for energy aware NF selection.

The following solution components are envisaged:

- NF registering new energy related information/state in its NF profile at the NRF, and updating such information at the NRF when needed, e.g. based on energy consumption or energy availability.

- Enhancing the NF Discovery procedure with new energy-related NF discovery query parameters and/or based on new energy-related policies at the NRF;

- As an option: enabling signaling of energy related information (i.e. energy state in which an NF is currently operating) in direct signaling towards peer NFs (e.g in HTTP headers, like for overload control – see clause 6.x.3.7 for details).

- NF service consumer taking into account the new energy related information from the NF profiles discovered from NRF using discovery procedure (and subsequent updates if subscribed to) and/or NF energy state received via direct signalling for the selection of a target NF service producer;

NRF maintains also energy related informationas part of NF profile for registered NF instance. Such energy related information includes:

Renewable Energy Ratio: Percentage of Energy Consumption using renewable sources in the total Energy Consumption using all sources.

Carbon emission factor: Amount of carbon emissions relative to an amount of resource consumption(e.g., carbon emissions / GB transmitted, carbon emissions / FLOPS, carbon emissions / GB stored).

Normalized Total energy: Amount of energy consumption relative to an amount of resource consumption (e.g., energy consumption / GB transmitted, energy consumption / FLOPS, energy consumption / GB stored).

Table 6.x.2-1: Potential Energy Information related enhancements included in the NF profiles.

| Information element name |  | Description |  |  |  | Category |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Energy Type information |  | Indicating energy type of the NF, e.g. renewable energy, non-renewable energy. |  |  |  | Optional |  |
| Energy cost | Information about energy cost, can be also based on time of day etc. |  |
| Renewable Energy Ratio/Carbon emission factor |  | Indicating percentage of Energy Consumption using renewable sources in the total Energy Consumption using all sources/ amount of carbon emissions relative to an amount of resource consumption (NOTE 1, 2) |  |  |  | Optional |  |
| Carbon Efficiency information |  | Indicate relative carbon efficiency level of the NF, e.g. an indication with Low, Medium, High, or a level value with 1, 2, 3, 4, 5. |  |  |  | Optional |  |
| Normalized Total energy |  | Amount of energy consumption relative to an amount of resource consumption (NOTE 1). |  |  |  | Optional |  |
| Energy inefficiency | Indicate if the NF face problems of excessive energy consumption as characterized by MDA in clause 8.4.4 TS 28.104. | Optional |
| Energy States (including potential schedule of energy states) |  | Energy state as specified in TS 28.310 [7], e.g., notEnergySaving state, energySaving state. Or other states. |  |  |  | Optional |  |
| NF profile attributes per energy states. | This enables NFs to register in advance different profile characteristics in their NF profiles, and then to only signal changes of their energy state towards the NRF and/or peer NFs, i.e. minimize the signalling in the network upon changes of the energy saving state. |  |
| Energy saving state stransition execution indication |  | Indicating whether the NF is subject to energy related execution (NOTE 3). |  |  |  | Optional |  |
| -energy saving state transition Priority |  | The priority to be subject to energy sving state transition execution. |  |  |  | Optional |  |
| Energy priority | an attribute that is set to determine the selection based on energy criteria by using this value than her than the above technical attributes | optional |
| NOTE 1: amount of resource consumption can be GB transmitted, GB stored, or FLOPs or summary of them per NF instance NOTE 2: Renewable Energy Ratio/Carbon emission factor KPI can be further adjusted/defined based on related study in SA5 NOTE 3: Energy related execution refers to switch the NF to energySaving state for energy saving. |  |  |  |  |  |  |  |

Besides energy states, side effects of energy related issues may cause further NF profile changes related to: (i) locality when an NF or NF service is relocated to a different data center due to energy reasons, or (ii) serving scope when the RAN coverage changes due to the configuration of energy saving states.

NRF may return energy related information as part of the NF profile in response to a NF discovery request. When receiving a request to discover a NF service, NRF considers also energy aware NF discovery policy/criteria (locally configured or indicated in the NF discovery request) and provides the NF information accordingly. The energy aware NF discovery policy/criteria include a score Metric for different information elements in the NF profile for the NF selection; or a network mode (e.g., low energy mode, high performance mode, green energy mode, etc.) which is mapped to a score Metric. NRF evaluates the discovered NF instances based on energy aware NF discovery policy/criteria (i.e., sum up the scores of each evaluated information element of a candidate NF instance, prioritizes the NF instance(s) with higher final score, and feedback to the related service consumer).

Table 6.x.2-2: Example NF discovery policy (score per information/Metric) in low energy mode

| Information/Metric | Score |
| --- | --- |
| Service Capacity [0..100] | 0 |
| Renewable Energy Ratio/Carbon emission factor [0..100] | 0 |
| Normalized Total Energy [0..100] | 1 |

NF service consumer may take into account energy related information (e.g. Energy States) from the NF profiles discovered from NRF using discovery procedure (and subsequent updates if subscribed to), for selection of a 5GC NF.

The NRF can receive at any time update of the NF energy attributes from NFs including change of energy state.

For example these energy states could be envisaged (these are for further study):

- Sleep state: the NF cannot serve any service request.​

- Energy saving state: energy saving is in progress. The NF supports limited-service capability (e.g. limited capacity of NF or NF services, shutting down NF services). ​

- Normal state: full performance mode

Editor's note: Potential additional states and substates of the Energy states are FFS. If energy substates were to be defined, it should also be possible to define related NF profile attributes.

The information may include a schedule of when it is applicable (e.g. the energy state transitions may be planned) and also information that is dependent on the energy state so there is no need to provide updated info to the service consumer when the state changes (see clause 6.x.3.6 for details)

And when SMF selects the UPF, the SMF can also make use of the NF Profiles by considering the potential usage of the UPF for energy related execution. The UPF with potential usage of energy related execution should be deprioritized to be chosen as the PDU Session Anchor of the PDU session with SSC mode 1.

### 6.x.3	Procedures

#### 6.x.3.1	General

Editor's note:	This clause describes high-level procedures and information flows for the solution.

The NRF service procedure as defined in clause 4.17 of TS 23.502 [3] are reused for energy related NF management and discovery.

The registered NF profile includes additional NF energy related information as described in clause 6.X.2. A service provider NF  registers the enhanced NF profile into the NRF.

An NF service consumer may include an NF discovery policy, such as score scheme, network mode in the NFDiscovery_Request to the NRF. Alternatively, OAM may configure the NRF with different NF discovery policy, e.g., per subscription, per network mode.

For UPF, the NF energy related information included in the NF profile can also be provisioned to SMF via N4 related procedures as defined in clause 4.4 of TS 23.502 [3].

#### 6.x.3.2	Service discovery considering NF energy related information

Figure 6.x.3.2-1: Service discovery considering NF energy related information

0a. Service provider NF registers the NF profile with additional energy related information into the NRF.

0b. NRF may be configured by OAM with a set of NF discovery policy (per subscription/network mode). Each NF discovery policy includes a score Metric for the to be evaluated information (defined in clause 6.x.2) in the NF profile. The following additions apply compared to clause 4.17.4 of TS 23.502 [3]:

1.	(Optional) The NF service consumer optionally indicates energy related criteria (e.g., network operation mode and/or energy efficiency mode) when invoking Nnrf_NFDiscovery_Request.

2.	The NRF authorizes the Nnrf_NFDiscovery_Request as in clause 4.17.4 of TS 23.502 [3].

3.	If allowed, the NRF determines a set of NF instance(s) matching the Nnrf_NFDiscovery_Request and local policies of the NRF and sends the NF profile(s) of the determined NF instances.

If Nnrf_NFDiscovery_Request includes energy related criteria, or based on local policy for energy aware NF discovery, NRF prioritizes the determined NF instances accordingly.

If allowed, the NRF selects the NF instance(s) from the prioritized NF instances. The NRF indicates the priority of the determined NF instances, and/or the selected NF instance(s) to the consumer NF in the Nnrf_NFDiscovery_Response.

The NF profile(s) of the determined NF instances may include additional energy related information in the NF profile (e.g., Renewable Energy Ratio/Carbon emission factor, Energy Type information, Carbon Efficiency information, Normalized Total Energy, Energy states (e.g. per S-NSSAI), or scores of the NF based on the energy aware NF discovery policy/criteria). w

4.	The NF service consumer selects the NF instance based on the NF profile information received from the NRF. If the NRF has selected the NF instance(s), the NF service consumer may skip this step.

#### 6.x.3.3	Selection of the UPF with considering energy related information

The information to be considered for UPF selection are defined in clause 6.3.3 of 3GPP TS 23.501 [2]. When SMF selects the UPF, the SMF can also make use of the NF Profiles by considering the potential usage of the UPF for energy related execution. The UPF with potential usage of energy related execution should be deprioritized to be chosen as the PDU Session Anchor of the PDU session with SSC mode 1.

Figure 6.x.3.3-1: Selection of the UPF with considering energy related information

0.	SMF provisioning of UPF instances using NRF, as described in clause 6.x.3.2.

1.	The UE registers to the PLMN, as described in clause 4.2.2.2 of TS 23.502 [3].

2.	The PCF makes use of UE Configuration Update procedure to provide UE Policy as defined in clause 4.2.4.3 of TS 23.502 [3].

3.	UE initiates the UE Requested PDU Session Establishment procedure with including the necessary information, including the requested SSC mode.

4.	Upon UPF selection, the SMF selects the UPF based on its Additional Energy Information  in the NF profile and the additional factors related to Additional Energy Information (Energy Efficiency and Energy saving parameters e.g. NF energy states, analytics, Energy Cost etc.). If the selected SSC mode of the PDU session is SSC mode 1, the SMF selects the UPF that is not subject to energy related execution, or selects the UPF with lower possibility/priority subject to energy related execution.

NOTE Z: These additional factors and NWDAF based related analytics will be determined within the scope of Key Issue 1 (e.g. types of energy related information, how such an information can be exposed, granularity etc.) and Key issue 3 related analytics solutions and referenced from this solution.

#### 6.x.3.4	Selection of the AMF considering energy related information

For AMF selection. the influencing factors are defined in the 3GPP TS 23.501 [2], cause 6.3.5.

On top of this, the additional factors related to Additional Energy Information (Energy Efficiency and Energy saving parameters e.g. NF energy states, analytics, Energy Cost etc.), may be taken into account while making AMF selection decision.

NOTE X: These additional factors and NWDAF based related analytics will be determined within the scope of Key Issue 1 (e.g. types of energy related information, how such an information can be exposed, granularity etc.) and Key issue 3 related analytics solutions and referenced from this solution.

#### 6.x.3.5	Selection of the SMF considering energy related information

It is assumed that the SMF had previously registered its NF profile including NF Energy Consumption into NRF, before SMF selection procedure.

Figure 6.X.3.5-1: SMF selection procedure

NOTE 1:	This procedure is based on Figure 4.3.2.2.3.2-1of TS 23.502 [3].

If the S-NSSAI is subject to energy control, AMF discovers SMF via NRF with the S-NSSAI.

Step 4: The NRF in serving PLMN provides to the AMF, e.g. FQDN or IP address, of a set of the discovered SMF instance(s) or Endpoint Address(es) of SMF service instance(s) in Nnrf_NFDiscovery_Request response message and possibly an NSI ID for the selected Network Slice instance corresponding to the S-NSSAI for subsequent NRF queries.

During this step, Energy Information (Energy Efficiency and Energy saving parameters e.g. NF energy states (e.g., per S-NSSAI), analytics, Energy Cost etc.) may be considered for the SMF selection and in addition, the other related analytics information provided by the NWDAF may be considered together.

Finally, NF profiles returned to AMF may include NF Energy Consumption as well as NF load information, NF capacity information, etc.

NOTE Y: These additional factors and NWDAF based related analytics will be determined within the scope of Key Issue 1 (e.g. types of energy related information, how such an information can be exposed, granularity etc.) and Key issue 3 related analytics solutions and referenced from this solution.

#### 6.x.3.6	NF selection based on time-based energy states/information from NRF

Figure 6.x.3.3-1: Example of NF selection based on pre-defined time-based energy states/information of NFs from NRF.

1a,1b. NF service producers include time-based energy related states/information.

2. The NF service consumer discover NF service producers. The NF profile of these NFs including time-based energy related information.

3. The NF service consumer select at step 3a NFp1 as it is in full performance mode, and additionally NF p2 after it enters full performance mode, based on the time-based received information from NRF.

#### 6.x.3.7	NF selection based on runtime energy state information received in service interactions between NFs

In addition to information on energy state that can be obtained from NRF by a NF service consumer, it is proposed it should be possible to exchange the information directly between NFs to have quicker updates at run time (assuming unscheduled changes can occur) as shown in figure 6.x.3.4-1. It is assumed that a NF instance provides its energy state to a peer NF if it is possible the NF instance can act as service producer for some services.

Figure 6.x.3.4-1: Example of NF selection based on runtime energy state information received in HTTP headers of service interactions.

1a,1b. NFs register with NRF including energy related information

2. NF service consumer discovers NF service producers

3a to 3d. During service interactions the NF consumer and NF produce exchange energy state information. A NF can use the information received in the event the NF in the future needs to contact the other NF for a service for which the other NF acts as a producer. The energy state information may also be associated to a validity time.

4. The NF service consumer may change the selected NF service producers based on the received energy state information in service interactions (e.g. as part of HTTP headers information). In step 4a the NF service consumer only interacts with NFp1 after it has detected it should no longer consider NFp2 based on the energy state.

### 6.x.4	Impacts on existing services, entities and interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

NRF:

-	Enhanced service discovery considering also energy related information and related NF discovery policy.

-	(optional) Feedback service consumer with the priority of the discovered NF instance.

-	(optional) Feedback service consumer with energy related information in the NF profile.

-	(In case of SMF selecting UPF) The NRF provides SMF the NF profiles of the UPFs including additional Energy Information.

NF service producers: registration at NRF of additional energy-related information in NF profiles

All NFs: handling of energy state information in HTTP headers

Service consumer NF:

-	Taking consideration of energy related information for 5GC NF selection.

-	(optional) includes energy related criteria (e.g., network operation mode and/or energy efficiency mode) in the service discovery request.

SMF:

-	SMF selects the UPF by considering the energy related information, e.g., SMF selects the UPF that is not subject to energy related execution, or selects the UPF with lower possibility/priority subject to energy related execution if the selected SSC mode is SSC mode 1 of the PDU session.

OAM:

-  Configure NRF with energy related NF discovery policy (e.g., score Metric)

Editor's note:	It is FFS whether the OAM can provision energy related information in the NF profile and energy related NF discovery policy to the NRF. The main approach considered is that NF register their information at NRF.

# * * * * End of changes * * * *