3GPP SA WG2 Meeting #160-Ad Hoc-e	S2-2401256

# Electronic meeting, 22-29 January, 2024 	(was S2-2312154)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4734 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TSCTSF subscribing to RAN TSS based on list of TAs, correction to ASTI without AF request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRS_URLLC |  |  |  |  |  | Date: |  |  | 2024-01-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR covers 4 areas for the support of TRS and alignment with stage 3: 1) Removal of ASTI_Subscribe/ASTI_Unsubscribe service operations. In the last CT3 meeting there was a discussion regarding the need for Nxxx_ASTI_Subscribe/Unsubscribe since the stage 3 encoding leads to the same methods and encoding for the subscribe operation than for the create/update service operation. As already described in stage 2, the subscription to ASTI can be done simultaneously to the creation of the ASTI configuration resource (with ASTI_Create operation). If not, the subscription over an already created ASTI configuration resource can be done with ASTI_Update operation (and also the unsubscription). 2) Clarification on the reporting of ASTI status to the AF. Ntsctsf_ASTI_UpdateNotify operation describes as optional input: Inputs, Optional: If the Status of the access stratum time distribution is inactive due to acceptance criteria, Clock quality acceptance criteria result, impacted UE list. “Clock quality acceptance criteria result” could be unclear for stage 3 (e.g., if the result contains specific time synchronization attributes or not). Following SA2 agreements to only repport acceptable/not acceptable result to the AF when clock quality acceptance criteria is provided, this CR adds a clarification in within the optional inputs as follows: Inputs, Optional: If the Status of the access stratum time distribution is inactive due to acceptance criteria, Clock quality acceptance criteria result (i.e., acceptable/not acceptable indication), impacted UE list. 3) Minor text correction for clause 4.15.9.3.2: When ASTI service is controlled based on subscription data by the AMF and the subscrition data include a Time Synchronization Coverage Area, the AMF determines the whether a UE is inside/outside of the Coverage Area without involving the TSCTSF. 4) Further clarification in clause 5.2.27.2.5 regarding the status notification for NW-TT ports. The operation of Ntsctsf_TimeSynchronization_ConfigUpdateNotify is unclear from stage 3 perspective, in TS 29.565 the following EN is open: Editor’s note: Whether it is required the report of the clock quality acceptance criteria for the NW-TTP ports (i.e., whether the clkQltIndOfNwtt attribute is needed) is FFS and requires SA2 clarifications. To resolve the EN in 29.565, Table 5.2.27.2.5-1 can further clarify the scope of the notifications of the clock quality acceptance criteria result from NW-TT side. This CR proposes additional description for this attribute. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removal of ASTI_Subscribe/ASTI_Unsubscribe service operations. - Clarification for Ntsctsf_ASTI_UpdateNotify operation. - Text correction in clause 4.15.9.3.2. - The AMF operation during the control of access stratum time synchronization service without AF request is corrected. - Further clarify the scope of NW-TT notification for clock quality acceptance criteria result I table 5.2.27.2.5-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For the first point, stage 3 will have redundant service operations for ASTI. For the second point, stage 3 may misunderstand the result the TSCTSF provides to the AF. For the fourth point, stage 3 will have a clearer description of NW-TT notification in Ntsctsf_TimeSynchronization_ConfigUpdateNotify operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.9.3.2, 4.15.9.4, 5.2.6.1, 5.2.6.28.7, 5.2.6.28.8, 5.2.27.1, 5.2.27.2.5, 5.2.27.4.6, 5.2.27.4.7, 5.2.27.4.8, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

##### 4.15.9.3.2	Time synchronization service activation

Figure 4.15.9.3.2-1: Time synchronization service activation

1.	The AF creates a time synchronization service configuration for a PTP instance by invoking Nnef_TimeSynchronization_ConfigCreate service operation. The request includes the parameters as described in Table 4.15.9.3-1. The request contains a Subscription Correlation ID and user-plane node ID as a reference to the target of the UEs and AF-sessions.

The create request creates also a subscription for the changes in the time synchronization service configuration. The AF may subscribe to receiving network time synchronization status report(s) as specified in clause 4.15.9.5.1.

2.	The NEF authorizes the request. After successful authorization, the NEF invokes the Ntsctsf_TimeSynchronization_ConfigCreate service operation with the corresponding TSCTSF, with the parameters as received from the AF.

If the request includes a spatial validity condition and if the AF uses a geographical area as a spatial validity condition, the NEF transforms this information into 3GPP identifiers (e.g. TAI(s)) based on pre-configuration.

The AF that is part of operator's trust domain may invoke the services directly with TSCTSF.

If the request includes a spatial validity condition and if the AF is within the operator's domain, the spatial validity condition shall comprise of a list of TA(s).

NOTE 1:	It is assumed that AFs within the operator's domain is aware of TAs that can be used to formulate a spatial validity condition for Time Synchronization Coverage Area (see clause 5.27.1.10 of TS 23.501 [2]).

3.	TSCTSF checks whether the AF requested parameters comply with the stored Time Synchronization Subscription data as defined in clause 5.27.1.11 of TS 23.501 [2], for that, the TSCTSF retrieves the Time Synchronization Subscription data from the UDM (as defined in clause 4.15.9.2). The TSCTSF determines the Time Synchronization Coverage Area and responds with the Ntsctsf_TimeSynchronization_ConfigCreate response as specified in clause 5.27.1.11 of TS 23.501 [2]. The Ntsctsf_TimeSynchronization_ConfigCreate response includes a PTP instance reference.

4.	The NEF responds with the Nnef_TimeSynchronization_ConfigCreate response, including a reference to the time synchronization service configuration (PTP instance reference).

5-6.	The TSCTSF uses the Subscription Correlation ID and user-plane node ID in Ntsctsf_TimeSynchronization_ConfigCreate to determine the target UEs and corresponding AF-sessions. The TSCTSF uses the parameters (e.g. requested PTP instance type, transport protocol and PTP profile) in the Ntsctsf_TimeSynchronization_ConfigCreate request to determine suitable DS-TT(s) and corresponding AF-sessions among all AF-sessions that are associated with the Subscription Correlation ID and user-plane node ID in the request.

The TSCTSF maintains association between list of suitable AF-sessions, corresponding time synchronization configuration, the PTP instance reference in 5GS, PTP instance references in each involved DS-TT and NW-TT and Subscription Correlation ID and user-plane node ID as given in step 1.

NOTE 2:	The AF-sessions that are not associated with a time synchronization configuration, are available to be selected as suitable AF-sessions in another Ntsctsf_TimeSynchronization_ConfigCreate request.

The TSCTSF uses the procedures described in clause K.2.2 of TS 23.501 [2] to configure and initialize the PTP instance in the DS-TT(s) and NW-TT. The TSCTSF constructs a PMIC to each DS-TT/UE to activate the time synchronization service in DS-TT in respect to the service parameters in the request in step 2. The TSCTSF constructs PMIC(s) and UMIC to NW-TT to activate the time synchronization service in NW-TT in respect to the service parameters in the request in step 2.

Upon reception of responses from each DS-TT and NW-TT, the TSCTSF determines the state of the time synchronization configuration.

The TSCTSF constructs a PMIC to each DS-TT/UE to subscribe for the port management information changes in the DS-TT. The TSCTSF constructs PMIC(s) and UMIC to NW-TT to subscribe for the port management and user-plane management information changes in NW-TT. The TSCTSF retrieves the PMIC(s) and UMIC via means of Npcf_PolicyAuthorization service operations.

The create request creates also a subscription for notifications for the changes in the time synchronization service configuration. If the AF provided clock quality acceptance criteria in step 1, the TSCTSF subscribes for notifications for changes in the NG-RAN and UPF/NW-TT timing synchronization status, as described in clause 4.15.9.5.1:

-	To determine the impacted UEs due to a timing synchronization status update reported by the NG-RAN, the TSCTSF follows the operation described in clause 5.27.1.12 of TS 23.501 [2].

-	To determine the impacted UEs due to a timing synchronization status update reported by the UPF/NW-TT, the TSCTSF verifies if the UPF/NW-TT is configured to send (g)PTP messages to the UEs/DS-TTs.

If the Ntsctsf_TimeSynchronization_ConfigCreate request contains a temporal validity condition with a start-time and/or the stop-time that is in the future, the TSCTSF maintains the start-time and stop-time for the time synchronization service for the corresponding time synchronization configuration. If the start-time is in the past, the TSCTSF treats the request as if the time synchronization service was activated immediately. When the start-time is reached, the TSCTSF proceeds as described in this step above. When the stop-time is reached for active time synchronization service configuration, the TSCTSF proceeds as Ntsctsf_TimeSynchronization_ConfigDelete was received as described in clause 4.15.9.3.4.

If the Ntsctsf_TimeSynchronization_ConfigCreate request contains a spatial validity condition, then the TSCTSF performs the following operations:

-	TSCTSF determines whether the TSCTSF has subscribed for the UE presence in Area of Interest composed by the TA(s) in the Time Synchronization Coverage Area. If not, the TSCTSF may either discover the AMF(s) serving the TA(s) comprising the Time Synchronization Coverage Area or discover the serving AMF(s) for each UE identified by a GPSI/SUPI as described in clause 5.27.1.10 of TS 23.501 [2].

Then the TSCTSF subscribes to the AMF(s) to receive notifications about the UE presence in Area of Interest using Namf_EventExposure operation with the corresponding event filters as described in clause 5.2.2.3 and in clause 5.3.4.4. of TS 23.501 [2]. The subscribed area of interest may be the same as the Time Synchronization Coverage Area or may be a subset of the Time Synchronization Coverage Area (e.g. a list of TAs) based on the latest known UE location.

-	In order to ensure that a TAI list specifying the AoI for the AMF is aligned with UE's Registration Area (RA), the following steps shall be performed:

-	When invoking the subscription with the AMF(s), the TSCTSF may provide an indication, a new Parameter Type = "Adjust AoI based on RA", that the AMF may adjust the received AoI depending on UE's RA.

-	After receiving the Namf_EventExposure_Subscribe request from the TSCTSF with the Parameter Type = "Adjust AoI based on RA" and specified AoI, the AMF compares TAs from the AoI with the UE's Registration Area (RA). If the AoI includes one or more TA(s) that are part of UE's current RA, the AMF reports the UE is inside the Area Of Interest, otherwise the AMF reports the UE is outside the Area Of Interest, as described in Annex D.

-	The AMF notifies the TSCTSF about the UE's presence in the AoI using the Namf_EventExposure_Notify service operation.

-	Based on the notification from the AMF and the Time Synchronization Coverage Area determined in step 3, the TSCTSF determines whether to activate time synchronization service for this UE:

-	If the UE location is within the Time Synchronization Coverage Area, the TSCTSF determines to activate time synchronization service for the UE/DS-TT creating the PTP port in DS-TT and adding it to the PTP instance. The TSCTSF uses the procedures described in clause K.2.2 of TS 23.501 [2] to configure and initialize the PTP instance in the DS-TT(s) and NW-TT.

-	If the UE location is outside the Time Synchronization Coverage Area, the TSCTSF determines not to activate time synchronization service and not to create a PTP port in a DS-TT.

The TSCTSF uses the procedure in clause 4.15.9.4 to activate or modify the 5G access stratum time distribution for the UEs that are part of the impacted PTP instance.

7.	The TSCTSF notifies the NEF (or AF) with the Ntsctsf_TimeSynchronization_ConfigUpdateNotify service operation, containing the PTP instance reference and the current state of the time synchronization service configuration.

If TSCTSF received spatial validity condition as part of the Ntsctsf_TimeSynchronization_ConfigCreate request, the TSCTSF notifies the NEF (or AF) with the Ntsctsf_TimeSynchronization_ConfigUpdateNotify service operation, whenever the UE moves in or out of the Area of Interest. The notification contains the PTP instance reference and the current state of the time synchronization service configuration.

8.	The NEF notifies the AF with the Nnef_TimeSynchronization_ConfigUpdateNotify service operation, containing the PTP instance reference and the current state of the time synchronization service configuration.

9.	Upon a change in the PTP instance in the DS-TT or NW-TT, the DS-TT or NW-TT report the change via PMIC or UMIC to the TSCTSF as described in clause K.2.2 of TS 23.501 [2].

Upon PDU Session release indication from a PCF, the TSCTSF removes the corresponding AF-session from the list of AF-sessions associated with the time synchronization configuration. The TSCTSF uses the procedure in clause 4.15.9.4 to remove the 5G access stratum time distribution parameters for the UE that is removed from the impacted PTP instance.

Upon PDU Session Establishment as defined clause 4.3.2.2.1, steps 10-13 in Figure 4.15.9.2-1 are repeated for the new PDU Session and the TSCTSF may notify the NEF (or AF) for the Time Synchronization capability event, optionally with the updated time synchronization capabilities, as described in step 12 in Figure 4.15.9.2-1.

NOTE 3:	Upon receiving the notification, the NEF (or AF) can use the Ntsctsf_TimeSynchronization_ConfigUpdate service operation to add the DS-TT/UE to the existing PTP instance and corresponding time synchronization service configuration.

If TSCTSF received spatial validity condition as part of the Ntsctsf_TimeSynchronization_ConfigCreate request, upon a change in the UE presence in Area of Interest, the TSCTSF determines if the spatial validity condition shall trigger an activation or deactivation of the time synchronization service:

-	If the UE has moved outside the Time Synchronization Coverage Area, then the TSCTSF temporarily removes the UE/DS-TT port from the PTP instance:

-	If the DS-TT is configured to send Sync, Follow_Up and Announce messages for the related PTP instance, then TSCTSF deactivates the Grandmaster functionality in the DS-TT using PMIC (see also clause K.2.2.4 of TS 23.501 [2]).

-	If NW-TT is configured to send Sync, Follow_Up and Announce messages on behalf of the DS-TT, then TSCTSF deactivates the Grandmaster functionality on behalf of the DS-TT in NW-TT using UMIC (see also clause K.2.2.4 of TS 23.501 [2]).

-	If the UE has moved inside the Time Synchronization Coverage Area, then the TSCTSF adds the DS-TT PTP port to the PTP instance and also (re-)activates the Grandmaster functionality (described in clause K.2.2 of TS 23.501 [2]).

Upon a NG-RAN timing synchronization status update, the NG-RAN report the change via AMF or provisioned via OAM to the TSCTSF as described in clause 4.15.9.5.1.

Upon a UPF/NW-TT timing synchronization status update, the UPF/NW-TT timing synchronization status update is reported via UMIC or provisioned via OAM to the TSCTSF as described in clause 4.15.9.5.1.

If TSCTSF received a NG-RAN or UPF/NW-TT timing synchronization status update and the time synchronization service has a configured clock quality acceptance criteria for the UE, the TSCTSF determines whether the clock quality acceptance criteria can still be met:

-	If the clock quality acceptance criteria can still be met, then TSCTSF may update the clockQuality information sent in Announce messages for the PTP instance using PMIC/UMIC reporting. The handling of Announce messages follows existing procedures as described in TS 23.501 [2].

-	If the clock quality acceptance criteria cannot be met or can be met again, then TSCTSF informs the AF about the acceptance criteria result (e.g., acceptable/not acceptable).

10.	The TSCTSF updates the state of the time synchronization configuration and may notify the NEF (or AF) with the Ntsctsf_TimeSynchronization_ConfigUpdateNotify service operation, containing the PTP instance reference and the updated state of the time synchronization service configuration, including whether there was a change in the UE's presence in the Time Synchronization Coverage Area (in cases when the AF has requested the service for a specific spatial validity condition), or the clock quality acceptance criteria result (in cases when the AF has requested the service with a clock quality acceptance criteria condition).

11.	The NEF notifies the AF with the Nnef_TimeSynchronization_ConfigUpdateNotify service operation, containing the reference to the time synchronization service configuration (PTP instance reference) and the updated state of the time synchronization service configuration.

If the AF receives a clock quality acceptance criteria result, the AF may update the configuration of the PTP instance by updating the PTP instance sending a Nnef/Ntsctsf_TimeSynchronization_ConfigUpdate or Nnef/Ntsctsf_TimeSynchronization_ConfigDelete request, as described in clauses 4.15.9.3.3 and 4.15.9.3.4

>>>> Next Change <<<<

#### 4.15.9.4	Procedures for management of 5G access stratum time distribution

The AF can use the procedure to activate, update or delete the 5G access stratum time distribution for one UE or a group of UEs.

The AF may query the status of the 5G access stratum time distribution using Nnef_ASTI_Get service operation. The Nnef_ASTI_Create and Nnef_ASTI_Update request may contain the parameters as described in Table 4.15.9.4-1. The AF may subscribe to 5G access stratum time synchronization status simultaneously while creating or updating the ASTI service. The AF may receive 5G access stratum time distribution status updates via Nnef_ASTI_UpdateNotify service operation.

Table 4.15.9.4-1: Description of 5G access stratum time distribution parameters

| Parameter | Description |
| --- | --- |
| 5G access stratum time distribution indication (enable, disable) | Indicates that the access stratum time distribution via Uu reference point should be activated or deactivated for the associated UE identities. |
| Time synchronization error budget | Indicates the time synchronization error budget for the time synchronization service (as described in clause 5.27.1.9 of TS 23.501 [2]). [optional] |
| Temporal Validity Condition | Indicates start-time and stop-time attributes that describe the time period when the time synchronization service is active. [optional] |
| Spatial Validity Condition | Indicates a geographical area (e.g. a civic address or shapes) or a TA list in which time synchronization service is enabled (NOTE 1). [optional] |
| Clock quality detail level | It indicates whether and which clock quality information to provide to the UE and can take one of the following values "clock quality metrics" or "acceptable/not acceptable indication". [optional] |
| Clock quality acceptance criteria | It indicates acceptance criteria for the UE. It is defined based on the attributes specified in Table 5.27.1.12-1 of TS 23.501 [2]. The parameter shall be included if the "clock quality detail level" = "acceptable/not acceptable". [conditional] |
| NOTE 1: A geographical area requested by an AF includes at least one tracking area (TA), i.e. the granularity is restricted to a TA level in this release of the specifications. |  |

Figure 4.15.9.4-1: Management of 5G access stratum time information

1.	AM Policy Association establishment as described in clause 4.16.1.

2.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

-	To create a new request, the AF provides access stratum time distribution parameters to the NEF using the Nnef_ASTI_Create service operation (together with the AF identifier and potentially further inputs as specified in table 4.15.9.4-1), including a target (one UE identified by SUPI or GPSI, a group of UEs identified by an External Group Identifier. The NEF forwards the GPSI or the External Group Identifier to the TSCTSF by including it inside the Ntsctsf_ASTI_Create request.

-	To update or remove an existing request, the AF invokes an Nnef_ASTI_Update or Nnef_ASTI_Delete service operation providing the corresponding time synchronization configuration id.

-	To query the status of the access stratum time distribution, the AF invokes Nnef_ASTI_Get service operation providing the target (a list of UE identities (SUPI or GPSIs) or an External Group Identifier).

-	If the AF includes clock quality detail level, the request creates a subscription at the TSCTSF for notifications for the changes in 5G access stratum time distribution status.

-	The create request creates also a subscription for the changes in 5G access stratum time distribution status. To subscribe to NG-RAN timing synchronization status updates, the AF can subscribe to notifications as described in clause 4.15.9.5.1.

The AF that is part of operator's trust domain may invoke the services directly with the TSCTSF and identifies the targeted UE(s) using SUPI(s) or an Internal Group Identifier.

If the request includes a spatial validity condition and the AF uses a geographical area as a spatial validity condition, the NEF transforms this information into 3GPP identifiers (e.g. TAI(s)) based on pre-configuration.

NOTE 1:	Steps 1 and 2 can occur in any order.

NOTE 2:	It is assumed that AFs within the operator's domain is aware of TAs that can be used to formulate a spatial validity condition for Time Synchronization Coverage Area (see clause 5.27.1.10 of TS 23.501 [2]).

3.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

-	The NEF authorizes the request. After successful authorization, the NEF invokes the Ntsctsf_ASTI_Create/Update/Delete/Get service operation with the TSCTSF discovered and selected as described in clause 6.3.24 of TS 23.501 [2].

-	The TSCTSF determines whether the targeted UE is part of a PTP instance in 5GS, if so the TSCTSF rejects the request (steps 4-11 are skipped).

-	The TSCTSF checks whether the AF requested parameters comply with the stored Time Synchronization Subscription Data as defined in clause 5.27.1.11 of TS 23.501 [2].

-	The TSCTSF calculates the Uu time synchronization error budget as described in clause 5.27.1.9 of TS 23.501 [2]. This does not apply in case of Ntsctsf_ASTI_Delete service operation.

(When the procedure is triggered by PTP instance activation or modification in the TSCTSF):

-	The TSCTSF calculates the Uu time synchronization error budget as described in clause 5.27.1.9 of TS 23.501 [2] for corresponding SUPIs that are part of the PTP instance.

4.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

If the AF targeted UE(s) are identified by GPSI(s) or an External/Internal Group Identifier, the TSCTSF uses the Nudm_SDM_Get request to retrieve the subscription information (SUPI(s)) from the UDM using each GPSI or the External Group Identifier as received from the NEF, or an Internal Group Identifier as provided by the AF directly.

5.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

The UDM provides the Nudm_SDM_Get response containing SUPI(s) that are mapped from each received GPSI or the External/Internal Group Identifier and identify the targeted UEs.

6a.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

If the Ntsctsf_ASTI_Create request in step 2 contains a spatial validity condition, then the TSCTSF performs the following operations:

-	For each target UE, TSCTSF checks with the stored Time Synchronization Subscription data if the spatial validity condition is allowed. The TSCTSF determines whether the TSCTSF has subscribed for the UE presence in Area of Interest composed by the TAs list in the spatial validity condition. If not, the TSCTSF may either discover the AMF(s) serving in the TAs comprising the spatial validity condition or discovers the serving AMF(s) for each UE identified by a GPSI/SUPI as described in clause 5.27.1.10 of TS 23.501 [2].

Then the TSCTSF subscribes to the AMF(s) to receive notifications about the UE presence in Area of Interest using Namf_EventExposure operation with the corresponding event filters as described in clause 5.2.2.3 and in clause 5.3.4.4 of TS 23.501 [2]. The subscribed area of interest may be the same as the spatial validity condition or may be a subset of the spatial validity condition (e.g. a list of TAs) based on the latest known UE location.

-	In order to ensure that a TAI list specifying the AoI for the AMF is aligned with UE's Registration Area (RA), the following steps shall be performed:

-	When invoking the subscription with the AMF(s), the TSCTSF may provide an indication, a new Parameter Type = "Adjust AoI based on RA", that the AMF may adjust the received AoI depending on UE's RA.

-	After receiving the Namf_EventExposure_Subscribe request from the TSCTSF with the Parameter Type = "Adjust AoI based on RA" and specified AoI, the AMF compares TAs from the AoI with the UE's Registration Area (RA). If the AoI includes one or more TA(s) that are part of UE's current RA, the AMF reports the UE is inside the Area Of Interest, otherwise the AMF reports the UE is outside the Area Of Interest, as described in Annex D.

-	The AMF notifies the TSCTSF about the UE's presence in the AoI using the Namf_EventExposure_Notify service operation.

-	Based on the notification from the AMF and spatial validity condition received in step 1, the TSCTSF determines whether to activate time synchronization service for this UE:

-	If the UE location is within the spatial validity condition, the TSCTSF determines to enable access stratum time distribution for the UE.

-	If the UE location is outside the spatial validity condition, the TSCTSF determines to disable access stratum time distribution for the UE.

If the Ntsctsf_ASTI_Create request in step 2 contains a temporal validity condition with a start-time and/or the stop-time that is in the future, the TSCTSF maintains the start-time and stop-time for the time synchronization service for the corresponding time synchronization configuration. If the start-time is in the past, the TSCTSF treats the request as if the time synchronization service was activated immediately. When the start-time is reached, the TSCTSF proceeds with the subsequent steps. When the stop-time is reached for active time synchronization service configuration, the TSCTSF proceeds as if Nnef_ASTI_Delete was received.

6b.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

The TSCTSF checks the AF request with the stored Time Synchronization Subscription data as described in clause 5.27.1.11 of TS 23.501 [2] for any targeted UE. If the AF is not authorized, steps 7-11 are skipped.

7.	The TSCTSF searches the PCF for the UE using Nbsf_Management_Subscribe with a SUPI as an input parameter, indicating that it is searching for the PCF that handles the AM Policy Association of the UE.

8.	The BSF provides to the TSCTSF the identity of the PCF for the UE for the requested SUPI via an Nbsf_Management_Notify operation. If matching entries already existed in the BSF when step 6 is performed, this shall be immediately reported to the TSCTSF.

9.	The TSCTSF sends to the PCF for the UE its request for the AM policy of the UE (identified by SUPI) using Npcf_AMPolicyAuthorization request, containing the 5G access stratum time distribution indication (enable, disable), optionally the calculated Uu time synchronization error budget, optionally the clock quality detail level and clock quality acceptance criteria.

(When the procedure is triggered by PTP instance activation, modification, or deactivation in the TSCTSF):

The TSCTSF ensures the 5G access stratum time distribution for the UE is enabled if at least one of the PTP instances the UE is part of is active.

10.	If the PCF receives multiple time synchronization error budgets for a given UE, then the PCF picks the most stringent budget. The PCF takes a policy decision and then the PCF may initiate an AM Policy Association Modification procedure for the UE as described in clause 4.16.2.2 to provide AMF the 5G access stratum time distribution parameters. As part of this, the AMF shall, if supported, store the 5G access stratum time distribution indication (enable, disable), the Uu time synchronization error budget, clock quality detail level and clock quality acceptance criteria, and the UE reconnection indication in the UE context in AMF and send the 5G access stratum time distribution indication (enable, disable), the Uu time synchronization error budget, clock quality detail level and clock quality acceptance criteria when they are available, to NG-RAN during mobility registration, AM policy modification, Service Request, N2 Handover and Xn handover as specified in TS 38.413 [10]. The NG-RAN node shall, if supported, store the information in the UE Context. Based on this information, the NG-RAN node provides the 5GS access stratum time to the UE according to the Uu time synchronization error budget as provided by the TSCTSF (if supported by UE and NG-RAN) and NG-RAN provides timing synchronization status reports to the UE (as described in clause 4.15.9.5.2).

If the UE has indicated a support for network reconnection due to RAN timing synchronization status change as described in clause 5.4.4a of TS 23.501 [2], and if the AMF received "clock quality detail level" as part of an AM Policy Association Modification procedure, the AMF sets the "UE reconnection indication" for the UE to connect to the network in the case when the UE later detects that the NG-RAN timing synchronization status has changed while the UE is in CM-IDLE or CM-CONNECTED with RRC_INACTIVE state. The AMF shall send the UE reconnection indication to the UE updating the UE configuration as defined in clause 4.2.4.2.

If the AMF receives Uu time synchronization error budget in the Access Stratum Time Synchronization Service Authorization as part of the Access and Mobility Subscription data during registration procedure (see clause 4.28.2.1) and in the AM Policy, the AMF should use the value received from the AM policy.

NOTE 3:	This release of the specification assumes that deployments ensure that the targeted UEs and the NG-RAN nodes serving those UEs support Rel-17 propagation delay compensation as defined in TS 38.300 [9].

11.	The PCF of the UE replies to the TSCTSF with the result of Npcf_AMPolicyAuthorization operation.

12.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

The TSCTSF responds the AF with the Ntsctsf_ASTI_Create/Update/Delete/Get service operation response.

13.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

The NEF informs the AF about the result of the Nnef_ASTI_Create/Update/Delete/Get service operation performed in step 2.

14.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

If TSCTSF received spatial validity condition as part of the Ntsctsf_ASTI_Create request in step 2, upon the reception of a change in the UE presence in Area of Interest notification, the TSCTSF determines if the spatial validity condition shall trigger an activation or deactivation of the access stratum time distribution:

-	If the UE has moved inside the Time Synchronization Coverage Area, then the TSCTSF determines to enable access stratum time distribution for the UE.

-	If the UE has moved outside the Time Synchronization Coverage Area, then the TSCTSF determines to disable access stratum time distribution for the UE.

If TSCTSF received clock quality acceptance criteria as part of the Ntsctsf_ASTI_Create request in step 2, upon a reception of a UE presence in Area of Interest notification, the TSCTSF determines if the UE is impacted correlating UE presence in Area of Interest notifications provided by the serving AMF and the timing synchronization status received (e.g. degradation, failure, recovery) as described in clause 5.27.1.12 of TS 23.501 [2].

15-16.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

If TSCTSF determines to modify access stratum time distribution for the UE in step 14 for which the AF requested access stratum time distribution, the TSCTSF notifies the service status to AF.

If the TSCTSF determines a change in the support of the clock quality acceptance criteria for the UE in step 14 for which the AF requested access stratum time distribution, the TSCTSF includes the clock quality acceptance criteria result to the notification to the AF. Based on this notification, the AF decides whether to modify the ASTI service configured for the UE using Ntsctsf_ASTI_Update service.

17.	If there was a failure/degradation/improvement event in step 14, the NG-RAN provides timing synchronization status reports to the UE (as described in clause 4.15.9.5.2).

>>>> Next Change <<<<

### 5.2.6	NEF Services

#### 5.2.6.1	General

The following table shows the NEF Services and Service Operations:

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_EventExposure | Subscribe | Subscribe/Notify | AF, NWDAF |
|  | Unsubscribe |  | AF, NWDAF |
|  | Notify |  | AF, NWDAF |
| Nnef_PFDManagement | Fetch | Request/Response | SMF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Notify |  | SMF |
|  | Unsubscribe |  | SMF |
|  | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_EASDeployment | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Unsubscribe |  | SMF |
|  | Notify |  | SMF |
| Nnef_ECSAddress | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Unsubscribe |  | SMF |
|  | Notify |  | SMF |
| Nnef_TrafficInfluenceData | Subscribe | Subscribe/Notify | SMF |
|  | Unsubscribe |  | SMF |
|  | Notify |  | SMF |
| Nnef_ParameterProvision | Update | Request/Response | AF |
|  | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_Trigger | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
| Nnef_BDTPNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_TrafficInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | AppRelocationInfo | Subscribe/Notify | AF |
| Nnef_ChargeableParty | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_AFsessionWithQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Revoke | Request/Response | AF |
| Nnef_MSISDN-less_MO_SMS | Notify | Notify | AF |
| Nnef_ServiceParameter | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_APISupportCapability | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe | Subscribe/Notify | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_NIDDConfiguration | Create | Request/Response | AF |
|  | TriggerNotify | Subscribe/Notify | AF |
|  | UpdateNotify | Subscribe/Notify | AF |
|  | Delete | Request/Response | AF |
| Nnef_NIDD | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
|  | GroupDeliveryNotify | Notify | AF |
| Nnef_SMContext | Create | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
|  | DeleteNotify | Subscribe/Notify | SMF |
|  | Delivery | Request/Response | SMF |
| Nnef_AnalyticsExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
|  | Fetch | Request/Response | AF |
| Nnef_UCMFProvisioning | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ECRestriction | Get | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ApplyPolicy | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_Location | LocationUpdateNotify | Notify | AF |
| Nnef_TimeSynchronization | ConfigUpdate | Request/Response | AF |
|  | ConfigCreate | Request/Response | AF |
|  | ConfigDelete | Request/Response | AF |
|  | ConfigUpdateNotify | Subscribe/Notify | AF |
|  | CapsSubscribe | Subscribe/Notify | AF |
|  | CapsUnsubscribe | Subscribe/Notify | AF |
|  | CapsNotify | Subscribe/Notify | AF |
| Nnef_ASTI | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
|  | UpdateNotify | Subscribe/Notify | AF |
| Nnef_AMPolicyAuthorization | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | Subscribe |  | AF |
|  | Unsubscribe |  | AF |
| Nnef_AMInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_UEId | Get | Request/Response | AF |
| Nnef_SMService | MoForwardSm | Request/Response | SMS-SC |
| Nnef_PDTQPolicyNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_MemberUESelectionAssistance | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
| Nnef_AF_request_for_QoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Revoke | Request/Response | AF |
| Nnef_DNAIMapping | Subscribe | Subscribe/Notify | AF, NWDAF |
|  | Unsubscribe |  | AF, NWDAF |
|  | UpdateNotify |  | AF, NWDAF |
| Nnef_UEAddress | Get | Request/Response | AF |

>>>> Next Change <<<<

##### 5.2.6.28.7

##### Void

>>>> Next Change <<<<

##### 5.2.6.28.8

##### Void

>>>> Next Change <<<<

### 5.2.27	TSCTSF Services

#### 5.2.27.1	General

The following table shows the TSCTSF Services and Service Operations:

Table 5.2.27.1-1: NF Services provided by the TSCTSF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ntsctsf_TimeSynchronization | ConfigUpdate | Request/Response | AF, NEF |
|  | ConfigCreate | Request/Response | AF, NEF |
|  | ConfigDelete | Request/Response | AF, NEF |
|  | ConfigUpdateNotify | Subscribe/Notify | AF, NEF |
|  | CapsSubscribe | Subscribe/Notify | AF, NEF |
|  | CapsUnsubscribe | Subscribe/Notify | AF, NEF |
|  | CapsNotify | Subscribe/Notify | AF, NEF |
| Ntsctsf_ASTI | Create | Request/Response | AF, NEF |
|  | Update | Request/Response | AF, NEF |
|  | Delete | Request/Response | AF, NEF |
|  | Get | Request/Response | AF, NEF |
|  | UpdateNotify | Subscribe/Notify | AF, NEF |
| Ntsctsf_QoSandTSCAssistance | Create | Request/Response | AF, NEF |
|  | Update | Request/Response | AF, NEF |
|  | Delete | Request/Response | AF, NEF |
|  | Notify | Subscribe/Notify | AF, NEF |
|  | Subscribe | Subscribe/Notify | AF, NEF |
|  | Unsubscribe | Subscribe/Notify | AF, NEF |

>>>> Next Change <<<<

##### 5.2.27.2.5	Ntsctsf_TimeSynchronization_ConfigUpdateNotify operation

Service operation name: Ntsctsf_TimeSynchronization_ConfigUpdateNotify

Description: Forward the notification for the time synchronization configuration. When the TSCTSF detects a change corresponding to a time synchronization configuration, it invokes Ntsctsf_TimeSynchronization_ConfigUpdateNotify service operation to the NF consumer(s) which has subscribed for the event. The following table describes the parameters in the event.

Table 5.2.27.2.5-1: Time Synchronization configuration event parameters

| Event filter | Description |
| --- | --- |
| For each PTP port in the PTP instance |  |
| Either UE identity (for a DS-TT port), or "N6 interface" indication | Identifies the UE/DS-TT which the parameters below apply. "N6 interface" indicates that the parameters below apply to the N6 interface. If the "PTP port" needs to be identified, this field refers to the UE identity (GPSI or SUPI). If the N6 termination needs to be identified, then this field indicates "N6 interface" flag, instead of SUPI or GPSI. |
| PTP port state | Active or Inactive. Active = PTP port state is in Leader, Follower or Passive state. Inactive = PTP port is in any other state. If any of the PTP ports in NW-TT are in Leader, Follower, or Passive, then "active" is reported for N6 interface, otherwise "passive" is reported for N6 interface. |
| Clock quality acceptance criteria result | Indicates if the clock quality acceptance criteria for the service can be met or not. For DS-TT: Acceptable = PTP port meets the clock quality acceptance criteria. Non acceptable = PTP port does not meet the clock quality acceptance criteria. For NW-TT: Acceptable = If any of the PTP ports in NW-TT meet the clock quality acceptance criteria, then “Acceptable” is reported for N6 interface. Non acceptable = Otherwise; in which case “Non acceptable” is reported for N6 interface. |

NOTE:	Leader and Follower terms in this specification are aligned with NOTE 2 in clause 5.27.1.2.2.1 of TS 23.501 [2].

Inputs, Required: PTP instance reference, current state of the time synchronization configuration (i.e. whether the time synchronization configuration is active or not for the PTP ports of the PTP instance).

Inputs, Optional: Time synchronization information as described in Table 5.2.27.2.5-1.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

>>>> Next Change <<<<

##### 5.2.27.4.6	Ntsctsf_ASTI_UpdateNotify operation

Service operation name: Ntsctsf_ASTI_UpdateNotify

Description: Forward the notification for the 5G access stratum time distribution status change. When the TSCTSF detects a change, it invokes Ntsctsf_ASTI_UpdateNotify service operation to the NF consumer(s) which has subscribed for the event.

Inputs, Required: Notification Correlation ID, Status of the access stratum time distribution (active or inactive).

Inputs, Optional: If the Status of the access stratum time distribution is inactive due to acceptance criteria, Clock quality acceptance criteria result (i.e., acceptable/not acceptable indication), impacted UE list.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

>>>> Next Change <<<<

##### 5.2.27.4.7

##### Void

>>>> Next Change <<<<

##### 5.2.27.4.8

##### Void

>>>> End of Changes <<<<