3GPP TSG-WG SA2 Meeting #160AHe	S2-2401259

# Electronic meeting, 22 - 29 Jan 2024		(revision of S2-24)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0114 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for alignments to TS 23.273/TS 23.586/SA3/RAN WGs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some unalignments have been identified between TS 23.586 and TS 23.273: - In TS 23.273 SL-MT-LR procedure, LMF can determine to apply “Network-assisted SL Positioning”, in which case, the Target UE performs result calculation; however, in TS 23.586, the LMF can also select the SL Positioning Server UE. It is proposed to follow TS 23.273 procedure. RAN2 has dedided to use SLPP/RSPP, instead of LPP, as the protocol between UE and LMF. The corresponding descriptions in TS 23.586 need to be updated. Additionally, RAN2#121 has reached the following agreements reguading to the functionalities of SL Positioning Server UE: “RAN2 confirm that for cases without LMF involvement, besides method determination, assistant data distribution and anchor UE selection (agreed in RAN2), the SL positioning server UE may perform SL-PRS configuration coordination and location calculation.” The related EN in clause 5.3.1 can be removed and the terminology of SL Positioning server UE should be updated. Moreover, it has been agreed on SA2#160 that “RSPP includes SLPP messages and Supplementary Service messages transferring between UE and LMF. Whether LCS Supplementary Service messages are reused or new Supplementary Service messages will be developed is determined by CT WGs. RSPP also includes SLPP messages and Supplementary RSPP signalling messages transferring over SR5. Supplementary RSPP signalling is assumed to be developed by CT WGs.”, which should also be refleted in both TS 23.586 and TS 23.273. Furthermore, SA3 has developed the solutions on service authurization and privacy checking, the corresponding ENs can be removed from both TS 23.586 and TS 23.273 by referring to TS 33.533. The text related to security and privacy requirements described in TS 33.533 is proposed to be removed by simply referring to TS 33.533. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrections on Network-assisted SL Positioning, and the related procedures - Corrections on SLPP, instead of LPP, usage between LMF and UE - Corrections on RSPP scope - Corrections on security related text |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unalignment between TS 23.586 and TS 23.273. Unalignment between TS 23.586 and TS 33.533. Unalignment between TS 23.586 and TS 38.355. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 5.3.1, 5.5.2, 5.5.4, 6.5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[6]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[7]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[8]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[9]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[10]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[11]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[12]	3GPP TS 38.355: "NR; Sidelink Positioning Protocol (SLPP); Protocol Specification".

[13]	3GPP TS 33.533: "Security aspects of ranging based services and Sidelink positioning".

[14]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[X]	3GPP TS 24.080: "Supplementary services specification; Formats and coding".

[Y]	3GPP TS 24.514: "Ranging based services and sidelink positioning in 5G system (5GS); Stage 3".

# * * * * Next change * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Located UE: A SL Reference UE of which the location is known or is able to be known using Uu based positioning. A Located UE can be used to determine the location of a Target UE using Sidelink Positioning.

Network-based Operation: Operation of Ranging/Sidelink Positioning with the involvement of 5GC NFs for the service request handling and result calculation.

Positioning: A functionality, which detects a geographical location and optionally, velocity (of e.g. a mobile terminal).

Ranging: Refers to the determination of the distance between two UEs or more UEs and/or the direction of one UE (i.e. Target UE) from another UE (i.e. SL Reference UE) via PC5 interface.

Ranging/SL Positioning Application Identifier: A globally unique identifier identifying a specific Ranging/SL Positioning application, which can be mapped to a V2X service type or a ProSe identifier.

Relative position: An estimate of the UE position relative to other network elements or relative to other UEs.

Relative velocity: An estimate of the UE velocity relative to another UE.

SL Reference UE: A UE, supporting positioning of target UE, e.g. by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc. using Sidelink.

NOTE 1:	SL Reference UE is understood as "Anchor UE" in RAN WGs.

Sidelink Positioning: Positioning UE using PC5 to obtain absolute position, relative position, or ranging information.

SL Positioning Client UE: A third-party UE, other than SL Reference UE and Target UE, which initiates Ranging/Sidelink positioning service request on behalf of the application residing on it.

NOTE 2:	The SL Positioning Client UE does not have to support Ranging/Sidelink positioning capability, but a communication between the SL Positioning Client UE and SL Reference UE/Target UE has to be established, either via PC5 or via 5GC, for the transmission of the service request and the result.

SL Positioning Server UE: A UE offering method determination, assistant data distribution, location calculation, SL-PRS configuration coordination and/or Located UE selection functionalities for Sidelink Positioning and Ranging based service. It interacts with other UEs over PC5 as necessary in order to determine Ranging/SL Position method, distribute assistant data and calculate the location of the Target UE. Target UE or SL Reference UE can act as SL Positioning Server UE if any of the functionalities is supported.

Target UE: A UE whose distance, direction and/or position is measured with the support from one or multiple SL Reference UEs using Sidelink in the Ranging based service and Sidelink positioning.

UE-only Operation: Operation of Ranging/Sidelink Positioning in which the service request handling and result calculation are performed by UE.

NOTE 3:	For UE-only Operation, the communication among UEs are over PC5.

User Info ID: The User Info ID is configured for Ranging/SL Positioning UE Discovery based on the policy of the HPLMN or via the Ranging/SL Positioning application server that allocates it. The definition of values of User Info ID is out of scope of this specification.

Application Layer ID: An identifier identifying a Ranging/Sidelink Positioning-enabled UE within the context of a specific application. The format of this identifier is outside the scope of 3GPP.

# * * * * Next change * * * *

### 5.3.1	General

Any UE supporting Ranging/SL Positioning, e.g. Target UE, SL Reference UE, Sidelink Positioning Server UE, Located UE, shall have a Ranging/SL Positioning layer, which is above the AS layer and handles service requests received from application layer to control the Ranging/SL Positioning operation. The Ranging/SL Positioning layer provides the support of authorization and provisioning as described in clause 5.1, the UE discovery and selection as described in clause 5.2, the Ranging/SL Positioning Protocol (RSPP), including protocols between UEs and the protocols between UE and LMF for Ranging/SL Positioning. Ranging/SL Positioning layer supports the Ranging/SL Positioning service sessions for one time or periodic ranging, ranging for distance or direction measurement or both. The Ranging/SL Positioning layer provides the ranging parameters to AS layer, including one time or periodic ranging, ranging for distance or direction measurement or both.

NOTE 1:	The Ranging/SL Positioning Protocol (RSPP) mention above includes both Sidelink Positioning Protocol (SLPP) defined in TS 38.355 [12], Supplementary Service messages defined in TS 24.080 [X] and Supplementary RSPP signalling messages defined in TS 24.514 [Y] .

The RSPP is exchanged over SR5 over PC5-U reference point between UEs (i.e. Target UE, SL Reference UE, Located UE and SL Positioning Server UE) to manage the Ranging/SL Positioning service sessions between UEs or among a group of UEs and between UE and LMF to manage the Ranging/SL Positioning service sessions between UE and LMF. Multiple UEs may be involved in a single Ranging/Sidelink Positioning session. When the Ranging/SL Positioning service session is among a group of UEs, the RSPP may provide group related identifier information for the PC5 transport layer handling. The transport of the RSPP over PC5-U is specified in clause 5.3.2. The transport of the RSPP between UE and LMF is specified in clause 5.3.3.

NOTE 2:	Group management is out-of-scope of this specification.

NOTE 3:		RSPP signalling negotiation determines whether measurement results or Ranging/Sidelink Positioning results are exchanged over the SR5.

NOTE 4:	The Ranging/Sidelink positioning supports the scheduled location time feature.

A LMF shall be involved when at least one of the Target UE and the SL Reference UE are in the network coverage and the serving network is capable to support ranging functionalities. The details of Network based Ranging/SL Positioning operation, and Network assisted SL Positioning are described in detail in clause 5.5. When Network assisted Ranging/SL Positioning is used, the use of RSPP is necessary for Ranging/SL Positioning control.

A SL Positioning Server UE can be discovered and selected for result calculation, method determination, assistant data distribution and SL Reference UE selection in case of out-of-coverage or for UE-only Operation if the serving network does not support Ranging/SL Positioning. If the LMF capable for Ranging/SL Positioning is reachable by Target UE and/or SL Reference UE, the LMF can still decide that a SL Positioning Server UE, Target UE or SL Reference UE executes the result calculation. A SL Positioning Server UE can be co-located with a Target UE or SL Reference UE.

# * * * * Next change * * * *

### 5.5.2	Network based SL Positioning for UE with NAS connection

A UE with a NAS connection is in RM-REGISTERED state. The UE can enter CM-Connected state by performing UE triggered Service Request for 5GC-MO-LR or SL-MO-LR, performing Network triggered Service Request for 5GC-NI-LR, 5GC-MT-LR or SL-MT-LR. As the Target UE can establish a NAS signalling connection with the AMF, the functionality specified in TS 23.273 [8] for location services can be reused including e.g. 5GC-MO-LR, 5GC-MT-LR, 5GC-NI-LR, SL-MT-LR and SL-MO-LR with the additional functionality captured in this clause.

NOTE 1:	Target UE can establish a NAS signalling connection directly or indirectly via a ProSe L2 UE-to-Network Relay.

-	The Location Service is triggered via the AMF serving the Target UE. The location request comes either from an AF/external client via the GMLC, a 5G NF, or the Target UE.

-	The LTE positioning protocol (LPP) as specified in TS 37.355 [9] is used between the Target UE and the LMF. When LCS procedures are used to estimate the location of the Located UE, LPP is used between Located UE and LMF.

-	The LMF determines if UE Positioning using SL positioning will be applied, when the 5GC-MO-LR, 5GC-MT-LR or SL-MT-LR service is triggered. When LMF determines that UE Positioning using SL positioning is used, LMF may trigger the Target UE to perform the discovery of Located UE(s).

-	The Target UE can determine if UE Positioning using SL positioning will be applied, when it is determined, the Target UE triggers the SL-MO-LR as described in clause 6.20.1 of TS 23.273 [8].

-	The Target UE discovers Located UE(s) for UE Positioning using SL positioning. Optionally, to assist the Target UE the LMF may provide to the Target UE a list of candidate Located UE(s). In this case the LMF maintains the candidate list of Located UE(s) including e.g. the capability and location information of the Located UE(s). If the Target UE selects the Located UE(s), the Target UE reports the UE identity i.e. Application Layer ID of the selected Located UE(s) to LMF, otherwise it reports the UE identity of all discovered Located UE(s) to the LMF.

-	The Target UE and Located UE(s) perform Ranging/SL positioning. The Target UE includes the UE identity i.e. Application Layer ID of the Located UE(s) to the LMF together with the Ranging measurement data or estimation result. The LMF may interact with GMLC to get the location of Located UE.

Editor's note:	The security impact will be coordinated with SA WG3.

-	The LMF uses the location of Located UE(s) together with the Ranging/SL positioning measurement data or estimation results reported by Target UE and optionally also by Located UEs to estimate the location of the Target UE.

-	LMF may either determine the absolute location of the Target UE in geographic coordinate or local coordinate expressed by a coordinate ID. For the latter case, Coordinate ID may be provided by LCS client or AF and used for Located UE selection by the Target UE. Coordinate ID should be considered for Located UE selection if Coordinate ID is received from LMF to determine the Target UE location in local coordinates.

-	In case the location of Located UE(s) is not known, the LMF interacts with GMLC a required QoS to get the location of Located UE. The Ranging/SL positioning and the positioning of the Located UE(s) can be scheduled with the same time using the scheduled location time, as specified in TS 23.273 [8], to improve the Target UE positioning accuracy.

NOTE 2:	The LMF may need to compensate the time difference of the positioning of Target UE and Located UE with additional information, e.g. velocity.

-	The LMF provides the UE location estimate to the Target UE over LPP, or to the AMF, and AMF forwards the UE location to the GMLC, or to the 5GC NF according to legacy functionality.

# * * * * Next change * * * *

### 5.5.4	Network-assisted SL Positioning for UE with NAS connection

For Network-assisted SL Positioning for UE with NAS connection, the procedure is similar to that of the Network based SL Positioning for UE with NAS connection (in clause 5.5.2), with the following exceptions:

-	Target UE performs result calculation.

-	The LMF may provide assistance information, i.e. absolute location of the Located UE, in the SLPP message to the Target UEto assist the Sidelink Positioning result calculation.

# * * * * Next change * * * *

#### 6.5.3.1	MO-LR Procedure for Network-assisted SL Positioning for UE with NAS connection

The procedures defined in this clause apply to Network-assisted SL Positioning for UE with NAS connection, as defined in clause 5.5.4, when the service request is initiated by the Target UE.

If the Located UE(s) is known to the Target UE (e.g. based on Located UE discovery and selection), SL-MO-LR procedure defined in clause 6.20.1 of TS 23.273 [8] is performed with the following adaptations:

-	UE1 is the Target UE, and UE2/.../UEn is the Located UE. UE1 provides application layer ID of UE2/.../UEn as the Located UE ID in the SL-MO-LR request.

-	It is indicated in step 8 that absolute location of UE1 is required.

-	It is indicated in step 15 that Network-assisted SL Positioning is applied.

-	In step 19, the response includes absolute location of UE1.

-	Step 20 and step 23 are skipped.

If the Located UE(s) is not known to the Target UE, 5GC-MO-LR procedure using SL positioning defined in clause 6.20.2 of TS 23.273 [8] is performed.

# * * * * End of changes * * * *