SA WG2 Meeting S2#160-Ad Hoc-e	S2-2401276r01

22 - 29 January, 2024, Online	revision of S2-24xxxxx

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#2: New solution on IMS capability exposure via DC3/DC4

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2

Abstract of the contribution: This contribution proposes a new solution to KI#2.

# Introduction

This contribution proposes a solution to KI#2 on “Impact on IMS architecture, interfaces and procedures to support IMS capability exposure in the context of IMS data channel session” including architectural aspects.

# 2	Proposal

It is proposed to add following text to TR 23.700-77.

# * * * * First change* * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | X |  |  |  |  |  |  |  |
| Y | X | X |  |  |  |  |  |  |

# * * * * Next change* * * *

## 6.X	Solution #Y: Solution for IMS capability exposure via DC3/DC4

### 6.X.1	Description

The architecture to enable IMS capability exposure in the context of IMS data channel (DC) sessions is based on Figure AC.2.1-1 in TS 23.228 [5], clause AC.2.1 and is depicted in Figure 6.X.1-1.

Figure 6.X.1-1: IMS capability exposure architecture

This solution defines the exposure of IMS capabilities related to DC session via DC3/DC4.

### 6.X.1.1	IMS capabilities to be exposed

With this solution, there are two types of capabilities, which can be exposed via DC3/DC4 interfaces (see Figure 6.X.1-1) from DCSF to DCAS via Ndcsf services.

The first type of capabilities are call related events from IMS AS via DC1 interface, for example:

-	establishing, updating, answering, terminating of IMS calls, and

-	subscription and notification to call related events (e.g., UE has established or terminated a call).

The second type of capabilities are DC related events, for example:

-	establishing, terminating of (P2A/A2P, P2A2P, P2P) application data channels,

-	downloading of an application via an existing bootstrap data channel, and

-	subscription and notification to DC related events (e.g., UE has established an application DC or downloaded a certain application via the bootstrap DC).

Editor's note:	Need for additional capabilities is FFS.

One Ndcsf service can provide event subscription/notification capabilities (e.g., Ndcsf_dc_event_exposure) while another one exposes capabilities related to DC operation (e.g., Ndcsf_dc_operation) including establishing, termination, update of application data channels.

Editor's note:	Exact definition of the Ndcsf services is FFS.

### 6.X.1.2	Information to invoke the exposure service

DCAS provides following information to invoke a service on DC3/DC4:

-	There are two types of actions which can be provided:

1.	First type of action is related to DC establishment, DC update, and DC termination. Following information can be provided:

-	Type of DC, which can be P2A/A2P, P2A2P, and P2P,

-	DC application binding information,

-	User ID which includes Calling ID and/or Called ID, and

-	User ID which the DC application apply to, and

-	Session ID which the DC application apply to, and

-	External DC identifier to determine a DC that is established, updated, or terminated; and

2.	Second type of action is related to DC event subscription and notification. Following information can be provided:

-	subscription to event for specific application DC establishment/termination,

-	subscription to event for downloading of a specific DC application, and

-	subscription to IMS capability exposure.

-	Event can be application DC is established or terminated, specific DC application was downloaded via bootstrap DC.

-	Unique external DC identifier to determine a DC that is established, updated, or terminated.

Editor's note:	The concrete list of information provided from DCAS to DCSF is FFS.

Editor's note:	Whether DCAS can request establishing, update, termination of bootstrap data channel is FFS.

### 6.X.1.3	Information to be exposed

One Ndcsf service can provide session control capabilities (e.g., Ndcsf_dc_session_control). This service allows DCAS to create DC session and take control on DC session via DCSF.

DCSF can return to DCAS the following information on DC3/DC4:

-	response code (success or failure),

-	other information, e.g., session identifier (session ID), IP address of the UE, location, MDC2 identifier, user identifier (user ID), PVNI, and external DC identifier, Call ID which can be used to determine a DC uniquely, and

-	notifications to the reported events (DC establishment/termination, downloaded DC application, IMS capability exposure).

-	session control status (DC session creation result, DC termination).

Editor's note:	The concrete information to be exposed is FFS.

### 6.X.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

### 6.X.2.1	IMS Capability Exposure

The event subscription and event exposure can be realized in two different ways.

Figure 6.X.2.1-1 depicts the call flow of the first approach, that both call and DC related events are subscribed and exposed via the same service.

Figure 6.X.2.1-1: Approach 1 Procedure of call and DC related event subscription and exposure via the same service

The steps in the call flow are as follows:

1.	DC Application Server subscribes for events (through NEF) via DC3/DC4 to DCSF. As described in clause 6.X.1.1, there are two types of capabilities, which can be subscribed to and exposed. Specifically, for call related events, the subscription request is forwarded to IMS AS via DC1 from DCSF. DC Application Server can provide the information listed in clause 6.X.1.2 to invoke the exposure service.

Editor's note:	How DCSF and IMS AS are selected in step 1 is FFS.

2.	DCSF (and IMS AS) respond(s) to DC Application Server via DC3/DC4 (and DC1), whether the event subscription is successful or not.

3.	The subscribed event(s) are occurred.

4.	Information about the subscribed events (listed in Clause 6.X.1.3) is exposed to DC Application Server (through NEF) via DC3/DC4 from DCSF. Specifically, for call related events, IMS AS exposes the information to DCSF via DC1, and then DCSF exposes the information to DC Application Server (through NEF) via DC3/DC4.

5.	DC Application Server respond(s) to DCSF (and IMS AS) via DC3/DC4 (and DC1).

Figure 6.X.2.1-2 depicts the call flow of the first approach, that the call and DC related events are subscribed and exposed via two different services. The call related events are subscribed by DC Application Server at IMS AS, and the DC related events are subscribed by DC Application Server at DCSF. Based on the event type (e.g., call related event or DC related event), IMS AS or DCSF is contacted.

Figure 6.X.2.1-2: Approach 2 Procedure of call and DC related event subscription and exposure via the separated service

The steps in the call flow are as follows:

1.	DC Application Server subscribes events for call related event exposure (through NEF) to IMS AS. The call related events are described in clause 6.X.1.1. DC Application Server can provide the information listed in clause 6.X.1.2 to invoke the exposure service. It is assumed that IMS AS has been registered at HSS, and NEF has fetched the IMS AS information from HSS.

Editor's note:	Which IMS AS is selected in step 1 is FFS.

2.	IMS AS responds to DC Application Server (through NEF), whether the call related event subscription is successful or not.

3.	DC Application Server subscribes events for DC related event exposure (through NEF) via DC3/DC4 to DCSF. The DC related events are described in Clause 6.X.1.1. DC Application Server can provide the information listed in Clause 6.X.1.2 to invoke the exposure service.

Editor's note:	Which DCSF to be subscribed in step 3 is FFS.

4.	DCSF responds to DC Application Server (through NEF) via DC3/DC4, whether the DC related event subscription is successful or not.

5.	The subscribed event(s) are occurred.

6.	Information about the subscribed call related events (listed in Clause 6.X.1.3) is notified to DC Application Server (through NEF) from IMS AS.

7.	DC Application Server responds to IMS AS (through NEF).

8.	Information about the subscribed call related events (listed in Clause 6.X.1.3) is notified to DC Application Server (through NEF) via DC3/DC4 from DCSF.

7.	DC Application Server responds to DCSF (through NEF) via DC3/DC4.

### 6.X.2.2	Person-to-Person (P2P) Application Data Channel Setup

Based on Figure AC.7.2.1-1 in TS 23.228 [5], clause AC.7.2.1, Figure 6.X.2.2-1 depicts a call flow for establishing an Application Data Channel triggered by IMS capability exposure in a person to person use case.

Figure 6.X.2.2-1: Person-to-Person (P2P) application data channel set up triggered by IMS capability exposure signalling procedure

The steps in the call flow are as follows:

1.	Event subscribed and exposed following the procedure described in clause 6.X.2.1. The event can be, e.g., an application has been downloaded to UE#1.

2.	DC Application Server requests IMS AS to establish a data channel with session ID, user IDs, binding information, through (NEF and) DCSF.

3.	IMS AS determines whether the media change request event should be notified to a DCSF and selects a DCSF.

4.	IMS AS notifies the DCSF of the media change request event.

5.	The DCSF determines the DC control policy.

6.	The DCSF responds to the notification received in Step 4.

7.	IM AS starts IMS DC negotiation with UE#1 and UE#2.

8.	IMS IM AS sends a SIP reINVITE request to I/S-CSCF.

9.	I/S-CSCF sends the SIP reINVITE request to Terminating Network/UE#2.

10.	Terminating Network/UE#2 responds a SIP 200 OK with audio/video SDP offer to I/S-CSCF.

11.	I/S-CSCF sends the SIP 200 OK with audio/video SDP offer to IMS AS.

12.	IMS AS sends a SIP reINVITE request with audio/video, application DC SDP offer to UE#1, through I/S-CSCF and P-CSCF.

13.	UE#1 responds a SIP 200 OK with audio/video, application DC SDP offer to IMS AS, through P-CSCF and I/S-CSCF.

14.5IMS AS notifies DCSF of the successful media change, with session ID and a list of media information.

15.	DCSF responds IMS AS.

16.	IMS AS sends a SIP ACK with audio/video SDP answer to I/S-CSCF.

17.	I/S-CSCF sends the SIP ACK with audio/video SDP answer to Terminating Network/UE#2.

18.	IMS AS returns a message to DC Application Server, through DCSF (and NEF), to notify if the application DC establishment is successful or failed. If the establishment is failed, Step 24 shall be skipped.

19.	The application data channel between the UE#1 and the UE#2 is established. In this example, it is not anchored in MF/MRF.

### 6.X.2.3	Person-to-Application (P2A) Application Data Channel Setup

Based on Figure AC.7.2.2-1 of TS 23.228 [5], clause AC.7.2.2, Figure 6.X.2.3-1 depicts a call flow for establishing an Application Data Channel in a person to application use case. Note that there are two options to reserve the MDC2 media resources. The first option is that the media resources are reserved by DC Application Server before the application DC establishment request. The second option is that the media resource reservation is requested by IMS AS and resources are reserved by MF/MRF after the application DC establishment request.

Figure 6.X.2.3-1: Person-to-Application (P2A) application data channel set up signalling procedure

The steps in the call flow are as follows:

1.	Event subscribed and exposed following the procedure described in clause 6.X.2.1. The event can be, e.g., an application has been downloaded to UE#1.

2.	Optionally, DC Application Server can reserve the media resource, before it starts the request of data channel establishment.

3.	If the media resource is not reserved in Step 2, the DC Application Server requests IMS AS to establish an application data channel with session ID, user IDs, and binding information, through (NEF and) DCSF. If the media resource is reserved in Step 2, together with the request, the reserved media resource shaw also be sent, so that DCSF is informed about the reserved media resource. The SDP offer to UE#1 is also provided.

4.	IMS AS determines whether the media change request event should be notified to a DCSF and selects a DCSF.

5.	IMS AS notifies the DCSF of the media change request event.

6.	The DCSF determines the DC control policy.

7.	The DCSF responds to the notification received in Step 5.

8.	If the media resource is not reserved in Step 2, IMS AS requests MF/MRF to reserve media resource for the originating side and the MDC2, for DC Application Server use. If the media resource is reserved in Step 2, the media resource reservation for the MDC2 in this step shaw be skipped.

9.	After the media resource is reserved, IMS AS responds DCSF with the information of the reserved media resource.

10.	If the media resource is not reserved in Step 2, in this step, DCSF sends the MDC2 SDP offer received from MF/MRF to the DC Application Server via DC3/DC4. If the media resource is reserved in Step 2, this step shaw be skipped.

11.	If the media resource is not reserved in Step 2, in this step, DC Application Server returns an MDC2 SDP answer and is prepared for UE#1 traffic through MDC2. If the media resource is reserved in Step 2, this step shaw be skipped.

12.	DCSF requests IMS AS to update the MF resource with MDC2 media endpoint information of DC Application Server.

13.	IM AS updates the MF/MRF resource.

14.	IM AS responds DCSF with the updated information of the reserved media resource.

15.	DCSF responds IMS AS.

16.	IMS AS starts IMS DC negotiation with UE#1 and UE#2.

17.	IMS AS sends a SIP reINVITE request to I/S-CSCF for media re-negotiation between UE#1 and UE#2.

18.	I/S-CSCF sends the SIP reINVITE request to Terminating Network/UE#2.

19.	Terminating Network/UE#2 responds a SIP 200 OK with audio/video SDP offer to I/S-CSCF.

20.	I/S-CSCF sends the SIP 200 OK with audio/video SDP offer to IMS AS.

21.	IMS AS sends a SIP reINVITE request with audio/video, application DC SDP offer to UE#1, through I/S-CSCF and P-CSCF.

22.	UE#1 responds a SIP 200 OK with audio/video, application DC SDP answer to IMS AS, through P-CSCF and I/S-CSCF.

23.	IMS AS notifies DCSF of the successful media change, with session ID and a list of media information.

24.	DCSF responds IMS AS.

25.	IMS AS sends a SIP ACK with audio/video SDP answer to I/S-CSCF.

26.	I/S-CSCF sends the SIP ACK with audio/video SDP answer to Terminating Network/UE#2.

27.	IMS AS returns a message to DC Application Server, through DCSF (and NEF), to notify if the application DC establishment is successful or failed. If the establishment is failed, Step 24 shall be skipped.

28.	The application data channel between the UE#1 and DC Application Server is established via MF/MRF. MF/MRF forwards data channel traffic between the UE#1 and DC Application Server based on MDC2 media point information received in step 8.

### 6.X.3	Impacts on existing nodes and functionality

Impacted nodes and functionality are as follows:

-	DC Signalling Function (DCSF): The DCSF interacts with the DCAS for exposes DC resource and session control related capabilities to IMS capability exposure via DC3/DC4. The DCSF sends call related event subscription to IMS AS via DC1. The DCSF receives call related event exposure from IMS AS via DC1.

-	Network Exposure Function (NEF): It shall securely expose IMS capability and events provided by DCSF to DCAS exposure and exposes new services to external AF.

-	DC Application Server (DCAS): An application server that interacts with the DCSF (through NEF) for IMS capability exposure using DC3/DC4 interfaces. DCAS acts as AF.

-	IMS Application Server (IMS AS): The IMS AS interacts with the DCSF via DC1 for event subscription and exposure.

# * * * * End of changes * * * *