3GPP TSG-SA WG2#160-Ad Hoc-e	S2-2401288r11

Electronic Meeting, 22-29 January 2024	(was S2-240xxxx)

Source:	Qualcomm Incorporated, Apple, Charter Communications, Nokia, Nokia Shanghai Bell, Ericsson

Title:	WT3: Assumptions and requirements

Document for:	Discussion/Approval

Agenda Item:	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This contribution proposes assumptions and requirements related to WTs 3, 3.1 and 3.2 in the FS_MASSS TR

1.	Discussion

This contribution proposes assumptions and requirements related to WTs 2, 3, 3.1 and 3.2 in the FS_MASSS TR.

2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-54:

# >>>>BEGINNING OF CHANGES<<<<

4.1.2	Architectural Assumptions for ATSSS_Ph4

The following general assumptions apply:

-	The Rel-18 ATSSS as specified in TS 23.501 [a], TS 23.502 [b], and TS 23.503 [c] are regarded as the baseline for this study.

The following assumptions apply to alternative architecture and system level enhancements for ATSSS that do not require the existing non-3GPP interworking/gateway function:

-	Current Non-3GPP InterWorking Function (N3IWF) and Trusted Non-3GPP Gateway Function (TNGF) are not used to simplify the operation over non-3GPP access.

- 	Any new ATSSS capabilities specified for UE would be applicable for ATSSS-capable 5G-RGs, if endorsed by BFF and/or CableLabs; FN-RG are not in the scope of the study.

The following assumptions apply to MPQUIC for non-UDP traffic:

-	Any new steering functionality shall be based on the multipath extensions of the QUIC protocols as defined in IETF.

- 	Any new ATSSS capabilities specified for UE would be applicable for ATSSS-capable 5G-RGs, if endorsed by BFF and/or CableLabs; FN-RG are not in the scope of the study.

4.2.2	Architectural Requirements for ATSSS_Ph4

-	New ATSSS capabilities shall either be able to co-exist with the existing ATSSS capabilities or any dependencies between new and existing capabilities shall be explicitly defined.

-	The benefit and drawback of new solutions compared to existing TNGF and N3IWF shall be analyzed before any conclusion on WT#3.

NOTE:	Coordination with BBF and CableLabs will take place as needed during the study.

# >>>>END OF CHANGES<<<<