3GPP TSG-WG SA2 Meeting #160AHE 	S2-2401304

# 21 – 29 Jan., 2024, Elbonia     		     	  (revision of S2-220xxxx)

Source:	Google

Title:	[KI#1] Solution X: Satellite Access Control

Document for:	Approval

Agenda Item:	19.3.2

Work Item / Release:	5GSAT_Ph3 / Rel-19

Abstract: This paper proposes a new solution for KI#1 to support Satellite Access control mechanism for UEs accessing Satellite with gNB on board.

# 1. Discussion

This PCR is proposed a solution for KI#1, Support of Regenerative-based satellite access, to enhancement access control for UEs using Satellite RAT.

Motivation of this solution:

When considering RAN (gNB/eNB) boarding on the NGSO satellite, the nature of the communication characteristics between the UE on the ground and the gNB/eNB boarding on the Satellite in the sky would result in some limitations to the applicable services:

- In view of required QoS provisioning, long latency and varied jitter would impose restrictions on accommodating UEs using Satellite access for delay sensitive services, e.g. streaming video.

Observation#1: UEs using satellite access would not be able to get required QoS of some services.

- For NTN, a satellite in geosynchronous orbit (GEO) may have a beam footprint diameter ranging from 100 Km to 3500 Km. For low-earth orbit satellite systems (LEO), the beam footprint diameter can range from 50 m to 1000 km (Ref: 3GPP TR 38.821). In contrast, TN typically have a coverage diameter of 0.5-10 Km. Because of its large footprint, a satellite beam/cell serves more users than a TN cell. The altitude at which satellites operate can range from approximately 36000 Kms for GEOs to 600 Kms for LEOs. Since these distances are several orders of magnitude larger than comparable distances for TN, link capacity for NTN can be relatively low, compared to TN.

Observation#2: NTN with gNB/eNB boarding on the satellite would experience more congestion than TN with gNB/eNB on the ground because a NTN cell typically serves more users and has lower capacity than a TN cell.

- Currently, in TS23.501 [5] clause 5.4.11.8, the access control mechanism is based on mobility Forbidden area information and service area restrictions mechanism. The AMF provides the UE with mobility Forbidden Area information defined by TAI(s) or Service Area Restrictions which consist of either Allowed Areas or Non-Allowed Areas. If the UE detects that the gNB broadcasts SIB indicating TAI(s) which is within its configured mobility Forbidden Area or Service area Restrictions, the UE does not use the Satellite access.

Observation#3: the restriction is too extreme to accommodate the traffic that may be suitable for Satellite access. As such, more sophisticated access control mechanisms are needed.

Proposal: This PCR proposes a solution to enhance the access control mechanism by enabling network deployments to control which services/traffic flows (e.g., applications and/or type of application) can get access to the 5GS when a UE using Satellite access to RAN boarding on the NGSO satellite.

# 2. Proposal

It is proposed to include this solution in TR 23.700-29.

# * * * * First change  * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | KI#1 | KI#2 | KI#3 |  |
| Solution X: Satellite Access Control for applicable services | X |  |  |  |

# * * * * Second change (All new) * * * *

## 6.X	Solution #X: Satellite Access Control for applicable services

### 6.X.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). (Sub) clause(s) may be added to capture details.

This solution proposes to enhance satellite access control for KI#1: Support of Regenerative-based satellite access.

When considering RAN boarding on NGSO satellite, the nature of the communication characteristics between the UEs on the ground and the RAN boarding on the Satellite in the sky would result in some limitations to the applicable services.

- In view of required QoS provisioning, long latency and varied jitter would impose restrictions on accommodating UEs using Satellite access for delay sensitive services, e.g. streaming video.

- NTN with RANs boarding on NGSO satellites would experience more congestion than RANs in TN on the ground because a satellite cell typically serves more users in broader coverage and has lower capacity than a cell in a TN.

- Currently, in TS23.501 [5] clause 5.4.11.8, the access control mechanism is based on mobility Forbidden area information and service area restrictions. If the UE detects that the RAN broadcasts SIB indicating TAI(s) which is within its configured mobility Forbidden Area or Service area Restrictions, the UE does not use the Satellite access. The restrictions are too extreme to accommodate the traffics that may be suitable for Satellite access.

This solution aims at enabling Satellite access control for allowing 5G network for controlling access for a particular services/traffic flows (e.g., applications and/or type of application) when a UE using Satellite access to RAN boarding on NGSO satellite. The principles of the solution are as follows:

- The network operator can provision URSP rules to UE with a new Route Selection Descriptor (RSD) component to indicate 3GPP access technology restriction or 3GPP Access technology preference.

  - The 3GPP access technology preference indicates the Preferred 3GPP access technology when the UE establishes or select a PDU Session for the matching TD of the URSP rule.

  - The 3GPP access technology restriction indicates the disallowed RAT Type when the UE establishes or select a PDU Session for the matching TD of the URSP rule.

- For example, when the UE using a Satellite access for an application associated with a particular Connection Capabilities determines a matched TD of an URSP rule, the RSD of the URSP indicates to the UE whether routing the traffic to the PDU session if a specific 3GPP access technology is preferred or restricted by the operator.

The implementation for the enhancement of the RSD for an URSP rule is as shown below:

Table 6.X-1: Route Selection Descriptor

| Information name | Description | Category | PCF permitted to modify in URSP | Scope |
| --- | --- | --- | --- | --- |
| Route Selection Descriptor Precedence | Determines the order in which the Route Selection Descriptors are to be applied. | Mandatory (NOTE 1) | Yes | UE context |
| Route selection components | This part defines the route selection components | Mandatory (NOTE 2) |  |  |
| SSC Mode Selection | One single value of SSC mode. (NOTE 5) | Optional | Yes | UE context |
| Network Slice Selection | Either a single value or a list of values of S-NSSAI(s). | Optional (NOTE 3) | Yes | UE context |
| DNN Selection | Either a single value or a list of values of DNN(s). | Optional | Yes | UE context |
| PDU Session Type Selection | One single value of PDU Session Type | Conditional (NOTE 8) | Yes | UE context |
| Non-Seamless Offload indication | Indicates if the traffic of the matching application is to be offloaded to non-3GPP access outside of a PDU Session. | Optional (NOTE 4) (NOTE 10) | Yes | UE context |
| ProSe Layer-3 UE-to-Network Relay Offload indication | Indicates if the traffic of the matching application is to be sent via a ProSe Layer-3 UE-to-Network Relay outside of a PDU session. | Optional (NOTE 4) (NOTE 10) | Yes | UE context |
| ProSe Multi-path Preference | Indicates if the traffic of the matching application is preferred to be sent via a PDU Session over the Uu reference point and a ProSe Layer-3 UE-to-Network Relay outside of a PDU session. | Optional (NOTE 9) (NOTE 10) | Yes | UE context |
| Access Type preference | Indicates the preferred Access Type (3GPP or non-3GPP or Multi-Access) when the UE establishes a PDU Session for the matching application. | Optional | Yes | UE context |
| [new] 3GPP Access preference | [new] Indicates the preferred 3GPP access (LTE or NR or Satellite Access) when the UE establishes/selects a PDU Session for the matching application. | Optional | Yes | UE context |
| PDU Session Pair ID | An indication shared by redundant PDU Sessions as described in clause 5.33.2.1 of TS 23.501 [2]. | Optional (NOTE 10) | Yes | UE context |
| RSN | The RSN as described in clause 5.33.2.1 of TS 23.501 [2]. | Optional (NOTE 10) | Yes | UE context |
| Route Selection Validation Criteria | This part defines the Route Validation Criteria components | Optional |  |  |
| Time Window | The time window when the matching traffic is allowed. The RSD is not considered to be valid if the current time is not in the time window. | Optional | Yes | UE context |
| Location Criteria | The UE location where the matching traffic is allowed. The RSD rule is not considered to be valid if the UE location does not match the location criteria. | Optional | Yes | UE context |

### 6.X.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

No changes are required for the related URSP provisioning procedures.

### 6.X.3	Impacts to Services, Entities and Interfaces

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

PCF: configures the URSP rule that includes a new RSD component.

UE: considers 3GPP access technology preference or restriction when enforcing the URSP rule.

# * * * *End change * * * *