SA WG2 Meeting #160-AH-e	S2-2401319r01

January 22 - 29, 2024, Electronic Meeting

Source:	Apple, ETRI

Title:	FS_MASSS Architectural Terms

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS /Rel-19

Abstract of the contribution: This paper proposes Terms for DualSteer for the FS_MASSS TR 23.700-54.

1	Discussion

This paper proposes text for the terms section of the FS_MASSS TR 23.700-54.

2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.0.0.

* * * * Start of Changes * * * *

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

DualSteer device: A device supporting traffic steering and switching of user data (for different services) across two 3GPP access networks; it can be a single UE, in case of non-simultaneous data transmission over the two networks, or two separate UEs, possibly based on a Multi-USIM UE in case of simultaneous data transmission over the two networks.

DualSteer Traffic steering: the procedure that selects an access network for a new service and transfers the service over the selected access network. This may include scenarios where all services use the same network connection (no simultaneous transmission over the two access networks, for DualSteer devices with single UE) or different services are steered across different access networks (with simultaneous transmission over the two networks, for DualSteer devices with two separate UEs only).

DualSteer Traffic switching: the procedure that moves all traffic of a service from one access network to another access network in a way that minimizes service interruption. This may include scenarios where all services are switched from one access network to another access network (no simultaneous transmission over the two access networks, for DualSteer devices with single UE) or sessions subject to traffic switching are switched from one access network to another access network (with simultaneous transmission over the two access networks, for DualSteer devices with two separate UEs only).

* * * * End of Changes * * * *