SA WG2 Meeting #160-AH-e	S2-2401320r16

January 22 - 29, 2024, Electronic Meeting

Source:	Apple, ETRI

Title:	FS_MASSS Architectural Assumptions and Requirements for DualSteer

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes architectural assumptions and requirements for DualSteer for the FS_MASSS TR 23.700-54.

1	Discussion

This paper proposes architectural assumptions and requirements for DualSteer based on content of the FS_MASSS SID (SP-231802) and the DualSteer requirements in TS 22.261.

The objectives of the FS_MASSS SID, SP-231802, include the following statements:

- The following scenarios are considered:

1.	Two NR/5GC accesses in a single PLMN (HPLMN or VPLMN) with each access being NR TN or NR NTN;

2.	Two NR/5GC accesses in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN) with each access being NR TN or NR NTN;

3.	NR/5GC access and E-UTRA/EPC access in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN);

4.	NR/5GC access and E-UTRA/EPC access in a single PLMN (HPLMN or VPLMN);

5.	PNI-NPN (integrated with the HPLMN or integrated with the VPLMN) and PLMN access (TN/NTN plus TN or NTN). This scenario assumes only non-simultaneous transmission.

- The study assumes there is no coordination in RAN between the two 3GPP access networks where the DualSteer Device is accessing simultaneously.

- For the PNI-NPN scenario, the subscriber is assumed to be a subscriber of the PNI-NPN.

- Each subscription / SUPI of the DualSteer Device is used to connect to only one of the 3GPP access at any given point in time.

- No impacts on network slicing features, i.e. network slicing features apply to each subscription / SUPI separately.

The Traffic steering and switching over two 3GPP access networks from TS 22.261, clause 6.50, include the following statements:

- Scenarios may also include traffic steering and/or switching across LTE/EPC and NR/5GC, with anchoring in 5GC.

- Traffic policies are intended to be in full control of the home network operator.

- The requirements can apply to different DualSteer device types (e.g., smartphones, IoT, UAV, VSAT devices).

- The requirements apply to a subscriber with two subscriptions/SUPIs, sharing one subscription profile from the same operator. For simultaneous transmission over two networks, a DualSteer device is assumed to include two separate UEs.

- Inter-PLMN requirements can apply also to PLMN-NPN scenarios assuming a PLMN-integrated NPN (NPN hosted by a PLMN or offered as a slice of a PLMN).

- For any particular service, at any given time, the DualSteer device shall transmit all traffic of that service using only a single 3GPP access network.

- Charging information should be collected for both 3GPP access networks; in case the two 3GPP access networks belong to different PLMNs, or a PLMN and NPN, a proper business/roaming agreement among network operators is assumed.

- The NPN is hosted by a PLMN or offered as a slice of a PLMN, data anchoring in the NPN, and a business/roaming agreement between the PLMN and the NPN operator (if different).

2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.0.0.

* * * * Start of Changes * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[a]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[b]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[c]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[d]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

* * * * Next Changes * * * *

4.1.1	Architectural Assumptions for DualSteer

-	The 5GC architecture as specified in TS 23.501 [a], TS 23.502 [b], and TS 23.503 [c] are regarded as the baseline for this study.

-	The subscriber is assumed to have two associated subscriptions/SUPIs from the same operator.

-	No coordination is assumed in RAN between the two 3GPP access networks.

-	Each subscription (i.e. SUPI) of the DualSteer device is used to connect to only one of the 3GPP access at any given point in time.

-	No impacts are assumed for network slicing features, i.e. network slicing features apply to each subscription / SUPI separately.

-	Solutions are expected to demonstrate not to impact PLMNs that do not support this functionality.

-	Traffic policies are controlled by the home network operator.

-	For sessions subject to traffic steering and/or switching of a DualSteer device's user data across two 3GPP access networks, it is assumed that the data anchoring is done at the HPLMN.

-    For any particular service, at any given time, it is assumed that the DualSteer device transmits all traffic of that service using only a single 3GPP access network.

-	No impacts on RAN and UICC apps.

-	Both DualSteer Device with single UE and DualSteer Device with separate UEs are allowed to register over two 3GPP accesses simultaneously.

* * * * Next Changes * * * *

4.2.1	Architectural Requirements for DualSteer

Editor's note:	This clause will document any DualSteer architectural requirements for the study.

Service Requirements for DualSteer are specified in clause 6.50 of TS 22.261 [d].

The following scenarios shall be supported:

1.	Two NR/5GC accesses in a single PLMN (HPLMN or VPLMN) with each access being NR TN or NR NTN;

2.	Two NR/5GC accesses in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN) with each access being NR TN or NR NTN;

3.	NR/5GC access and E-UTRA/EPC access in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN);

4.	NR/5GC access and E-UTRA/EPC access in a single PLMN (HPLMN or VPLMN);

5.	PNI-NPN (integrated with the HPLMN) and PLMN access (TN/NTN plus TN or NTN). This scenario assumes only non-simultaneous transmission.

The following architectural requirements are applicable to this study:

-	Scenario 5) requires the subscriber to be a subscriber of the PNI-NPN. In this scenario the PNI-NPN is hosted by a HPLMN. For sessions subject to traffic steering and/or switching, this scenario requires data anchoring in the PNI-NPN.

-

In case the two 3GPP access networks belong to different PLMNs, or a PLMN and PNI-NPN, a proper business/roaming agreement among network operators is required.

-	The 5GS shall be able to identify and link the two SUPIs associated with a DualSteer device, differentiate the two connections for the DualSteer device and minimize impacts to CN, O&M or IT systems.

-	The 5GS shall be able to support mechanisms to minimize service interruption when switching a DualSteer device's user data, for one or multiple services, between two 3GPP access networks.

-	For traffic steering and/or switching of user data across two 3GPP access networks, the 5GS may be able to support mechanisms to change one 3GPP access network to the non-3GPP access network of the same subscription (and vice versa).

* * * * End of Changes * * * *