SA WG2 Meeting #160-AH-e	S2-2401329r01

January 22 - 29, 2024, Electronic Meeting

Source:	Apple

Title:	FS_MASSS DualSteer Key Issue for Session management aspects

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes a new Key Issue to cover the Session management aspects of WT#1.3 for DualSteer for the FS_MASSS TR 23.700-54.

# 1	Discussion

This paper proposes a new Key Issue to cover the Session management aspects of WT#1.3 of the FS_MASSS SID (SP-231802).

# 2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.0.0.

* * * * Start of Changes * * * *

## 5.1	Key Issue for DualSteer

### 5.1.x	Key Issue #X: DualSteer – Session management aspects

#### 5.1.x.1 	Description

This key issue will study whether and how to enhance session management procedures for initial steering and potential subsequent switching:

- 	for a DualSteer device, how to support traffic steering and/or switching of user data for different services across two 3GPP access networks belonging to the same PLMN (either HPLMN or VPLMN) or two different PLMNs, assuming data anchoring in the HPLMN or, in case of PNI-NPN plus PLMN, assuming data anchoring in the PNI-NPN;

-

-	Whether and what enhancements are required in PDU Session establishment/modification/termination procedures to support DualSteer. UE initiated DualSteer session shall be studied. Network initiated DualSteer may be studied.

-	How the network identifies the PDU Sessions from the two SUPIs of a DualSteer device that requires DualSteer support.

-	Whether and how the network authorizes a UE's request to establish a DualSteer PDU Session.

-	How the network selects a common PDU Session anchor in the HPLMN (or in the PNI-NPN for the PLMN and PNI-NPN scenario) for the two PDU Sessions from the two SUPIs/UEs of a DualSteer device that require DualSteer support.

-	Whether and what enhancements are needed to assign a single IP address for the two PDU Sessions from the two SUPIs/UEs of a DualSteer device that require DualSteer support.

-	Whether and what enhancements are required for N4 session management between the SMF and the UPF to support the DualSteer PDU Sessions.

-	Whether and what enhancements are required in subscription data to support DualSteer PDU sessions.

- 	whether existing functionality is impacted related to the change of a service-related data between a 3GPP access network and a non-3GPP access network.

* * * * End of Changes * * * *