SA WG2 Meeting #160-AH-e	S2-2401329r06

January 22 - 29, 2024, Electronic Meeting

Source:	Apple, ETRI

Title:	FS_MASSS DualSteer Key Issue for Session management aspects

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes a new Key Issue to cover the Session management aspects of WT#1.3 for DualSteer for the FS_MASSS TR 23.700-54.

1	Discussion

This paper proposes a new Key Issue to cover the Session management aspects of WT#1.3 of the FS_MASSS SID (SP-231802).

2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.0.0.

* * * * Start of Changes * * * *

5.1	Key Issue for DualSteer

5.1.x	Key Issue #X: DualSteer – Session management aspects

5.1.x.1 	Description

This key issue will study whether and how to enhance session management procedures for initial steering and potential subsequent switching:

- 	For a DualSteer device, how to support traffic steering and/or switching of user data for different services across two 3GPP access networks belonging to the same PLMN (either HPLMN or VPLMN) or two different PLMNs;

-

Whether and what enhancements are required in PDU Session establishment/modification/termination procedures to support DualSteer services (i.e. steering and/or switching);

-	How the network correlates the two SUPIs of a DualSteer device that requires DualSteer support for session management;

-

-	For sessions subject to potential switching, how the network selects an anchor SMF or SMF+PGW-C in the HPLMN (or in the PNI-NPN for the PLMN plus PNI-NPN scenario) to allow routing the traffic across 3GPP accesses towards the same PSA UPF(s) or UPF+PGW-U(s) during the switching;

-	Whether and what enhancements are required for N4 session management between the SMF and the UPF to support DualSteer services;

-

* * * * End of Changes * * * *