SA WG2 Meeting #160-AH-e	S2-2401329r08

January 22 - 29, 2024, Electronic Meeting

Source:	Apple, ETRI

Title:	FS_MASSS DualSteer Key Issue for Session management aspects

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes a new Key Issue to cover the Session management aspects of WT#1.3 for DualSteer for the FS_MASSS TR 23.700-54.

# 1	Discussion

This paper proposes a new Key Issue to cover the Session management aspects of WT#1.3 of the FS_MASSS SID (SP-231802).

# 2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.0.0.

* * * * Start of Changes * * * *

## 5.1	Key Issue for DualSteer

### 5.1.x	Key Issue #X: DualSteer – Session management aspects

#### 5.1.x.1 	Description

This key issue will study whether and how to enhance session management procedures for initial steering and potential subsequent switching:

- 	For a DualSteer UE with two separate UE stacks, how to steer the user data for different services across two 3GPP access networks belonging to the same PLMN (either HPLMN or VPLMN) or two different PLMNs;

-	For sessions subject to potential switching, how the network selects an anchor SMF or SMF+PGW-C in the HPLMN (or in the PNI-NPN for the PLMN plus PNI-NPN scenario) to allow routing the traffic across 3GPP accesses towards the same PSA UPF(s) or UPF+PGW-U(s) during the switching;

To study the above aspects, following aspects may be further considered:

- 	Whether and what enhancements are required in PDU Session establishment/modification/release procedures;

-	Whether and what enhancements are required for N4 session management between the SMF and the UPF devices; and

Solution for this key issue shall take into account that a DualSteer UE with single UE stack is not allowed to perform simultaneous transmission over two 3GPP access networks. Splitting functionality is not supported for DualSteer in any scenario.

- 	Whether existing functionality is impacted related to the change of a service-related data between a 3GPP access network and a non-3GPP access network.

* * * * End of Changes * * * *