SA WG2 Meeting #160-AH-e	S2-2401329r11

January 22 - 29, 2024, Electronic Meeting

Source:	Apple, ETRI

Title:	FS_MASSS DualSteer Key Issue for Session management aspects

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes a new Key Issue to cover the Session management aspects of WT#1.3 for DualSteer for the FS_MASSS TR 23.700-54.

1	Discussion

This paper proposes a new Key Issue to cover the Session management aspects of WT#1.3 of the FS_MASSS SID (SP-231802).

2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.0.0.

* * * * Start of Changes * * * *

5.1	Key Issue for DualSteer

5.1.x	Key Issue #X: Session management aspects for DualSteer

5.1.x.1 	Description

This key issue will study the following potential session management enhancement:

-	Whether and how to enhance session management procedures for initial steering across two 3GPP access networks belonging to the same PLMN (either HPLMN or VPLMN) or two different PLMNs, and potential subsequent switching, which may further include following:

- 	Whether and what enhancements are required in PDU Session establishment/modification/release procedures;

-	Whether and what enhancements are required for N4 session management between the SMF and UPF, or between SMF+PGW-C and UPF+PGW-U;

-	How the network selects an anchor SMF or SMF+PGW-C in the HPLMN (or in the PNI-NPN for the PLMN plus PNI-NPN scenario) to allow routing the traffic across 3GPP accesses towards the same PSA UPF or UPF+PGW-U;

- 	Whether and what enhancements are required in PDU Session establishment/modification/release procedures;

Splitting functionality is not supported for DualSteer in any scenario.

NOTE:   Impact to existing functionality related to the change of a service-related data between a 3GPP access network and a non-3GPP access network will be considered as part of this study

* * * * End of Changes * * * *