SA WG2 Meeting #160-AH-e	S2-2401329r15

January 22 - 29, 2024, Electronic Meeting

Source:	Apple, ETRI, Nokia, Nokia Shanghai Bell

Title:	FS_MASSS DualSteer Key Issue for Session management aspects

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes a new Key Issue to cover the Session management aspects of WT#1.3 for DualSteer for the FS_MASSS TR 23.700-54.

1	Discussion

This paper proposes a new Key Issue to cover the Session management aspects of WT#1.3 of the FS_MASSS SID (SP-231802).

2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.0.0.

* * * * Start of Changes * * * *

5.1	Key Issue for DualSteer

5.1.x	Key Issue #X: Session management aspects for DualSteer

5.1.x.1 	Description

This key issue will study the following potential session management enhancements:

-	Whether and how to enhance session management procedures for initial steering of a new service for DualSteer, across two access networks belonging to the same PLMN (either HPLMN or VPLMN) or two different PLMNs or PLMN and PNI-NPN, and potential subsequent switching, which may further include following:

- 	Whether and what enhancements are required in PDU Session establishment/modification/release procedures;

-	Whether and what enhancements are required for N4 session management between the SMF and UPF, or between SMF+PGW-C and UPF+PGW-U;

-	How the network selects an anchor SMF or SMF+PGW-C in the HPLMN (or in the PNI-NPN for the PLMN plus PNI-NPN scenario) to allow routing the traffic across multi-accesses towards the same PSA UPF or UPF+PGW-U;

- 	Whether and what enhancements are required in PDU Session establishment/modification/release procedures;

Splitting functionality is not supported for DualSteer in any scenario.

NOTE:   Impact to existing session management functionality related to the change of a service related data between a 3GPP access network and a non-3GPP access network will be considered as part of this key issue.

* * * * End of Changes * * * *