SA WG2 Meeting #160-Ad Hoc-e	S2-2401351

E-meeting, January 22 – 29, 2024	(revision of S2-xxxxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0117 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of Ranging/SL Positioning Application Identifier |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Philips International B.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The term “Ranging/SL Positioning Application Identifier” is introduced in the definitions clause 3.1, but not used elsewhere in the specification. However, the term is used in SA3 specification TS 33.533. Furthermore, the current description on how the related ProSe identifiers/RSCs and V2X Service Types (onto which the Ranging/SL Positioning Application Identifiers get mapped), are used in TS 23.586 is unclear, which seems to imply only a single value indicating "Ranging/Sidelink Positioning”. However, this can be multiple values depending on the mapping from a Ranging/SL Positioning Application Identifier to a respective ProSe identifier/RSC/V2X Service Type. Although the exact details of the mapping between Ranging/SL Positioning Application Identifier and ProSe identifiers/RSCs and V2X Service Types is/can be left to stage 3, some clarification is needed in the SA2 spec on how the “Ranging/SL Positioning Application Identifier” is used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clause 5.1.1.2: clarify whether or not a mapping between Ranging/SL Positioning Application Identifiers onto ProSe identifiers/RSCs/V2X service types will be provided will be decided in stage 3. - Clauses 5.2.1 and 5.3.2.1: clarify that "Ranging/Sidelink Positioning” is not a single value for a V2X Service Type, but rather can be range of service types based on a mapping from Ranging/SL Positioning Application Identifier to V2X Service Type. - Clauses 6.4.3: update wrong reference and clarify that "Ranging/Sidelink Positioning” is not a single value for a V2X Service Type, similar to the change in clause 5.2.1 and 5.3.2.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between SA2 spec TS 23.586 and related SA3/CT1 specs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.2, 5.2.1, 5.3.2, 6.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.1.1.2	Policy/Parameter provisioned to UE

The following sets of information for Ranging/SL positioning service is provisioned to the UE:

-	When the UE is served by NG-RAN, the PLMNs in which the UE is authorized to perform Ranging/SL positioning service over PC5 reference point for acquiring relative distance, relative direction, high accuracy relative distance and high accuracy relative direction.

-	When the UE is not served by NG-RAN, indicates whether the UE is authorized to perform Ranging/SL positioning service over PC5 reference point for acquiring relative distance, relative direction, high accuracy relative distance and high accuracy relative direction.

-	The mapping between Ranging/SL positioning services (e.g. ProSe identifiers, V2X service types) and Ranging/SL positioning QoS parameters as defined in clause 5.7.2.

NOTE 1:	Details on mapping between Ranging/SL Positioning Application Identifiers onto ProSe identifiers/V2X service types or RSCs needs to be provisioned will be decided in stage 3.

-	The mapping between Ranging/SL positioning services (e.g. ProSe identifiers, V2X service types) and PQI values for RSPP transport QoS as defined in clause 5.7.3.

NOTE 2:	Details on mapping between Ranging/SL Positioning Application Identifiers onto ProSe identifiers/V2X service types or RSCs needs to be provisioned will be decided in stage 3.

-	The authorized Ranging/SL Positioning role per PLMN, e.g. Target UE, SL Reference UE, Located UE, SL Positioning Client UE, and SL Positioning Server UE.

-	When the UE is not served by NG-RAN, indicates the authorized Ranging/SL positioning role, e.g. Target UE, SL Reference UE, Located UE and SL Positioning server UE.

-	When the UE is served by NG-RAN and when Network-based Operation is not supported by the 5GC network, whether it is allowed to use UE-only operation to perform Ranging/ SL Positioning.

### 5.2.1	General

Both Model A and Model B discovery as defined in clause 6.3.2 of TS 23.304 [7] are supported for the 5G ProSe capable UEs discovery (including commercial and public safety use cases). During Ranging/SL Positioning UE direct discovery, the discovery message indicates this discovery message is for "Ranging/Sidelink Positioning"; During Ranging/SL Positioning Group Member Discovery, the Application Layer Group ID indicates a Ranging/Sidelink Positioning group that the UE belongs to.

NOTE:	The content type value indicates the discovery is for Ranging/Sidelink Positioning UE discovery. The details for Ranging/Sidelink Positioning UE discovery depends on stage 3.

Procedures for V2X communication over PC5 reference point as defined in TS 23.287 [6] are supported for the V2X capable UEs discovery. When unicast mode V2X communication procedure as defined in clause 6.3.3 of TS 23.287 [6] is used, the Service Type in Layer-2 link establishment procedure indicates "Ranging/Sidelink Positioning" based on a mapping from Ranging/SL Positioning Application Identifiers onto respective V2X Service Types denoting Ranging/Sidelink Positioning related services; when broadcast mode or groupcast mode V2X communication procedure as defined in clause 6.3.1 or clause 6.3.2 of TS 23.287 [6] is used, Service Type indicates "Ranging/Sidelink Positioning" based on a mapping from Ranging/SL Positioning Application Identifiers onto respective V2X Service Types denoting Ranging/Sidelink Positioning related services.

The role(s) of the discovered UE (e.g. Target UE, SL Reference UE, SL Positioning Server UE, Located UE) is (are) included in discovery message (for 5G ProSe capable UE) and unicast link establishment messages (for V2X capable UE) as RSPP specific metadata. More details on this and other information conveyed during discovery can be found in clause 6.4.

The procedures for Ranging/Sidelink Positioning UE Discovery & Selection are defined in clause 6.4.

### 5.3.2	Transport of RSPP over PC5

#### 5.3.2.1	General

Depending on type of the UE (e.g. V2X capable or 5G ProSe capable), V2X Communication procedures as defined in TS 23.287 [6] or 5G ProSe Direct Communication procedures as defined in TS 23.304 [7] are used for the RSPP transport between UEs over PC5:

-	For V2X capable UEs, policies and parameters defined in TS 23.287 [6] clause 5.1.2.1 are used to determine the corresponding transport configurations for the RSPP signalling. The V2X service type shall take the value(s) defined for "Ranging/SL Positioning Protocol" (i.e. based on a mapping from Ranging/SL Positioning Application Identifiers onto respective V2X Service Types denoting Ranging/Sidelink Positioning related services) in corresponding format in different regions, e.g. ITS-AID, PSID, or AID as described in TS 23.287 [6] clause 3.1.

-	For 5G ProSe capable UEs, policies and parameters defined in TS 23.304 [7] clause 5.1.3.1 applies. To determine the parameters, the ProSe services (i.e. ProSe identifiers/RSCs) shall take the value(s) defined for "Ranging/SL Positioning Protocol" (i.e. based on a mapping from Ranging/SL Positioning Application Identifiers onto respective ProSe identifiers/RSCs denoting Ranging/Sidelink Positioning related services) in corresponding format described in TS 23.304 [7] clause 3.1.

NOTE:	For a UE that is both V2X capable and 5G ProSe capable, separate RSPP sessions will be used, i.e. no V2X and 5G ProSe cross communication is supported.

Depends on the policy and parameter configuration on the UE, the RSPP may be transported using different communication mode over PC5 (i.e. broadcast mode, groupcast mode or unicast mode), with associated QoS configurations. Procedures defined in clause 6.3 of TS 23.287 [6] and clause 5.3 of TS 23.304 [7] are used to support the corresponding communicated modes.

RSPP messages are common for V2X capable UEs and 5G ProSe capable UEs. PC5-U is used as the transport layer for RSPP. The RSPP message is carried using "Non-IP" PDCP SDU type for V2X capable UEs as defined in clause 6.1.1 of TS 23.287 [6], and "Unstructured" PDCP SDU type for 5G ProSe capable UEs as defined in clause 6.1.2.2 of TS 23.304 [7].

#### 5.3.2.2	Identifiers used for RSPP transport over PC5

As defined in clause 5.6.1 of TS 23.287 [6] and clause 5.8.2 of TS 23.304 [7], multiple identifiers need to be determined for the PC5 operations, depending on the communication mode used.

For broadcast mode, the destination Layer-2 ID and source Layer-2 ID(s) are chosen based on UE configuration, as defined in clause 5.6.1.2 of TS 23.287 [6] and clause 5.8.2.2 of TS 23.304 [7].

For groupcast mode, the RSPP may provide the group identifier information. The UE converts the provided group identifier information to a destination Layer-2 ID. If the RSPP does not provide the group information, the UE determines the destination Layer-2 ID based on configuration of the mapping between V2X Service type or ProSe Identifier/RSC and Layer-2 ID. Details of the identifier determination are described in clause 5.6.1.3 of TS 23.287 [6] and clause 5.8.2.3 of TS 23.304 [7].

For unicast mode, the UE needs to establish a unicast link to the peer UE. To support the mechanisms as defined in clause 5.6.1.4 of TS 23.287 [6] and clause 5.8.2.4 of TS 23.304 [7], the RSPP needs to provide the Application Layer IDs of the UE and the peer UE, and to be used in the PC5 unicast link establishment procedure. When the Application Layer ID changes, the Layer-2 IDs used for the unicast link shall also be changed.

NOTE:	The Application Layer ID to be used is determined by RSPP layer signalling.

Based on privacy configurations, the UE may periodically change its source Layer 2 ID, as defined in TS 23.287 [6] and TS 23.304 [7].

### 6.4.3	Ranging/SL Positioning UE discovery with V2X capable UE

The procedure for Ranging/SL Positioning UE discovery and selection with V2X capable UE uses the procedures of Layer-2 link establishment over PC5 reference point in clause 6.3.3 of TS 23.287 [6] as basis with following differences and clarifications:

-	The Service Type in Layer-2 link establishment procedure indicates "Ranging/Sidelink Positioning" service " (i.e. based on a mapping from Ranging/SL Positioning Application Identifiers onto respective V2X Service Types denoting Ranging/Sidelink Positioning related services), the Policy/Parameter provisioning for "Ranging/Sidelink Positioning" service is defined in clause 5.1.

-	In step3, the UE determines the destination Layer-2 ID based on the configuration in clause 5.1 for the RSPP transport. The V2X service info indicates "Ranging/Sidelink Positioning" in the Direct Communication Request message. The Direct Communication Request message may include the RSPP metadata information.

Editor's note:	The RSPP metadata information (e.g. the specific Role(s) of to be discovered) is included as the metadata in the Direct Communication Request message, which value is determined by RAN WG2.

-	In step 4, the QoS info is for the RSPP transport.

-	In step 5, the Direct Communication Accept message includes the RSPP metadata information for the Ranging/Sidelink Service oriented Layer-2 link establishment.

Editor's note:	The RSPP metadata information (e.g. the role(s) of the response UE) is included as the metadata in the Direct Communication Accept message, which value is determined by RAN WG2.