SA WG2 Meeting #160-Ad Hoc-e	S2-2400130r01

# January 22 – 29, 2024, Elbonia

Source: 	NTT DOCOMO

Title: 	KI#1: Vertical Federated Learning

Document for: 	Approval

Agenda Item: 	19.15 (FS_AIML_CN)

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract of the contribution: This contribution proposes a KI for supporting Vertical Federated Learning in the context of WT2 of FS_AIML_CN SID.

# 1	Discussion

According to the WT2 of the FS_AIML_CN SID (SP-231800), the following aspects need to be studied.

WT2:  Study whether and what potential enhancements are needed to enable 5G system to assist in collaborative AI/ML operation involving 5GC/NWDAF and/or AF for “Vertical Federated Learning (VFL)”. The work will be based only on and limited to the scope of justified use cases.

Federated learning is a distributed machine learning technique that trains machine learning models collaboratively among multiple independent training entities, each using its own dataset. There are two general categories of federated learning: horizontal federated learning (which is supported among NWDAFs in Rel-18) and vertical federated learning (which is being studied for Rel-19).

The most fundamental difference between these two categories is the training data partitioning. As shown in Figure 1, in horizontal federated learning, all entities have the same features in their training data, meaning that there are different samples of the data in each entity, but all samples have the same set of features. However, in vertical federated learning, the training data samples in different entities have different features, meaning that for a given sample, some features may not be available in some of the training entities.

Figure 1. Training data partitioning in Horizontal vs. Vertical Federated Learning

[source: https://arxiv.org/pdf/2107.03428.pdf]

The use cases where the VLF is applicable and how to implement it are the aspects need to be studied.

# 2 Proposal

It is proposed to update the TR according to the following text.

# 5	Key Issues

********** First Change (all new text) **********

## 5.X	Key Issue #X: VLF support by 5GC

This KI is related to the WT#2.

Vertical Federated Learning is a type of distributed machine learning wherein a model is trained collaboratively by multiple parties who have a subset of training data features, contrary to horizontal FL where all parties have the same set of features (which is supported in Rel-18). The objective of this KI is to investigate whether and what use cases involving VFL are beneficial and the possible impacts on the 5GC to support them. More specifically, this KI will study the following aspects:

-	Which Use Case involving VFL are beneficial for the 5GC when training ML Model across multiple entities, why and how.

-

-

-

-	Whether and how to enhance 5GC to enable VLF supporting justified/beneficial Use Cases

NOTE 1: VFL in application layer, requiring 5GS communications between AF and UE application client, is out of scope.

********** End of Changes **********