SA WG2 Meeting #160	S2-2313384

Chicago, USA, November 13 – 17, 2023	(revision of S2-2312236)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4572 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UPF event exposure of public UE IP address |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This paper proposes to introduce the service that UPF can expose the public UE IP address to assist the UE data collection in TE 23.288. If NAT is introduced in network, as described in section 6.2.8.2.4.2, 6.2.8.2.4.3 and 6.2.8.2.4.4 in TS 23.288, the AF or NWDAF uses the SUPI/GPSI to obtain the serving SMF id(s) and the corresponding PDU Session id(s) from UDM, and request the SMF to obtain the allocated IPv4 address or IPv6 prefix for the UE. After these procedures, the AF or NWDAF gets the UE IP address. However, the allocated IPv4 address here that AF or NWDAF gets from SMF is a private IP address, not the public UE IP address. The SMF or DHCP can only allocate the private UE IP address. And in the internal 5GC, between the interaction of each 5GC elements, only the private UE IP address is used. If NAT deployed in network, the SMF determines the UPF that serving the target UE. This UPF stores the mapping table between private UE IP address and public UE IP address for the target UE that perform data collection. The SMF sends the Nupf_GetPublicUEIPaddr_Get request including the private UE IP address to the UPF to get the public UE IP address corresponding to this private network address. The UPF sends the Nupf_GetPublicUEIPaddr_Get response including the public UE IP address to the SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the service for UPF to expose the public UE IP address. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to handle the NAT deployment in UE data collection is unclear. And if the UPF can’t provide the public UE IP address, the UE data collection in NAT is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.26.1, 5.2.26.X(all new text) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

5.2.26.1	General

The following table shows the UPF Services and UPF Service Operations.

Table 5.2.26.1-1: NF services provided by the UPF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nupf_EventExposure | Notify | Subscribe/Notify | NEF, AF, NWDAF, TSNAF/DCCF/MFAF/TSCTSF |
|  | Subscribe | Subscribe/Notify | NWDAF/DCCF, SMF |
|  | Unsubscribe | Subscribe/Notify | NWDAF/DCCF, SMF |
| Nupf_GetPrivateUEIPaddr | Get | Request/Response | NEF |
| Nupf_GetPublicUEIPaddr | Get | Request/Response | SMF |

* * * Start of Change 2 (all new text) * * *

5.2.26.X	Nupf_GetPublicUEIPaddr service

5.2.26.X.1	General

This service provides the UE IP address translation information.

5.2.26.X.2	Nupf_GetPrivateUEIPaddr_Get service operation.

Service operation name: Nupf_GetPublicUEIPaddr_Get

Description: NF service consumer gets the UE public IP address corresponding to a private UE IP address assigned by 5GC.

Inputs, Required: UE private IP address, port number.

Inputs, Optional: DNN, S-NSSAI.

Outputs, Required: UE public IP address, port number.

Outputs, Optional: None.

* * * End of Change * * *