3GPP TSG-WG SA2 Meeting #160	S2-2313645

# Chicago, US, Nov 13 – 17, 2023		(revision of S2-2313398, 2810, 0544)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0951 | rev | 3 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Source of the PFD contained in PFD information retrieved from UDR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-11-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The sentence talking about the NEF action is removed because it should not be described in TS 23.288. Furthermore, the possiblitiy for a single request of PFD Determination analytics is removed as TS 23.503 mentions only the subscription to this analytics (and it would also not be clear how PFDs created based on a one-time PFD Determination analytics would be updated or removed). At the SA2#158 meeting, it was clarified that the PFD information derived from NWDAF can only be used to update PFD information created by NWDAF but not PFD information created by AF (because the PFD information derived from NWDAF can only refer to traffic that is ongoing). Hence, the NWDAF needs to know which of the existing PFD information can be subject to updates. Furthermore, the Input data IP flow including URL and domain which is more than the IP flow concept, needs to be updated as application traffic flow. The Application ID handling in UPF as described in the Input data table note is implementation related, not mandatory requirement. The procedure figure consumer NEF needs to be replaced as consumer NEF(PFDF), the Option 1 and Option 2 need to be show as optional lines. The application identifier is mandatory required for the know application, while in the procedure still describes as "may" for application identifier. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the sentence about the NEF action in the general clause as well as the the possiblitiy for a single request of PFD Determination analytics. - Add PFD ID and source NF type in the input data. - Change the IP Flow Information to Application Traffic Flow Information in the input data table and update the table note description on Application ID handling in UPF. - Update of description in step 5 in the procedure for NWDAF-assisted PFD Determination. - Addition of several missing details and some terminology alignment. - Correct procedure figure with NEF(PFDF) as consumer, update optional step lines and remove "may" for application identifier in step 3 description |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NWDAF could update AF provided PFD information. Incomplete description of PFD information management. Not correct IP flow input description and UPF handling. Not correct procedure figure and not aligned procedure descriptions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.16.1, 6.16.2, 6.16.3, 6.16.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Changes from Ericsson CR#0936 and Nokia CR#1008 have been incorporated. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.16.1	General

This clause specifies the procedure on how NWDAF can provide NWDAF-assisted PFD Determination analytics, in the form of statistics.

To assist determination of PFDs for known application identifiers, if the related Service Level Agreement does not preclude this, an NWDAF may perform data analytics on existing PFD information and user plane traffic and provide analytics results in the form of new or updated PFD determination statistics, e.g. new or updated IP 3-tuple list, to an analytics consumer in the 5GC. The NEF(PFDF) as the consumer uses the PFD determination statistics provided by the NWDAF as defined in TS 23.502 [3] and in TS 23.503 [4].

The service consumer may be a NEF(PFDF).

The consumer of these analytics shall indicate in the subscription:

-	Analytics ID = " PFD Determination";

-	Target of Analytics Reporting: "any UE";

-	Application identifier;

-	Analytics Filter Information optionally containing:

-	S-NSSAI;

-	DNN;

-	An Analytics target period indicates the time period over which the statistics are requested;

-	Analytics Reporting Parameters indicating either periodic reporting (together with the parameter periodicity) or reporting when threshold is reached (together with the parameter Reporting type (as defined in clause 4.15.1 of TS 23.502 [3]) to indicate that the report shall only occur when the information differs from the previous report);

-	Optionally, the preferred level of accuracy on the PFD determination statistics;

-	Optionally, maximum number of objects. This refers to the analytic output (i.e. maximum number of new or updated application traffic descriptions in the PFD determination statistics);

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

# * * * * Second change * * * *

### 6.16.2	Input Data

The NWDAF collects information on user data traffic from NF(s) for a known Application ID optionally together with a specific S-NSSAI and/or DNN, and retrieves the existing PFDs from the NEF(PFDF). The detailed input data are described in Table 6.16.2-1.

Table 6.16.2-1: Input data to detect known application from NFs

| Information | Source | Description |
| --- | --- | --- |
| Application ID for detection of application traffic description (NOTE 1) | UPF | Identifies the application detection algorithm used for the detection of application traffic description |
| Application Traffic Flow Information (1..max) | UPF | Per Application Traffic flow related information for the application |
| > IP 5-tuple | UPF | Identifies a service flow of the UE that uses the application. |
| > Start time | UPF | Start time of traffic detection for the flow. |
| > End time | UPF | End time of traffic detection for the flow. |
| > UL Data volume | UPF | Measured UL data traffic volume for the flow. |
| > DL Data volume | UPF | Measured DL data traffic volume for the flow. |
| > UL Data Rate | UPF | Measured UL data rate for the flow. |
| > DL Data Rate | UPF | Measured DL data rate for the flow. |
| > URL list | UPF | List of URLs extracted from the inspected user plane packets in the flow. |
| > Domain Name list | UPF | List of domain names extracted from the inspected user plane packets in the flow. |
| PFD Information | NEF(PFDF) | PFD Information stored in the UDR (as Application Data) and retrieved by NEF (PFDF), as defined in clause 6.1.2.3.2 of TS 23.503 [4]. |
| > Application ID | NEF(PFDF) | Identification of the application that refers to one or more application detection filters. |
| > PFD(s) (1..max) | NEF(PFDF) |  |
| >> PFD ID | NEF(PFDF) |  |
| >> IP 3-tuple list | NEF(PFDF) | Including protocol, server side IP address and port number. |
| >> URL list | NEF(PFDF) | The significant parts of the URL to be matched, e.g. host name. |
| >> Domain Name list | NEF(PFDF) | A Domain Name matching criteria and information about applicable protocol(s). |
| >> Source NF type (NOTE 2) | NEF(PFDF) | Indicates the source NF type which generated the PFD (i.e. AF or NWDAF) |
| NOTE 1: In the UPF, the Application ID for detection of application traffic description can refer to a detection algorithm that has a wider scope than the detection algorithm the Application ID provided by the consumer refers to. NOTE 2: The absence of the Source NF type indicates that this PFD was generated by the AF. |  |  |

NOTE:	Extensive reporting of all traffic flows may conflict with the requirement to avoid extra UPF load. An NWDAF may subscribe only for reporting for some UEs to limit the load.

# * * * * Third change * * * *

### 6.16.3	Output Analytics

The output analytics of PFD Determination is defined in Table 6.16.3-1. The output analysis may be used to provision new or updated PFDs information for known applications. The NWDAF may assign a confidence to the PFD Determination statistics based on the analytics of input data provided by UPF and NEF(PFDF).

Table 6.16.3-1: PFD Determination statistics

| Information | Description |
| --- | --- |
| Application ID | Identifies the application for which the PFD information applies. |
| List of application traffic description (1..max) | Application traffic description information derived by the analytics for the application. |
| > S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| > DNN | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| > IP 3-tuple list | Including protocol, server side IP address and port number. |
| > URL list | The significant parts of the URL to be matched, e.g. host name. |
| > Domain Name list | A Domain Name matching criteria and information about applicable protocol(s). |
| > Confidence | Confidence on the provided application traffic description for the Application ID. |

# * * * *Fourth change* * * *

### 6.16.4	Procedures

Figure 6.16.4-1 shows the procedure that a consumer can use to request PFD Determination analytics.

Figure 6.16.4-1: A procedure for NWDAF-assisted PFD Determination

1.	The Consumer NF (NEF(PFDF)) subscribes to the NWDAF to request PFD Determination analytics for a known application identifier. The Target of Analytics Reporting is set to Any UE.

The Analytics Filter Information may optionally include the S-NSSAI and/or DNN.

2.	The NWDAF fetches currently stored PFD information in use from UDR via NEF(PFDF) for the Application ID provided in step 1, as listed in Table 6.16.2-1.

3.	The NWDAF collects information from the UPF (event "UserDataUsageMeasures") as listed in Table 6.16.2-1 either via SMF (Option 1) or directly from UPF (Option 2) as defined in clause 5.8.2.17 of TS 23.501 [2]. NWDAF discovers the SMF (Option 1) or UPF (Option 2) through NRF as defined in clause 4.15.4.5.3 of TS 23.502 [3]. The event "UserDataUsageMeasures" is defined in clause 5.2.26.2 of TS 23.502 [3], the subscription to the event includes an Application ID set to a value that allows the NWDAF to retrieve input information to derive PFD Determination Analytics for the Application ID that is provided by the consumer.

NOTE:	The Application ID provided by the consumer and the Application ID provided to the UPF to retrieve input data for PFD Determination Analytics can be different ones.

4.	UPF reports the data directly to NWDAF.

5.	The NWDAF derives PFD Determination analytics, e.g. new or updated application traffic description information for the existing Application ID. The NWDAF can only update or delete application traffic description information previously generated by the NWDAF, i.e. that contains "NWDAF" as source NF type. When the consumer provides the preferred level of accuracy, the NWDAF provides PFD Determination Analytics that reaches this level to the consumer.

6.	The NWDAF notifies PFD Determination analytics to the consumer NF (i.e. NEF(PFDF)) according to the reporting mode indicated by the Consumer NF (i.e. NEF(PFDF)) during the subscription in step 1.

# * * * * End of changes * * * *