3GPP TSG-SA2 Meeting #160	S2-2312763

Chicago, USA, November 13-17, 2023

Source:	Lenovo

Title:	New solution for KI#3: map PDU Set importance to DSCP marking over N3/N9 in the transport network

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM Ph2/ Rel-19

Abstract of the contribution: new solution on mapping PDU Set importance to DSCP marking over N3/N9 in the transport network

1	Discussion

During SA2#159 meeting, KI#3: Leverage PDU Set QoS information for DSCP marking over N3/N9 in the transport network is agreed. In this paper, a new solution for KI#3 is proposed.

PDU set information includes PDU Set Sequence Number, Indication of End PDU of the PDU Set, PDU Sequence Number within a PDU Set, PDU Set Size in bytes, PDU Set importance (PSI). Intuitively, PSI can be mapped into corresponding DSCP value, which determines the transmit priority in the transport network. Therefore, in order to leverage PDU Set QoS information for DSCP marking over N3/N9 in the transport network, one possible solution is that SMF provides PSA UPF with the mapping between PSI and DSCP value. In this way, PSA UPF marks DL PDU with the DSCP value based on PSI associated with the DL PDU, and the mapping provided by SMF. When delivering the DL PDU towards NG-RAN, the UPF(s) within 5GC will take the DSCP value into consideration.

3	Proposal

* * * * Start of Change * * * *

## 6.x	Solution #X: map PDU Set importance to DSCP marking over N3/N9 in the transport network

### 6.x.1	Key Issue mapping

This solution is for Key Issue #3, which addresses leveraging PDU Set QoS information for DCSP marking over N3/N9 in the transport network.

### 6.x.2	Description

PDU set information includes PDU Set Sequence Number, Indication of End PDU of the PDU Set, PDU Sequence Number within a PDU Set, PDU Set Size in bytes, PDU Set importance (PSI). Intuitively, PSI can be mapped into corresponding DSCP value, which determines the transmit priority in the transport network. Therefore, in order to leverage PDU Set QoS information for DSCP marking over N3/N9 in the transport network, one possible solution is that SMF provides PSA UPF with the mapping between PSI and DSCP value. In this way, PSA UPF marks DL PDU with the DSCP value based on PSI associated with the DL PDU, and the mapping provided by SMF. When delivering the DL PDU towards NG-RAN, the UPF(s) within 5GC will take the DSCP value into consideration.

### 6.x.3   Procedures

#### 6.x.3.1   provisioning mapping of PDU set importance and DSCP value

Figure 6.x.3.1-1: provisioning mapping of PDU set importance and DSCP value

This procedure provides the procedure of provioning mappong of PDU set importance and DSCP value.

- From PCF to SMF: flow description and PDU Set QoS parameters.

- Upon receiving the PDU Set QoS parameters associated with the flow, SMF determines the mapping of PDU set importance (PSI) and DSCP value for the QoS flow.

- From SMF to UPF: QFI and the mapping of PSI and DSCP value.

- When DL PDU arrives, UPF identifies PDUs that belong to PDU sets and obtains the PSI associated with the DL PDU. UPF marks DL PDU with DSCP value based on PSI associated with the DL PDU, and the mapping provided by SMF.

### 6.x.4	Impacts on services, entities and interfaces

The proposed solution is based on Rel-18 procedures but some enhancements may be needed to make it possible for the PSA UPF to know the mapping of PDU set importance and DSCP value, such as:

# SMF：

-	upon receiving PDU Set QoS parameters from PCF, determines the mapping of PDU set importance and DSCP value for the QoS flow

-	configure UPF with QFI and the mapping of PDU set importance and DSCP value

# PSA UPF：

-	mark DL PDU with DSCP value based on PDU set importance and the mapping provided by SMF

* * * * End of Change * * * *