SA WG2 Meeting #160	S2-2313292

13 - 17 November, 2023, Chicago, USA	(Revision of S2-2312937)

Source: 	China Mobile

Title: 	KI #1: Update on interaction between DC application server and IMS network

Document for: 	Approval

Agenda Item: 	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract of the contribution: This contribution updates key issue #1 on supporting interactions between DC AS and IMS network via N33 and DC3.

# 1	Discussion

This contribution proposes to update key issue #1 to clarify that both scenarios that DC application server interacts with IMS network via N33 and via DC4 need to be supported.

# 2 Proposal

It is proposed to capture the following contents into TR 23.700-77.

- 5	Key Issues

- 5.1	Key Issue #1: Extensible IMS mechanism supporting IMS events in the context of DC communication

- 5.1.1	Description

This key issue investigates needed architectural enhancement to IMS architecture to support an extensible mechanism that enables DC based services/applications to subscribe to and receive reports about IMS events related to IMS services in the context of DC communication.

The KI covers:

-	identification of IMS events that Network Function/Application function for DC based services/applications can subscribe to. The IMS events can include events related to a specific IMS subscriber / groups of IMS subscribers.

Editor’s Note: Handling of groups of IMS subscribers is FFS.

- -	event subscription and notification mechanisms. When the IMS event is related to IMS specific subscriber(s) this includes functionality to determine the IMS NF(s)/Nodes serving the specific IMS subscriber(s) for which the specific event was requested.

NOTE:	Both scenarios that trusted and untrusted DC application server interacts with IMS networkneed to be supported.