3GPP TSG-WG SA2 Meeting #160Adhoc 	S2-2402363

22nd – 29th January, 2024, Electronic                            (revision of S2-2401628)

Source:	Oracle

Title:	WID: TEI19_SLUPiR – Spending Limits for UE Policies in Roaming scenario

Document for:	approval

Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: Spending Limits for UE Policies in Roaming scenario

## Acronym: TEI19_SLUPiR

## Unique identifier: 	{A number to be provided by MCC at the plenary}

Potential target Release: Rel-19.

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

Dependency on non-3GPP (draft) specification:

## 3	Justification

UE policies encompass the management of ANDSP, URSP, V2XP, ProSeP (see 23.503 clause 4.2.2). In the context of this document, UE policies refer to URSP only.

Policy decisions based on spending limits is a function that allows PCF taking actions related to the status of policy counters that are maintained in the CHF. In R18 this is defined as Policy decisions based on spending limits (23.503 v18.3 clause 6.1.1.4)

These policy decisions can be AM policies or UE policies or Session related policies.

However, UE Policy decisions based on spending limits are currently scoped to the non-roaming scenario.

It seems useful to enhance the scope of UE Policy decisions based on spending limits to cover the roaming scenario as well. This is applicable to both – LBO and HR roaming scenarios.

The following scenarios have been identified:

Scenario 1 (‘SL based UE policies for HR roaming’): Subscriber has a plan that allows the UE to use a special promo S-NSSAI/DNN until the monthly spending limit of $60 is reached. This is applicable also when the UE roams out to a partner PLMN that uses an HR roaming architecture for inbound roamers from the HPLMN. Monthly spending limit of $60 has reached. H-CHF notifies the H-PCF for a UE. H-PCF for a UE removes the special promo URSP rule. So now the UE cannot use anymore the special promo S-NSSAI/DNN neither at HPLMN or/and at the partner PLMN.

Scenario 2 (‘SL based UE policies for LBO roaming’): An operator has an LBO roaming agreement with a partner PLMN for its outbound roaming subscribers. According to the agreement, the UE is allowed to use a dedicated local S-NSSAI/DNN in the partner PLMN until the monthly spending limit of $60 is reached. Once quota has reached, H-CHF notifies the H-PCF for a UE. H-PCF for a UE removes the VPLMN specific URSP rule. So now the UE cannot use anymore the dedicated local S-NSSAI/DNN in the partner PLMN.

## 4	Objective

The objective is to enable the dynamic change of UE policies based on spending Limits in the roaming scenario.

The expected time for completing this work is 0.5 TUs. 0.25 time slots in one SA2 meeting to define the functional description and 0.25 time slots in next SA2 meeting to define the procedures and services.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.503 TS 23.501 TS 23.502 | Define the PCF impacts. Update clause 6.1.1.4 to add roaming support for UE policies. Update clause 6.3.11 to add roaming support for UE policies (Specifically, CHF selection aspects). Update clauses 4.16.11/4.16.12/4.16.13 to add roaming functionality to the interaction between PCF and CHF during UE policy association establishment/modification/termination. | TSG#106 (Dec 2024) |  |

## 6	Work item Rapporteur(s)

Baniel, Uri	uri dot baniel at oracle dot com

## 7	Work item leadership

SA2.

## 8	Aspects that involve other WGs

Charging aspects will be studied by SA5.

## 9	Supporting Individual Members

Supporting IM name

Oracle

Verizon UK Ltd

China Telecom

Dish Network

Vodafone