SA WG2 Meeting #161 - Athens	S2-2402941

February 26 – March 1, 2024, Athens

Source:	CableLabs, Charter Communications

Title:	KI#6: Update Solution #17: Procedures for L4S in non-3GPP access networks

Document for:	Approval

Agenda Item:	19.3

Work Item / Release: 	FS_XRM_Ph2 / Rel-19

Abstract: This paper proposes adding procedures specific for wireline, untrusted and trusted non-3GPP accesses currently not present in Solution #17.

# 1. Introduction

This pCR proposes adding procedures specific for wireline, untrusted and trusted non-3GPP accesses which are currently not present in Solution #17.

# 2. Proposal

This contribution proposes updates to Solution #17 in TR 23.700-70 clause 6.17.3.

Start of changes

#### 6.17.3.1	Procedures in wireline 5G access network

Figure 6.17.3.1-1: High-level procedure of the solution for W-AGF and 5G-RG

1.	Steps 1-2a specified in clause 7.3.1.1 of TS 23.316 [17], with the difference that the 5G-RG 5GSM Core Network Capability included in the PDU Session Establishment Request within the N1 SM container indicates whether the 5G-RG supports ECN marking for L4S.

2a. The N2 SM information carries information that the AMF shall forward to the W-AGF which includes: For each QoS Flow, an ECN marking for L4S indicator to W-5GAN in the case of ECN marking for L4S in W-5GAN.

The N1 SM container that the AMF shall provide to the 5G-RG contains the: For each QoS Flow, an ECN marking for L4S indicator to 5G-RG in the case of ECN marking for L4S in the 5G-RG, if the 5G-RG indicated support for ECN marking for L4S.

2b. The AMF shall under request of the SMF send a N2 PDU Session Resource Setup Request message, which includes the ECN marking for L4S indicator, to W-AGF to establish the access resources for this PDU Session.

3.	Based on the QoS flows and QoS parameters received in the previous step, W-AGF determines the corresponding L4S-enabled wireline QoS resource needed for the PDU session.

4.	The W-AGF sets up the W-UP resources for the PDU session.

5.	After all W-UP resources are established, the W-AGF shall forward to 5G-RG via the W-CP signalling connection the PDU Session Establishment Accept message (including the ECN marking for L4S indicator) received in step 2b.

6.	The W-AGF shall send to AMF an N2 PDU Session Resource Setup Response including: established QoS Flows status (active/not active) for ECN marking for L4S in wireline access.

7.	All steps specified after step 14 in clause 4.3.2.2.1 of TS 23.502 [3] are executed according to the PDU Session Establishment procedure over 3GPP access.

Similar procedures would be applicable for the 5G-RG connected to 5GC via NG-RAN.

Figure 6.17.3.1-2: High-level procedure of the solution for W-AGF and FN-RG

- Steps 0-2a specified in TS 23.316 [17] clause 7.3.4.

- The N2 SM information carries information that the AMF shall forward to the W-AGF which includes: For each QoS Flow, an ECN marking for L4S indicator to W-5GAN in the case of ECN marking for L4S in W-5GAN.

- The AMF shall under request of the SMF send a N2 PDU Session Resource Setup Request message, which includes the ECN marking for L4S indicator, to W-AGF to establish the access resources for this PDU Session.

- Based on its own policies, configuration and based on the QoS flows, QoS parameters received in the previous step, the W-AGF shall determine the corresponding L4S-enabled wireline QoS resource needed for the PDU session.

The W-AGF may perform Access specific resource reservation with the AN, that is, it sets up the W-UP resources for the PDU session.

- The W-AGF shall send to AMF an N2 PDU Session Resource Setup Response.

- All steps specified in TS 23.502 [3] clause 4.3.2.2.1 after step 13 are executed according to the PDU Session Establishment procedure over 3GPP access.

Next change, all new text

#### 6.17.3.2	Procedures in untrusted non-3GPP access

Figure 6.17.3.2-1: High-level procedure for untrusted non-3GPP access

- Steps 1-2a specified in clause 4.12.5 of TS 23.502 [3], with the difference that the N2 SM information carries information that the AMF shall forward to the N3IWF which includes: For each QoS Flow, an ECN marking for L4S indicator in the case of ECN marking for L4S in untrusted non-3GPP access.

- The AMF shall under request of the SMF send a N2 PDU Session Resource Setup Request message, which includes the ECN marking for L4S indicator, to N3IWF to establish the non-3GPP access resources for this PDU Session.

- For an L4S-enabled QoS Flow, N3IWF establishes a dedicated IPsec Child SA.

- The N3IWF shall send to UE an IKE Create_Child_SA request which may include a DSCP value associated with the Child SA, based on operator policy.

- Steps 4b-7 specified in clause 4.12.5 of TS 23.502 [3] are executed according to the PDU Session Establishment procedure via untrusted non-3GPP access.

- On the user-plane, PDUs belonging to L4S-enabled QoS flow are transferred over dedicated IPsec child SA. If a DSCP value is included, then the UE and the N3IWF shall mark all IP packets sent over this Child SA with this DSCP value.

#### 6.17.3.3	Procedures in trusted non-3GPP access

Figure 6.17.3.3-1: High-level procedure for trusted non-3GPP access

- Steps 1-2a specified in clause 4.12.5 with the modifications specified in clause 4.12a.5 of TS 23.502 [3], with the difference that the N2 SM information carries information that the AMF shall forward to the TNGF which includes: For each QoS Flow, an ECN marking for L4S indicator in the case of ECN marking for L4S in trusted non-3GPP access.

- The AMF shall under request of the SMF send a N2 PDU Session Resource Setup Request message, which includes the ECN marking for L4S indicator, to TNGF to establish the non-3GPP access resources for this PDU Session.

- For an L4S-enabled QoS Flow, TNGF establishes a dedicated IPsec Child SA.

- The TNGF shall send to UE an IKE Create_Child_SA request which may include a DSCP value associated with the Child SA, based on operator policy.

- Steps 4b-7 specified in clause 4.12.5 with the modifications specified in clause 4.12a.5 of TS 23.502 [3] are executed according to the PDU Session Establishment procedure via trusted non-3GPP access.

- On the user-plane, PDUs belonging to L4S-enabled QoS flow are transferred over dedicated IPsec child SA. If a DSCP value is included, then the UE and the TNGF shall mark all IP packets sent over this Child SA with this DSCP value.

End of changes