SA WG2 Meeting #161	S2-2403177

# 26 February – 1 March 2024, Athens, Greece

Source:	Lenovo

Title:	New Solution: Identifying traffic corresponding to a human user during PDU session establishment

Document for:	Approval

Agenda Item:	19.8

Work Item / Release:	FS_UIA_ARC/ Rel-19

Abstract of the contribution:

# 1		Discussion

The solution assumes that there is User Profile information stored in the UDR (e.g. see solution in S2-2402066). The user profile is identified by a User Identifier (User ID). The same User ID which is also configured in the UE.

A network connection [PDU session] associated with a user identifier may be requested by a UE when a human user logs in to the UE and the UE determines that a User Profile is associated with this user (the determination may be based on UE implementation or based on User Profile configuration in the UE). In such a case the UE requests a PDU session establishment including the User Identifier that requested the network connection. The UE includes the User Identifier within the NAS message and also includes the User Identifier within the SM PDU DN Request container in the form of an EAP-Response/Identity NAI message.

The AMF determines the applicable User Profile by retrieving the associated User Profile (identified by a User Profile ID) from the UDM corresponding to the User Identifier provided by the UE (the AMF can retrieve the user profile(s) from the UDM during registration and store in the UE context). The AMF determines if the User Profile is allowed to be used by the device (by retrieving the device's PEI and determining if the PEI or Device ID of the UE is included in the allowed PEI list of the User Profile).

The AMF then creates an SM Context Request to the SMF including the User Profile Identifier of the retrieved User Profile and also includes the SM Container.

The SMF retrieves the SM Subscription Data corresponding to the User Profile identifier from the UDR and retrieves the user specific SM related subscription data. From the data the SMF determines the AAA server identity (if user authentication is required) and the type of authentication required. The SMF invokes a AAA authentication request including the NAI provided by the UE in the SM PDU DN Request Container. The AAA server initiates an EAP authentication with the UE.

After authentication is completed the SMF selects a PCF and establishes an SM Policy Control Request with the PCF. The SMF includes the User Profile Identifier in the request. The PCF retrieves the user specific PDU session policy control data according to User Profile ID and derives policy rules which are sent to the SMF.

The SMF then sends a PDU session establishment accept message to the UE.

Further details for the solution are provided below:

# 2		Proposal

The following key issue is proposed.

******************************** First change  *******************************

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> | <Key Issue #3> | <Key Issue #4> |
| ##X: Identifying traffic corresponding to a human user during PDU session establishment | X | X | X |  |
| #2 |  |  |  |  |

******************************** Second change (all new text)  *******************************

## 6.X	Solution #X: Identifying traffic corresponding to a human user during PDU session establishment

### 6.X.1	Key Issue mapping

Editor's note:	This clause lists the key issue(s) addressed by this solution.

The solution addresses Key Issue 1 aspects of Key Issue 2 and 3.

### 6.X.2	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). Sub-clause(s) may be added to capture details.

The solution assumes that there is User Profile information stored in the UDR

User Profile information is available in AM subscription, SM subscription data and Policy control subscription data and  includes one or more of the following:

-	A User Profile Reference ID that uniquely identifies the User Profile among all User Profiles in the same mobile subscription;

-	One of more User Identifiers (e.g. user@example.com);

-	In AM subscription data

-	One or more devices that can use this User Profile;

-	Allowed network slices

-	In SM subscription data

-	Authentication information, such as credentials (e.g. a password, digital certificated, etc.) and authentication types;

-	In PDU session related control data

-	One or more applications associated with this User Profile;

-	The QoS settings that should be applied to the traffic associated with this User Profile and;

-	The list of services available for this User Profile;

There may be more than one User Profiles each associated with a User Profile Reference ID that are linked to the 3GPP subscription (SUPI).

It is also assumed that User Profile information is also configured in the UE containing user id and credential information.

A network connection [PDU session] associated with a user identifier may be requested by a UE when a human user logs in to the UE and the UE determines that a User Profile is associated with this user (the determination may be based on UE implementation or based on User Profile configuration in the UE). In such a case the UE requests a PDU session establishment including the User Identifier that requested the network connection. The UE includes the User Identifier within the NAS message and also includes the User Identifier within the SM PDU DN Request container in the form of an EAP-Response/Identity NAI message.

The AMF determines the applicable User Profile by retrieving the associated User Profile (identified by a User Profile ID) from the UDM corresponding to the User Identifier provided by the UE (the AMF can retrieve the user profile(s) from the UDM during registration and store in the UE context). The AMF determines if the User Profile is allowed to be used by the device (by retrieving the device's PEI and determining if the PEI or Device ID of the UE is included in the allowed PEI list of the User Profile).

The AMF then creates an SM Context Request to the SMF including the User Profile Identifier of the retrieved User Profile and also includes the SM Container.

The SMF retrieves the SM Subscription Data corresponding to the User Profile identifier from the UDR and retrieves the user specific SM related subscription data. From the data the SMF determines the AAA server identity (if user authentication is required) and the type of authentication required. The SMF invokes a AAA authentication request including the NAI provided by the UE in the SM PDU DN Request Container. The AAA server initiates an EAP authentication with the UE.

After authentication is completed the SMF selects a PCF and establishes an SM Policy Control Request with the PCF. The SMF includes the User Profile Identifier in the request. The PCF retrieves the user specific PDU session policy control data according to User Profile ID and derives policy rules which are sent to the SMF.

The SMF then sends a PDU session establishment accept message to the UE.

### 6.X.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

The procedure is shown below:

Figure 6.x.3-1 - User specific PDU establishment request

0a.	The UE registers to the network based on the Registration procedure described in Section 4.2.2.2 of 3GPP TS 23.502

0b.	During registration the AMF retrieves from the UDM the subscription data which includes a list of User Profiles (identified by a User Profile Identifier) which contain a list of user identifiers, list of allowed devices.

1. A user logs in the device using a user identifier. 1b. The UE may determine from the User Profile Data that a User Profile is associated with the User Identifier.

2.	The UE sends a PDU session establishment request including the User Identifier in the UL NAS transport message. In addition, the UE includes the User Identifier in the form of an EAP-Response/Identity NAI message within the SM PDU DN Request container included within the SM PDU session establishment request container.

3a.	The AMF determines, from the list of User Profiles associated to the 3GPP subscription, the user profile associated to the user identifier provided by the UE in the NAS message of the PDU session establishment request. The AMF can retrieve the user profile(s) from the UDM during registration and store in the UE context.

3b.	The AMF also determines if the user identifier is allowed for this UE (from the list of allowed PEI).

4a.	The AMF selects an SMF and sends a Create SM Context request. The AMF includes in the Create SM Context Request the User Profile Identifier of the retrieved user profile.

4b.	The SMF retrieves from the UDM/UDR the user specific Session Management Subscription Data associated to the User Profile identifier provided by the AMF. The SMF uses the User Profile to determine the address of the AAA server (if user authentication is required)

5a.	The SMF sends a AAA request message to the operator or 3rd party AAA server including the EAP message provided by the UE in the SM PDU DN Request Container

5b.	An authentication method is derived from the AAA server based on the user identifier provided_

6.	The AAA server starts an EAP authentication method with the SMF and the UE.

7a.	The SMF selects a PCF and sends an SM Policy Control Create Request message including in the request the User Profile identifier of the User Profile associated to the user identifier

7b. The PCF retrieves from the UDM/UDR PDU Session policy control subscription information according to User Profile Identifier and derives policy rules.

7c.	The PCF provides the Policy rules to the SMF in the SM Policy Control Create response message.

8.	The SMF selects UPF(s) and establish an N4 session

9., 10, 11 The SMF sends a PDU session establishment accept message to the UE

### 6.X.4	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

-	 UE includes during PDU session establishment request user identifier information

-	During PDU session est. req.

-	AMF retrieves from UDM during PDU session est., user profile associated with user identifier and determines if user/device is allowed

-	SMF retrieve from UDM/UDR authentication data to authenticate user

-	PCF retrieves from UDM/UDR user profile related information (e.g. QoS policies).

******************************** End of change *******************************