3GPP WG2 Meeting #161	S2-2401951

# Athens, Greece, February 26- March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | XXXX | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Replication and elimination for redundancy transmission for FRER |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2024-01-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clarify how the 5GC support the replication and elimination for redundancy, protocol aspects to replicate/eliminate for FRER.​ |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description of replication and elimination for redundancy for FRER. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5GS is not supported for replication and elimination for redundancy for FRER. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.33.2.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change all the text are new* * * *

5.33.2.X	Support FRER based on Redundant User Plane Paths

When the 5GS supports TSC communication, a UE may establish two redundant PDU Sessions over the 5G network, and the DS-TT and the NW-TT support the replication and elimination for redundancy transmission based FRER, as defined in IEEE 802.1CB [XX].

In order to support IEEE 802.1CB features related to FRER, the DS-TT and NW-TT supports the following functions as defined in Clause 7.3 in IEEE 802.1CB [XX]:

-	Sequencing function;

-	Stream splitting function;

-	Individual recovery function;

-	Sequence encode/decode function;

-	Stream identification.

For single UE case, support of redundant PDU Sessions for FRER include:

-	A UE initiates two redundant PDU Sessions, respectively based on Dual Connectivity based end to end Redundant User Plane Paths as described in clause 5.33.2.1 of TS 23.501 [x] but select the same UPF or a UE initiates two redundant PDU Sessions, respectively via a single RAN and select the same UPF.

-	The SMF should select the same UPF to ensure FRER based redundancy transmission, based on local configuration or the specific DNN.

-	DS-TT provides a PMIC(which includes the FRER indication) and the DS-TT port MAC address to the UE, which includes the PMIC as an optional Information Element of an N1 SM container and triggers the UE requested PDU Session Establishment procedure or PDU Session Modification procedure to forward the PMIC to the SMF. SMF forwards the PMIC and the port number of the related DS-TT port to TSN AF or TSCTSF as described in clauses 4.3.2.2 and 4.3.3.2 of TS 23.502 [3]

NOTE: how the UE determines to trigger the redundant PDU session for FRER is based on implementation.

-	During the PDU session establishment, the UPF/NW-TT provides the PMIC/UMIC to SMF. The SMF provide the PMIC and UMIC of DS-TT/NW-TT transparently to TSCTSF, as described in Clause 5.28.3.1.

NOTE: For the redundant PDU session, only one PMIC from DS-TT, and one PMIC/UMIC from UPF/NW-TT, are provided to SMF.

-	The TSN AF or TSCTSF provides the FRER configurations in PMIC to the UE/DS-TT for UL replication and DL elimination.

-	The TSN AF or TSCTSF provides the FRER configurations in PMIC/UMIC to the UPF/NW-TT for DL replication and UL elimination.

TSN System

TSN Bridge/End Station

DS-TT

UE

UPF

NW-TT

PDU Session1

PDU Session2

Figure 5.33.2.X-1: Single UE based FRER transmision

# * * * * End of changes * * * *