SA WG2 Meeting #161	S2-2401959

Athens, GREECE, February 26 – March 01, 2024

Source:	ETRI

Title:	New Sol. to KI #1.2: Enhancement to registration procedure to support DualSteer

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes a new solution for KI #1.2.

1	Discussion

TR 23.700-54 describes the key issue #1.2 “Registration and mobility management for DualSteer” including the following:

-	Whether and what enhancements are needed in functions and procedures of registration, deregistration and mobility management for supporting DualSteer.

Regarding to the key issue #1.2, this contribution proposes the following enhancement to registration procedure to support DualSteer.

-	“DualSteer policy” may be provisioned during the registration procedure.

-	“DualSteer policy” may be used for both traffic steering and traffic switching.

The need for further clarification of the term DualSteer device is remarked in editor’s note.

The need for further refinement of the terms “DualSteer subscription data” and “DualSteer policy” are remarked in editor’s note.

2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.1.0.

* * * * Start of Changes * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of DualSteer Solutions to Key Issues

|  | Key Issues for DualSteer |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# | <Key Issue #1.1> | <Key Issue #1.2> |  |  |
| #X: Enhancement to registration procedure to support DualSteer |  | X |  |  |

Second Change

6.1	Solutions for DualSteer

6.1.X	Solution #X: Enhancement to registration procedure to support DualSteer

6.1.X.1	Description

When a UE registers with a PLMN, “DualSteer policy” may be provisioned by the HPLMN or PNI-NPN. During the registration procedure, “DualSteer policy” will be provisioned by PNI-NPN if the UE is subscriber of PNI-NPN, and it will be provisioned by HPLMN otherwise.

UE’s “DualSteer subscription information” and “DualSteer policy” for a DualSteer device are stored in UDR. The “DualSteer subscription information” is stored as part of subscription data of a UE. The “DualSteer policy” for a DualSteer device is stored as policy subscription data in UDR

Editor's note:	The term DualSteer device will be refined and UDR storing will be clarified

Editor's note:	The terms “DualSteer subscription information” and “DualSteer policy” will be refined.

6.1.X.2	Procedures

Impact on registration procedure:

In step16, The PCF may get “DualSteer subscription information” in the subscription data of a UE and “DualSteer policy” for the DualSteer device from UDR of HPLMN.

In step21b, The PCF may provision “DualSteer policy” to the DualSteer device. The “DualSteer policy” may include preferred PLMN/RAT combination for service traffic descriptor with further criteria (e.g., time, location, QoS).

NOTE :	UE Policy Association Establishment/Modification procedure is not changed, only stored and exposed data will be impacted.

Editor's note:	The term DualSteer device will be refined.

Editor's note:	The terms “DualSteer subscription information” and “DualSteer policy” will be refined.

6.1.X.3	Impacts on services, entities and interfaces

Impact on UE:

-	The UE may receive “DualSteer policy” from the 5GCN.

Impact on PCF:

-    The PCF may create “DualSteer policy” and provision it to the UE.

Impact on UDR:

-	The UDR may store “DualSteer subscription information” in subscription data of a UE.

-	The UDR may provision “DualSteer subscription information” in the subscription data of a UE to the PCF.

-	The UDR may store “DualSteer policy” for a DualSteer device.

-	The UDR may provision “DualSteer policy” for a DualSteer device to the PCF.

Editor's note:	The term DualSteer device will be refined.

Editor's note:	The terms “DualSteer subscription information” and “DualSteer policy” will be refined.

* * * * End of Changes * * * *