SA WG2 Meeting #161  	  S2-2401994

26 February-1 March 2024, Athens,Greece

Source:	ZTE

Title:	TR 23.700-84: KI#2 – Solution about 2-sides VFL inference process

Document for:	Approval

Agenda Item:	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract: This contribution proposes a solution for KI#2 about inference process of VFL

# 1. Discussion

In this contribution, we propose a solution for the inference process of 2-sides VFL, which includes VFL server and one or more VFL client(s).

# 2. Proposal

This paper proposes to add a solution for Key Issue #2 to TR23.700-84 as follows.

---------- Start of change ----------

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues and Use Cases

|  | Key Issues |  | Use cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <use case #x> | <use case #y> |
| #1 |  | X |  |  |
| #2 |  |  |  |  |

---------- Next change (all new text) ----------

6.x	Solution #X: 2-side VFL inference process

6.x.1	Description

This is a 2-sides VFL training inference process that includes one service consumer, one VFL server and one or more VFL client(s). This process can be conducted after VFL training and each VFL client has a trained ML model.

6.x.2	Procedures

The 2-side VFL inference process can be conducted as follow:

1. Analytic request

0.NF discovery/selection, sample/feature alignment and VFL training are done

VFL clients(1 or more)

VFL server

Service consumer

2. Forward analytic request

3.Data collection for inference input data

4.Local inference

5. Send analytic output of local model

6.Aggregation of local inference result

7. Send aggregated result

Figure 6.x.2-1: Procedure for VFL inference

- The NF discovery, sample/feature alignment and training are done.

- The service consumer sends an analytic request to VFL server with information of prediction(s) it wants to obtain.

- The VFL server forwards the information of predictions to VFL clients with an optional maximum response time.

- The VFL clients conduct data collection if the inference input data are not fully available.

- The VFL clients perform local inference with the trained local model.

- The VFL clients send the local inference result to VFL server before maximum response time expires.

- The VFL server aggregate the local inference result from VFL clients. If some clients did not send the local inference result on time, the VFL server could decide to aggregate inference results from other clients first and continue the step below.

- The VFL server send the final aggregated result to service consumer.

- NOTE： If one of the VFL server or VFL client(s) is inside 5GC and another one is an external AF, the interaction between them shall via an NEF.

- 6.x.3	Impacts on existing services, entities and interfaces

To actualize the solution above, the VFL server(e.g. NWDAF/AF) shall have ability to receive the analytic request from consumer and forward the request to every client. It shall also have ability to aggregate local inference result and send the aggregated inference result to consumer. For VFL client(e.g. NWDAF/AF), it shall have the ability to perform inference using the trained model and send the local inference result to VFL client.

---------- End of change ----------