SA WG2 Meeting #161	S2-2402019

26 February - 1 March, 2024, Athens, Greece

Source:	vivo

Title:	New Sol: PDU Session establishment for MA without 5G NAS over non-3GPP

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes a new Solution for PDU Session establishment to address KI#2.2 "non-3GPP access without 5G NAS over non-3GPP".

1	Discussion

This paper proposes a new solution for KI#2.2 of the FS_MASSS SID (SP-231802).

2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.1.0.

* * * * Start of Changes * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of DualSteer Solutions to Key Issues

|  | Key Issues for DualSteer |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# | <Key Issue #1.1> | <Key Issue #1.2> |  |  |
| #X |  |  |  |  |

Table 6.0-2: Mapping of ATSSS_Ph4 Solutions to Key Issues

|  | Key Issues for ATSSS_Ph4 |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# | <Key Issue #2.1> | <Key Issue #2.2> |  |  |
| #Y |  | x |  |  |

* * * * Second Change (all new text) * * * *

### 6.2.Y	Solution #Y: PDU Session establishment for MA without 5G NAS over non-3GPP

6.2.Y.1	Description

This solution addresses the PDU Session establishment aspect of Key Issue#2.2 "Simplified ATSSS architecture over non-3GPP access" that related to "non-3GPP access without 5G NAS over non-3GPP".

6.2.Y.2	Procedures

Figure 6.2.Y.2-1: PDU Session Establishment with non-3GPP access

1.	The UE sends a PDU Session Establishment Request message to AMF via 3GPP access for establishing MA PDU Session.

2.	The AMF invokes Nsmf_PDUSession_CreateSMContext Request service operation towards an SMF.

3.	The SMF determines that the MA PDU Session is configured without N3IWF based on subscription information.

4.	The SMF query an Anchor Function with the PDU Session ID and GPSI to get the Anchor Info, which includes FQDN or IP address of the Anchor Function.

5.	The SMF invokes Namf_Communication_N1N2MessageTransfer Request (N1 Info (PDU Session Establishment Accept (Anchor Info))) service operation towards the AMF.

6.	The AMF sends the PDU Session Establishment Accept to the UE via 3GNN access.

7.	The UE uses the Anchor Info to initiates IKEv2 procedure with the Anchor Function via non-3GPP access network, which results an IPSec SA creation for the PDU Session. The UE requests connectivity to the PDU Session providing the PDU Session ID, that is conveyed with IKEv2 (e.g., in the IDr payload).

6.1.X.3	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing 3GPP services, entities and interfaces.

SMF:

-	Indicates FQDN or IP address of Anchor Function to UE.

* * * * End of Changes * * * *